## The National Compliance Monitoring Dataset for the Second Six-Year Review

(the “Six-Year Review-ICR Dataset”)

EPA conducted a voluntary data call-in from the states and primacy agencies to obtain the compliance monitoring data necessary to analyze national contaminant occurrence in support of the second Six-Year Review.  This data call-in was conducted through the ICR process.  EPA requested that states and primacy agencies submit their SDWA compliance monitoring collected between January 1998 and December 2005.

Through extensive data management efforts, quality assurance evaluations, and communications and consultations with state data management staff, EPA established a high quality contaminant occurrence dataset that consists of compliance monitoring data from 47 states/primacy agencies (45 states plus Region 8 and Region 9 tribes).  This dataset is referred to as the National Compliance Monitoring ICR Dataset for the Second Six-Year Review (or “Six-Year Review-ICR Dataset”).  The Six-Year Review-ICR Dataset was used to estimate a variety of occurrence measures to characterize the national occurrence of regulated contaminants in public water systems to support the Six-Year Review process.

This Six Year Review-ICR Dataset represents the largest drinking water contaminant occurrence dataset ever compiled and analyzed by EPA’s Office of Groundwater and Drinking Water.   The data from these 47 states/primacy agencies comprise more than 15 million analytical records from approximately 132,000 public water systems.  Approximately 254 million people are served by these systems nationally.  The dataset includes the results of all compliance monitoring data (all sample analytical detections and non-detections) from January 1998 to December 2005 for 69 regulated contaminants, including 65 chemical contaminants and 4 radionuclides.

The data are presented in 69 separate files (one file per contaminant).  The number of records and the size of the files ranges considerably from contaminant to contaminant, generally reflecting the frequency of required sampling.  The smallest file is for dioxin (~2 MBs) and the largest is for nitrate (~184 MBs).  Typical file sizes are 30-50 MBs for IOCs, 20-30 MBs for SOCs, and 60-70 MBs for VOCs.

The following tables describe the information contained the chemical data sets.  Table 1 is a data dictionary for the chemical data sets; it contains the list of data elements, column names, and a brief description of the data for each data element.  Table 2 lists the names of the 69 contaminants, the chemical identification number for each contaminant, and the specific name for each of the 69 files.  (The categories in Table 2 refer to internal contaminant review priorities; the contaminants are listed alphabetically within each category.)

Table 1:  Six-Year Review-ICR Dataset Data Element List

| Data Element |  | Column Name | Description |
| --- | --- | --- | --- |
| Inventory Data |  |  |  |
|  | State Code | STATE | 2- digit state code |
|  | Public Water System Identification Number (PWSID) | PWSID | The code used to identify each PWS. The code begins with the standard 2-character postal state abbreviation or region code; the remaining 7 numbers are unique to each PWS in the state. |
|  | System Name | PWSNAME | Name of the PWS. |
|  | Federal Public Water System Type Code | PWSTYPE | A code to identify whether a system is: • Community Water System (C); • Non-transient Non-community Water System (NTNC); or • Transient Non-community Water System (NC; nitrate/nitrite only). |
|  | Population-served | POPULATION | Highest average daily number of people served by a PWS, when in operation. |
|  | Source Water Type | SRCWATER | Type of water at the source. Source water type can be: • Ground water (GW); or • Surface water (SW); or • Ground water under the direct influence of surface water (GU) |
| Chemical-Specific Information |  |  |  |
|  | State ID | ID | State-specific unique identification code |
|  | Sampling Point Identification Code | SAMPLEPOINTID | A sampling point identifier established by the state, unique within each applicable facility, for each applicable sampling location (e.g., entry point to the distribution system). This information enables occurrence assessments that address intra-system variability. |
|  | Sample Identification Number | SAMPLEID | Identifier assigned by state or the laboratory that uniquely identifies a sample. |
|  | Sample Collection Date | DATE | Date the sample was collected, including month, day, and year. |
|  | Contaminant | CHEMID | 4-digit SDWIS contaminant identification number for which the sample is being analyzed. |
|  | Sample Analytical Result - Sign | DETECT | The sign indicates whether the sample analytical result was: • (0) "less than" means the contaminant was not detected or was detected at a level "less than" the MRL. • (1) "equal to" means the contaminant was detected at a level "equal to" the value reported in "Sample Analytical Result - Value." |
|  | Sample Analytical Result - Value | VALUE | Actual numeric (decimal) value of the analysis for the chemical result, or the MRL if the analytical result is less than the contaminant's MRL. |
|  | Sample Analytical Result - Unit of Measure | UNITS | Unit of measurement for the analytical results reported (usually expressed in either µg/L or mg/L for chemicals; or pCi/L for radionuclides). |

Table 2:  Six-Year Review-ICR Dataset List

| No. | Category | Chem. Name | Chem. ID | Dataset Name | Record Count |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1,1,2-Trichloroethane | 2985 | 1,1,2-Trichloroethane_Chem2985 | 372,386 |
| 2 | 1 | 1,2-Dibromo-3-chloropropane (DBCP) | 2931 | 1,2-Dibromo-3-chloropropane (DBCP)_Chem2931 | 231,169 |
| 3 | 1 | 1,2-Dichloroethane (Ethylene Dichloride) | 2980 | 1,2-Dichloroethane (Ethylene Dichloride)_Chem2980 | 375,636 |
| 4 | 1 | 1,2-Dichloropropane | 2983 | 1,2-Dichloropropane_Chem2983 | 374,409 |
| 5 | 1 | 2,4-D (2,4-Dichlorophenoxyacetic acid) | 2105 | 2,4-D (2,4-Dichlorophenoxyacetic acid)_Chem2105 | 124,285 |
| 6 | 1 | Benzene | 2990 | Benzene_Chem2990 | 374,503 |
| 7 | 1 | Carbofuran | 2046 | Carbofuran_Chem2046 | 114,142 |
| 8 | 1 | Carbon Tetrachloride | 2982 | Carbon Tetrachloride_Chem2982 | 385,864 |
| 9 | 1 | Chlordane | 2959 | Chlordane_Chem2959 | 128,453 |
| 10 | 1 | Dichloromethane (Methylene chloride) | 2964 | Dichloromethane (Methylene chloride)_Chem2964 | 372,009 |
| 11 | 1 | Endothall | 2033 | Endothall_Chem2033 | 49,820 |
| 12 | 1 | Heptachlor | 2065 | Heptachlor_Chem2065 | 133,726 |
| 13 | 1 | Heptachlor Epoxide | 2067 | Heptachlor Epoxide_Chem2067 | 134,708 |
| 14 | 1 | Hexachlorobenzene | 2274 | Hexachlorobenzene_Chem2274 | 135,512 |
| 15 | 1 | Hexachlorocyclopentadiene | 2042 | Hexachlorocyclopentadiene_Chem2042 | 135,290 |
| 16 | 1 | Oxamyl (Vydate) | 2036 | Oxamyl (Vydate)_Chem2036 | 113,380 |
| 17 | 1 | Tetrachloroethylene | 2987 | Tetrachloroethylene_Chem2987 | 410,042 |
| 18 | 1 | Toluene | 2991 | Toluene_Chem2991 | 374,250 |
| 19 | 1 | Toxaphene | 2020 | Toxaphene_Chem2020 | 115,403 |
| 20 | 1 | Trichloroethylene | 2984 | Trichloroethylene_Chem2984 | 403,609 |
| 21 | 1 | Vinyl chloride | 2976 | Vinyl chloride_Chem2976 | 373,161 |
| 22 | 1 | Xylenes (Total) | 2955 | Xylenes (Total)_Chem2955 | 363,466 |
| 23 | 2 | 1,1,1-Trichloroethane | 2981 | 1,1,1-Trichloroethane_Chem2981 | 374,950 |
| 24 | 2 | 1,1-Dichloroethylene | 2977 | 1,1-Dichloroethylene_Chem2977 | 378,630 |
| 25 | 2 | 1,2,4-Trichlorobenzene | 2378 | 1,2,4-Trichlorobenzene_Chem2378 | 371,132 |
| 26 | 2 | 1,2-Dichlorobenzene (o-Dichlorobenzene) | 2968 | 1,2-Dichlorobenzene (o-Dichlorobenzene)_Chem2968 | 372,946 |
| 27 | 2 | 1,4-Dichlorobenzene (p-Dichlorobenzene) | 2969 | 1,4-Dichlorobenzene (p-Dichlorobenzene)_Chem2969 | 370,698 |
| 28 | 2 | 2,3,7,8-TCDD (Dioxin) | 2063 | 2,3,7,8-TCDD (Dioxin)_Chem2063 | 11,076 |
| 29 | 2 | 2,4,5-TP (Silvex) | 2110 | 2,4,5-TP (Silvex)_Chem2110 | 122,713 |
| 30 | 2 | Alachlor | 2051 | Alachlor_Chem2051 | 155,989 |
| 31 | 2 | Alpha Particles (Gross Alpha, excl. Radon & U) | 4000 | Alpha Particles_Chem4000 | 73,002 |
| 32 | 2 | Antimony | 1074 | Antimony_Chem1074 | 185,986 |
| 33 | 2 | Arsenic | 1005 | Arsenic_Chem1005 | 224,035 |
| 34 | 2 | Asbestos | 1094 | Asbestos_Chem1094 | 18,181 |
| 35 | 2 | Atrazine | 2050 | Atrazine_Chem2050 | 161,690 |
| 36 | 2 | Barium | 1010 | Barium_Chem1010 | 183,118 |
| 37 | 2 | Benzo[a]pyrene | 2306 | Benzo[a]pyrene_Chem2306 | 119,871 |
| 38 | 2 | Beryllium | 1075 | Beryllium_Chem1075 | 183,575 |
| 39 | 2 | Beta Particles (Gross beta) | 4100 | Beta Particles (Gross beta)_Chem4100 | 59,983 |
| 40 | 2 | Cadmium | 1015 | Cadmium_Chem1015 | 181,667 |
| 41 | 2 | Chromium (total) | 1020 | Chromium (total)_Chem1020 | 185,952 |
| 42 | 2 | cis-1,2-Dichloroethylene | 2380 | cis-1,2-Dichloroethylene_Chem2380 | 373,316 |
| 43 | 2 | Combined Radium-226/228 | 4010 | Combined Radium-226/228_Chem4010 | 36,502 |
| 44 | 2 | Cyanide | 1024 | Cyanide_Chem1024 | 120,368 |
| 45 | 2 | Dalapon | 2031 | Dalapon_Chem2031 | 112,423 |
| 46 | 2 | Di(2-ethylhexyl) phthalate (DEHP) | 2039 | Di(2-ethylhexyl) phthalate (DEHP)_Chem2039 | 120,846 |
| 47 | 2 | Di(2-ethylhexyl)adipate (DEHA) | 2035 | Di(2-ethylhexyl)adipate (DEHA)_Chem2035 | 121,792 |
| 48 | 2 | Dinoseb | 2041 | Dinoseb_Chem2041 | 121,896 |
| 49 | 2 | Diquat | 2032 | Diquat_Chem2032 | 55,634 |
| 50 | 2 | Endrin | 2005 | Endrin_Chem2005 | 134,336 |
| 51 | 2 | Ethylbenzene | 2992 | Ethylbenzene_Chem2992 | 374,141 |
| 52 | 2 | Ethylene dibromide (EDB) | 2946 | Ethylene dibromide (EDB)_Chem2946 | 218,797 |
| 53 | 2 | Fluoride | 1025 | Fluoride_Chem1025 | 333,211 |
| 54 | 2 | Glyphosate | 2034 | Glyphosate_Chem2034 | 58,174 |
| 55 | 2 | Lindane | 2010 | Lindane_Chem2010 | 135,585 |
| 56 | 2 | Mercury (inorganic) | 1035 | Mercury (inorganic)_Chem1035 | 181,629 |
| 57 | 2 | Methoxychlor | 2015 | Methoxychlor_Chem2015 | 137,068 |
| 58 | 2 | Monochlorobenzene (Chlorobenzene) | 2989 | Monochlorobenzene (Chlorobenzene)_Chem2989 | 370,519 |
| 59 | 2 | Nitrate (as N) | 1040 | Nitrate (as N)_Chem1040 | 1,080,748 |
| 60 | 2 | Nitrite (as N) | 1041 | Nitrite (as N)_Chem1041 | 397,175 |
| 61 | 2 | Pentachlorophenol | 2326 | Pentachlorophenol_Chem2326 | 136,475 |
| 62 | 2 | Picloram | 2040 | Picloram_Chem2040 | 121,920 |
| 63 | 2 | Polychlorinated biphenyls (PCBs) | 2383 | Polychlorinated biphenyls (PCBs)_Chem2383 | 67,216 |
| 64 | 2 | Selenium | 1045 | Selenium_Chem1045 | 181,684 |
| 65 | 2 | Simazine | 2037 | Simazine_Chem2037 | 158,256 |
| 66 | 2 | Styrene | 2996 | Styrene_Chem2996 | 372,269 |
| 67 | 2 | Thallium | 1085 | Thallium_Chem1085 | 185,047 |
| 68 | 2 | trans-1,2-Dichloroethylene | 2979 | trans-1,2-Dichloroethylene_Chem2979 | 371,258 |
| 69 | 2 | Uranium | 4006 | Uranium_Chem4006 | 41,581 |