Revised Public Notification Handbook

3rd Revision, March 2023

Tier 1 Public Notification Instructions and Templates

The pages that follow contain instructions and templates for issuing Tier 1 public notification. Along with each template are instructions, including the required method of delivery and suggestions for completing individual sections of the notice. These instructions are designed to supplement Chapter 5, so you may see much of the information repeated here. The following templates are included:

- RTCR E. coli MCL Violation Notice – Template 1-1

- RTCR E. coli MCL Violation Notice in Spanish – Template 1-1A

- Nitrate MCL Violation – Template 1-2

- Nitrate MCL Violation Notice in Spanish – Template 1-2

- Nitrate Failure to Take a Confirmation Sample – Template 1-3

- Special Notice: NCWS Allowed Up to 20 mg/L Nitrate – Template 1-4

- NCWS Allowed > 20 mg/L Nitrate that Exceed 20 mg/L – Template 1-5

- Chlorine Dioxide MRDL – Template 1-6

- Combined Filter Effluent (CFE) Maximum Turbidity Exceedance, or Turbidity Single Exceedance – Template 1-7

- Waterborne Disease Outbreak – Template 1-8

- Fecal Indicator-Positive Source Sample – Template 1-9

- Problem Corrected – Template 1-10

Each template also includes the mandatory health effects language from Appendix B to 40 CFR 141 Subpart Q (presented in italics in each notice, with an asterisk on either end). This information must be included as written; with additional violation or situation specific information added in the brackets.

You must also include the following italicized language in all notices, where applicable [40 CFR 141.205(d)]. Use of this language does not relieve you of your obligation to take steps reasonably calculated to notify all persons served:

Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.

Instructions for E. coli MCL Violation Notice – Template 1-1

Template Below

Description of Violation or Situation

An E. coli maximum contaminant level (MCL) violation requires Tier 1 public notification. This violation occurs when any PWS has:

- A total coliform-positive routine sample result followed by an E. coli-positive repeat sample result;

- An E. coli-positive routine sample result followed by a total coliform-positive repeat sample result;

- An E. coli-positive routine sample result and fails to take all required repeat samples; or

- A total coliform-positive repeat sample result and fails to test for E. coli.

You must provide public notice to persons served as soon as practical but no more than 24 hours after learning of the MCL violation [40 CFR 141.202(b)]. During this time, you must also contact your state. You should also coordinate with your local health department. You must use one or more of the following methods to deliver the notice to consumers [40 CFR 141.202(c)]:

- Radio

- Television

- Hand or direct delivery

- Posting in conspicuous locations

- Another method approved in writing by the state

You may need to use additional methods (e.g., newspaper or delivery of multiple copies to hospitals, clinics, or apartment buildings) since notice must be provided in a manner reasonably calculated to reach all persons served. If you post or hand deliver, EPA recommends printing your notice on your system’s letterhead, if you have it.

The notice on the reverse is appropriate for hand delivery or for publication in a newspaper. However, you may wish to modify it before using it for a radio or television broadcast. If you do modify the notice on the reverse, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on each end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on each end.

Alternative Sources of Water

If you are providing bottled water, your notice should say where it can be obtained. If you are providing bottled water, make sure it meets U.S. Food and Drug Administration (FDA) and/or state bottled water safety standards.

Corrective Action

In your notice, you must describe corrective actions you are taking [40 CFR 141.205(a)(7)] and when you expect to return to compliance or resolve the situation [40 CFR 141.205(a)(8)]. Listed below are some steps commonly taken by water systems with the presence of E. coli. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are completing a comprehensive assessment of our water system and of our monitoring and operational practices to identify and correct any causes of the contamination.

- We are chlorinating and flushing the water system.

- We are switching to an alternate drinking water source.

- We are increasing sampling for coliform bacteria to determine the source of the contamination.

- We are repairing the wellhead seal.

- We are repairing, cleaning, and disinfecting the storage tank.

- We are restricting water intake from the river/lake/reservoir to prevent additional bacteria from entering the water system and restricting water use to emergencies.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is recommended that you notify health professionals in the area of the violation. People may call their doctors with questions about how the violation may affect their health, and the doctors should have the information they need to respond appropriately. In addition, health professionals, including dentists, use tap water during their procedures and need to know about the potential contamination so they can use bottled water.

It is a good idea to inform your consumers when the violation has been resolved. See Template 1-10 for a “problem corrected” notice template.

Revised Total Coliform Rule (RTCR) E. coli MCL Violation Notice – Template 1-1

DRINKING WATER WARNING

E. coli is present in [Water System Name]’s water

BOIL YOUR WATER BEFORE USING

[Briefly describe the situation, such as: “E. coli bacteria were found in the water supply on [give date]” or “We did not perform required testing of the water system and must assume that E. coli bacteria are in the water as of [give date]]. These bacteria can make you sick and are especially a concern for people with weakened immune systems.

Bacterial contamination can occur when increased run-off enters the drinking water source (for example, following heavy rains). It can also happen due to a break in the distribution system (pipes) or a failure in the water treatment process.

What should I do? What does this mean?

- DO NOT DRINK THE WATER WITHOUT BOILING IT FIRST. Bring all water to a boil, let it boil for one minute and let it cool before using, or use bottled water. Boiled or bottled water should be used for drinking, making ice, brushing teeth, washing dishes, and food preparation until further notice. Boiling kills bacteria and other organisms in the water.

- *E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Human pathogens in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a greater health risk for infants, young children, the elderly, and people with severely compromised immune systems. *

- The symptoms above are not caused only by organisms in drinking water. If you experience any of these symptoms and they persist, you may want to seek medical advice. People at increased risk should seek advice from their healthcare providers about drinking this water.

What is being done?

[Describe corrective action]. We will inform you when tests show no bacteria are present and you no longer need to boil your water. We anticipate resolving the problem within [estimated timeframe].

For more information, please contact [name of contact] at [phone number] or [mailing address]. General guidelines on ways to lessen the risk of infection by bacteria and other disease-causing organisms are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail. *

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for E. coli MCL Violation Notice in Spanish – Template 1-1A

Template on Reverse

The template on reverse is a Spanish translation of Template 1-1 for an E. coli maximum contaminant level (MCL) violation. All the instructions of Template 1-1 apply. This template is an exact translation of the English template, so if you need to modify the English templates, you should modify this template accordingly. Schools or universities may be able to provide low-cost translations. See the discussion on translations in Chapter 3 for suggestions on multilingual notices.

Revised Total Coliform Rule (RTCR) E. coli MCL Violation Notice in Spanish – Template 1-1A

AVISO SOBRE SU AGUA POTABLE

[E. coli está] presente en el agua del Sistema [Water System Name]

HIERVAN EL AGUA ANTES DE USARLA

[Briefly describe the situation in Spanish, such as: “E. coli fue encontrada en su servicio de agua el día [given date]” or “No hicimos las pruebas necesarias del servicio de agua y debemos suponer que la bacteria E. coli está en el agua a partir de [given date]]. Estas bacterias pueden enfermarle, y son especialmente peligrosas para personas con los sistemas inmunológicos débiles.

Contaminación bacteriana puede ocurrir cuando un exceso de aguas rebasa sus cauces y entran en las fuentes de agua potable (por ejemplo, después de una lluvia fuerte). También, puede ocurrir cuando se rompe un sistema de recolección de aguas negras (por ejemplo, una tubería), o cuando hay una falla en el tratamiento de agua.

¿Qué debo hacer? ¿Qué es la significa de este?

- NO BEBA EL AGUA SIN HERVIRLA ANTES. Hierva toda el agua, déjela hervir por un minuto, y déjela reposar antes de usarla, o utilice agua embotellada. Agua hervida o embotellada debe ser usada para beber, hacer hielo, lavarse los dientes, lavar los platos y para preparar la comida hasta próximo aviso. El proceso de hervir mata a bacteria y otros organismos en el agua.

- *E. coli son bacterias cuya presencia indican que el agua está contaminada con desechos humanos o de animales. Los agentes patógenos humanos en estos desechos pueden causar consecuencias a corto plazo, como diarrea, cólicos, nausea, dolores de cabeza u otros síntomas. Pueden representar un peligro más grave para la salud de bebés, niños y niñas de corta edad, los ancianos y personas con sistemas inmunológicos en alto riesgo. *

- Los síntomas descritos arriba no ocurren solamente debido a los microbios; pueden ser resultados de otros factores. Sin embargo, si usted siente estos síntomas y los persisten, usted puede optar por hacer una consulta con su médico. Personas en situaciones de alto riesgo deben consultar con sus proveedores de servicios médicos.

¿Qué se está haciendo al respecto?

[Describe corrective action in Spanish]. Le informaremos cuando las pruebas demuestren que no hay bacterias y cuando usted ya no necesita hervir su agua. Anticipamos que resolveremos el problema el [date of expected resolution in Spanish day-month-year].

Para obtener más información, contacte a [name of contact] al [phone number] o [mailing address]. Reglas generales sobre las maneras de reducir el riesgo de infección por bacterias y otros organismos causantes de enfermedades están disponibles en el sitio web de Office of Ground Water and Drinking Water de la EPA en https://www.epa.gov/ground-water-and-drinking-water.

*Por favor, comparta esta información con otros que toman de esta fuente de agua, especialmente con aquellos que no hayan recibido el aviso directamente, por ejemplo: personas en apartamentos, hospitales, hogares de infantes, escuelas o comunidades de negocios. Usted puede compartirlo en un lugar público o distribuyendo unas copias a mano o por correo. *

Este aviso ha sido enviado a usted por [water system name]. Número de Identificación: ___________.

Fecha de distribución: ________.

Instructions for Nitrate MCL Violation – Template 1-2

Template on Reverse

Since violating the nitrate maximum contaminant level (MCL) of 10 milligrams per liter (mg/L) requires Tier 1 public notification and you must provide public notice to persons served as soon as practical but no more than 24 hours from learning of the violation [40 CFR 141.202(b)]. During this time, you must also contact your state. You should also coordinate with your local health department. This template is also applicable to nitrite and total nitrate and nitrite violations. You must use one or more of the following methods to deliver the notice to consumers [40 CFR 141.202(c)]:

- Radio

- Television

- Hand or direct delivery

- Posting in conspicuous locations

- Another method approved in writing by the state

You may need to use additional methods (e.g., newspaper, delivery of multiple copies to hospitals, clinics, or apartment buildings), since notice must be provided in a manner reasonably calculated to reach all persons served. If you post or hand deliver, print your notice on your system’s letterhead, if you have it.

The notice on the reverse is appropriate for hand delivery or for publication in a newspaper. However, you may wish to modify it before using it for a radio or television broadcast. If you do modify the notice on the reverse, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.  You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Alternative Sources of Water

If you are providing bottled water, your notice should say where it can be obtained. If you are providing bottled water, make sure it meets U.S. Food and Drug Administration (FDA) and/or state bottled water safety standards.

Repeat Notices

If this is a repeat notice (as required by your state), or if your water system’s nitrate levels fluctuate around the MCL, you may wish to include an explanation similar to the following:

- On [give date], you were also notified of high nitrate levels that occurred during the [give quarter e.g., first, second, third or fourth] of the year. Since that time the water system has been monitoring the nitrate concentration every three months. Seasonal fluctuations in nitrate concentrations have been observed due to nitrates contained in fertilizer. It appears the high nitrates occur during the later summer and fall. Note that previous tests prior to [give year] show that we were meeting drinking water standards for nitrate.

Corrective Action

In your notice, describe corrective actions you are taking. The bullet below describes one action commonly taken by water systems with nitrate/nitrite violations. You can use the following language, if appropriate, or develop your own:

- We are investigating water treatment and other options. These may include drilling a new well, mixing the water with low-nitrate water from another source, or buying water from another water system.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is recommended that you notify health professionals in the area of the violation. People may call their doctors with questions about how the violation may affect their health, and the doctors should have the information they need to respond appropriately. In addition, health professionals, including dentists, use tap water during their procedures and need to know of potential contamination so they can use bottled water.

It is a good idea to inform your consumers when the violation has been resolved. See Template 1-10 for a “problem corrected” notice template.

Nitrate MCL Violation Notice – Template 1-2

DRINKING WATER WARNING

[Water System Name] water has high levels of nitrate -

DO NOT GIVE THE WATER TO INFANTS UNDER 6 MONTHS OLD OR USE IT TO MAKE INFANT FORMULA

On [give date], we received notice that the sample collected on [give date] showed nitrate levels of [level and units]. This is above the nitrate standard, or maximum contaminant level (MCL), of [state/federal MCL]. Nitrate in drinking water is a serious health concern for infants less than six months old.

Nitrate in drinking water can come from natural, industrial, or agricultural sources (including septic systems and run-off). Levels of nitrate in drinking water can vary throughout the year.

What should I do? What does this mean?

- DO NOT GIVE THE WATER TO INFANTS. * Infants below the age of six months who drink water containing nitrate in excess of the MCL could become seriously ill and, if untreated, may die. Symptoms include shortness of breath and blue baby syndrome. * Blue baby syndrome is indicated by blueness of the skin. Symptoms in infants can develop rapidly, with health deteriorating over a period of days. If symptoms occur, seek medical attention immediately.

- Ice, juice, and formula for children under six months of age should not be prepared with tap water. Bottled water or other water low in nitrates should be used for infants until further notice.

- DO NOT BOIL THE WATER. Boiling, freezing, filtering, or letting water stand does not reduce the nitrate level.  Boiling can make the nitrates more concentrated, because nitrates remain behind when the water evaporates.

- Adults and children older than six months can drink the tap water (nitrate is a concern for infants because they can’t process nitrates in the same way adults can). However, if you are pregnant or have specific health concerns, you may wish to consult your doctor.

What is being done?

[Describe all corrective actions being taken and when your water system expects to return to compliance.]

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail. *

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Nitrate MCL Violation Notice in Spanish – Template 1-2A

Template on Reverse

The template on reverse is a Spanish translation of Template 1-2 for a nitrate maximum contaminant level (MCL) violation. All the instructions of Template 1-2 apply. This template is an exact translation of the English template, so if you need to modify the English templates, you should modify this template accordingly. Schools or universities may be able to provide low-cost translations. See the discussion on translations in Chapter 3 for suggestions on multilingual notices

Nitrate MCL Violation Notice in Spanish – Template 1-2A

AVISO SOBRE SU AGUA POTABLE

Agua del sistema [Water System Name] tiene altos niveles de nitratos –

NO DAR DE BEBER ESTA AGUA A BEBES MENORES DE 6 MESES DE EDAD NI USARLA PARA HACER LECHE DE FORMULA

Recibimos un aviso el [give date of when notice was received] que la muestra tomada el [give date of when sample was taken] demuestra una concentración de nitrato de [level found and units in Spanish]. Este nivel está por encima de la norma, o nivel máximo de contaminación (NMC) de [state/federal MCL with units in Spanish]. Nitratos en agua potable puede generar serios problemas de salud para bebés menores de 6 meses de edad.

Nitratos en el agua pueden provenir de fuentes naturales, industriales, o de la agricultura (incluyendo descargas de tanques sépticos y lluvias). Las concentraciones de nitratos en el agua potable varían a lo largo del año.

¿Qué debo hacer? ¿Qué es la significa de este?

- NO LE DE ESTA AGUA A BEBES. *Bebés menores de seis (6) meses que ingieran agua con nitratos en exceso del nivel máximo de contaminación (NMC) se pueden enfermar seriamente y, de no ser tratados, pueden morir. Los síntomas incluyen dificultad en respirar y síndrome de bebé azul. * El síndrome de bebé azul se refiere al color azulado que toma la piel del bebé. Los síntomas en los bebés pueden desarrollarse con rapidez, con el deterioro de su salud en los días subsiguientes. Si los síntomas ocurren en infantes menores de seis (6) meses de edad, busque atención médica inmediatamente.

- Hielo, jugo o leche en polvo para bebés menores de seis (6) meses de edad no debe prepararse con agua del grifo. Debe emplear agua embotellada u otra agua baja en nitratos hasta próximo aviso.

- No hierva el agua. Hervir, congelar, filtrar, o dejar el agua en reposo no reduce el nivel de nitratos. De hecho, al hervir el agua puede aumentar aún más la concentración de nitratos, debido a que los nitratos permanecen cuando parte del agua se evapora.

- Adultos e infantes mayores de seis (6) meses de edad pueden tomar el agua del grifo. (Los nitratos son peligrosos para los bebés debido a que ellos no pueden procesar los nitratos de la misma manera que los adultos). Sin embargo, si usted está embarazada o tiene algún problema de salud en particular, puede optar por hacer una consulta con su médico.

¿Qué se está haciendo al respecto?

[Describe all corrective actions being taken and when your water system expects to return to compliance in Spanish.]

Para más información, favor contactar a [name of contact] al [phone number] o [mailing address].

*Por favor comparta esta información con otros que pueden tomar de esta agua, colocando este aviso en lugares visibles, o remitiéndolo por correo, o entregándolo manualmente. Es de particular interés distribuir este aviso ampliamente si usted lo recibe representando un negocio, un hospital u hogar de infantes u hogar de ancianos o comunidad residencial. *

Este aviso ha sido enviado a usted por [water system name]. Número de Identificación: ___________.

Fecha de distribución: ________.

Instructions for Nitrate Failure to Take a Confirmation Sample – Template 1-3

Template on Reverse

Since failure to take a confirmation sample for nitrate within 24 hours after learning that an initial sample exceeded the maximum contaminant level (MCL) requires Tier 1 public notification and you must provide public notice to persons served as soon as practical but no more than 24 hours from learning of the violation [40 CFR 141.202(b)]. During this time, you must also contact your state. You should also coordinate with your local health department. This template is also applicable to missed confirmation samples for nitrite. You must use one or more of the following methods to deliver the notice to consumers [40 CFR 141.202(c)]:

- Radio

- Television

- Hand or direct delivery

- Posting in conspicuous locations

- Another method approved in writing by the state

You may need to use additional methods (e.g., newspaper, delivery of multiple copies to hospitals, clinics, or apartment buildings), since notice must be provided in a manner reasonably calculated to reach all persons served. If you post or hand deliver, print your notice on your system’s letterhead, if you have it.

The notice on the reverse is appropriate for hand delivery or for publication in a newspaper. However, you may wish to modify it before using it for a radio or television broadcast. If you do modify the notice on the reverse, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information. You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Alternative Sources of Water

If you are providing bottled water, your notice should say where it can be obtained. If you are providing bottled water, make sure it meets U.S. Food and Drug Administration (FDA) and/or state bottled water safety standards.

Repeat Notices

If this is a repeat notice (as required by your state), you may wish to include an explanation similar to the following:

- You were initially notified of our water system’s failure to take a confirmation sample following high nitrate level on [give date]. Since that time, we have taken a confirmation sample on [give date] that also resulted in a high nitrate level. We are currently monitoring the nitrate concentration [give sampling timeframe e.g., every three months]. Seasonal fluctuations in nitrate concentrations have been observed, due to nitrates contained in fertilizer. It appears that high nitrates occur during the later summer and fall. Note that prior to [give year] we were meeting drinking water standards for nitrate.

Corrective Action

In your notice, describe corrective actions you are taking. The bullet below describes one action commonly taken by water systems with nitrate/nitrite violations. You can use the following language, if appropriate, or develop your own text:

- We are in the process of collecting a confirmation sample to determine if we have high nitrate levels. If the sample shows we are meeting our drinking water standards, you will not receive another notice. However, if the sample shows that we do have high nitrate levels in our water, another notification will be issued within 24 hours after we receive the results.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is recommended that you notify health professionals in the area of the violation. People may call their doctors with questions about how the violation may affect their health, and the doctors should have the information they need to respond appropriately. In addition, health professionals, including dentists, use tap water during their procedures and need to know of potential contamination so they can use bottled water.

It is a good idea to inform your consumers when the violation has been resolved. See Template 1-10 for a “problem corrected” notice template.

Nitrate Failure to Take a Confirmation Sample Notice – Template 1-3

DRINKING WATER WARNING

[Water System Name] did not take a confirmation sample to determine if the water has high levels of nitrate –

DO NOT GIVE THE WATER TO INFANTS UNDER 6 MONTHS OLD OR USE IT TO MAKE INFANT FORMULA

*We are required to monitor your drinking water for nitrate on a regular basis. Results of regular monitoring are an indicator of whether or not our drinking water meets health standards. During [compliance period] we [‘did not monitor or test’ or ‘did not complete all monitoring or testing’] for nitrate and therefore cannot be sure of the quality of your drinking water during that time. *

On [give date], we received notice that the sample collected on [give date] showed nitrate levels above the nitrate standard, or maximum contaminant level (MCL), of [give state/federal MCL]. We were required to take a confirmation sample within 24 hours. We did not complete the required confirmation sample monitoring. Nitrate in drinking water is a serious health concern for infants less than six months old.

Nitrate in drinking water can come from natural, industrial, or agricultural sources (including septic systems and run-off). Levels of nitrate in drinking water can vary throughout the year.

What should I do? What does this mean?

- DO NOT GIVE THE WATER TO INFANTS. *Infants below the age of six months who drink water containing nitrate in excess of the MCL could become seriously ill and, if untreated, may die. Symptoms include shortness of breath and blue baby syndrome. * Blue baby syndrome is indicated by blueness of the skin. Symptoms in infants can develop rapidly, with health deteriorating over a period of days. If symptoms occur, seek medical attention immediately.

- Ice, juice, and formula for children under six months of age should not be prepared with tap water. Bottled water or other water low in nitrates should be used for infants until further notice.

- DO NOT BOIL THE WATER. Boiling, freezing, filtering, or letting water stand does not reduce the nitrate level.  boiling can make the nitrates more concentrated, because nitrates remain behind when the water evaporates.

- Adults and children older than six months can drink the tap water (nitrate is a concern for infants because they can’t process nitrates in the same way adults can). However, if you are pregnant or have specific health concerns, you may wish to consult your doctor.

What is being done?

[Describe all corrective actions being taken and when your water system expects to return to compliance.] After receiving the results of our first sample, we failed to collect a second sample within 24 hours to confirm the results of the first sample. We have since collected the second sample and are waiting for the results.

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail. *

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Special Notice: Instructions for NCWSs Allowed Up to 20 mg/L Nitrate – Template 1-4

Template on Reverse

NCWSs that have received approval by their state to exceed the nitrate maximum contaminant level of 10 milligrams per liter (mg/L) but no more than 20 mg/L are required to provide Tier 1 public notification. You do not incur a violation but are still required to provide public notice to persons served as soon as practical but no more than 24 hours from learning of the situation [40 CFR 141.202(b)]. You must use one or more of the following methods to deliver the notice to consumers [40 CFR 141.202(c)]:

- Posting in conspicuous locations

- Hand or direct delivery

- Radio

- Television

- Another method approved in writing by the state

You may need to use additional methods if needed to reach all persons served. If you post or hand deliver, print your notice on your system’s letterhead, if you have it.

If you do modify the notice on the reverse, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Continuous Notices

If you are granted permission by the state to exceed the nitrate MCL, you must provide continuous posting that the nitrate levels exceeded 10 mg/L and of the potential health effects of exposure [40 CFR 141.209(b)].

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

This template also includes mandatory language encouraging further distribution to persons who may not have received the notice, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end. However, if you post the notice such that all possible users have access to the notice, this language is not applicable and can be omitted.

Alternative Sources of Water

If you are providing bottled water, your notice should say where it can be obtained. If you are providing bottled water, make sure it meets U.S. Food and Drug Administration (FDA) and/or state bottled water safety standards.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

Special Notice: NCWSs Allowed Up to 20 mg/L Nitrate Notice – Template 1-4

DRINKING WATER WARNING

[Water System Name] water has high levels of nitrate –

DO NOT GIVE THE WATER TO INFANTS UNDER 6 MONTHS OLD OR USE IT TO MAKE INFANT FORMULA

Water sample results show nitrate levels of [level and units]. This is above the nitrate standard or maximum contaminant level (MCL), of 10 milligrams per liter (mg/L). Nitrate in drinking water is a serious health concern for infants less than six months old.

We have been given permission by [state] to provide water in excess of the standard as long as:

- Nitrate levels do not exceed 20 mg/L.

- The water is not made available to children under 6 months of age.

- We continuously post this notice meeting all public notice requirements.

- Local and state health officers are notified annually.

- No adverse health effects result.

What should I do? What does this mean?

- DO NOT GIVE THE WATER TO INFANTS. *Infants below the age of six months who drink water containing nitrate in excess of the MCL could become seriously ill and, if untreated, may die. Symptoms include shortness of breath and blue baby syndrome. * Blue baby syndrome is indicated by blueness of the skin. Symptoms in infants can develop rapidly, with health deteriorating over a period of days. If symptoms occur, seek medical attention immediately.

- Ice, juice, and formula for children under six months of age should not be prepared with tap water. Bottled water or other water low in nitrates should be used for infants until further notice.

- DO NOT BOIL THE WATER. Boiling, freezing, filtering, or letting water stand does not reduce the nitrate level.  boiling can make the nitrates more concentrated, because nitrates remain behind when the water evaporates.

- Adults and children older than six months can drink the tap water (nitrate is a concern for infants because they can’t process nitrates in the same way adults can). However, if you are pregnant or have specific health concerns, you may wish to consult your doctor.

For more information, please contact [name of contact] at [phone number] or [mailing address].

* Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail. *

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: __________

Instructions for NCWSs Allowed > 20 mg/L Nitrate that Exceed 20 mg/L – Template 1-5

Template on Reverse

Since exceeding the maximum contaminant level (MCL) of 20 milligrams per liter (mg/L) for NCWSs that have received approval by their state to exceed the nitrate (MCL) of 10 mg/L but no more than 20 mg/L requires Tier 1 public notification, you must provide public notice to persons served as soon as practical but no more than 24 hours from learning of the violation [40 CFR 141.202(b)]. During this time period you must also contact your state. You should also coordinate with your local health department. This template is also applicable to nitrite and total nitrate and nitrite violations. You must use one or more of the following methods to deliver the notice to consumers [40 CFR 141.202(c)]:

- Posting in conspicuous locations

- Hand or direct delivery

- Radio

- Television

- Another method approved in writing by the state

You may need to use additional methods if needed to reach all persons served. If you post or hand deliver, print your notice on your system’s letterhead, if you have it.

If you do modify the notice on the reverse, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

This template also includes mandatory language encouraging further distribution to persons who may not have received the notice, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end. However, if you post the notice such that all possible users have access to the notice, this language is not applicable and can be omitted.

Alternative Sources of Water

If you are providing bottled water, your notice should say where it can be obtained. If you are providing bottled water, make sure it meets U.S. Food and Drug Administration (FDA) and/or state bottled water safety standards.

Repeat Notices

If this is a repeat notice (as required by your state), or if your system’s nitrate levels fluctuate around the MCL, you may wish to include an explanation similar to the following:

- You were initially notified of high nitrate levels on [give date]. Since that time, we have been monitoring the nitrate concentration every three months. Seasonal fluctuations in nitrate concentrations have been observed, due to nitrates contained in fertilizer. It appears that high nitrates occur during the later summer and fall. Note that prior to [give year] we were meeting drinking water standards for nitrate.

Corrective Action

In your notice, describe corrective actions you are taking. The bullet below describes one action commonly taken by water systems with nitrate/nitrite violations. You can use the following language, if appropriate, or develop your own:

- We are investigating water treatment and other options. These may include drilling a new well, mixing the water with low-nitrate water from another source, or buying water from another water system.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

NCWS Allowed > 20 mg/L Nitrate that Exceed 20 mg/L Notice – Template 1-5

DRINKING WATER WARNING

[Water System Name] water has high levels of nitrate -

DO NOT GIVE THE WATER TO INFANTS UNDER 6 MONTHS OLD OR USE IT TO MAKE INFANT FORMULA

Water sample results show nitrate levels of [level and units]. This is above the nitrate standard or maximum contaminant level (MCL), of 10 milligrams per liter (mg/L). Nitrate in drinking water is a serious health concern for infants less than six months old. We have been given permission by [state] to provide water in excess of the standard as long as:

- Nitrate levels do not exceed 20 mg/L.

- The water is not made available to children under 6 months of age.

- We continuously post this notice meeting all public notice requirements.

- Local and state health officers are notified annually.

- No adverse health effects result.

On [give date], we received notice that the sample collected on [give date] showed nitrate levels of [level and units]. This is above the 20 mg/L level allowed by [state]. Nitrate in drinking water is a serious health concern for infants less than six months old.

Nitrate in drinking water can come from natural, industrial, or agricultural sources (including septic systems and run-off). Levels of nitrate in drinking water can vary throughout the year.

What should I do? What does this mean?

- DO NOT GIVE THE WATER TO INFANTS. * Infants below the age of six months who drink water containing nitrate in excess of the MCL could become seriously ill and, if untreated, may die. Symptoms include shortness of breath and blue baby syndrome. * Blue baby syndrome is indicated by blueness of the skin. Symptoms in infants can develop rapidly, with health deteriorating over a period of days. If symptoms occur, seek medical attention immediately.

- Ice, juice, and formula for children under six months of age should not be prepared with tap water. Bottled water or other water low in nitrates should be used for infants until further notice.

- DO NOT BOIL THE WATER. Boiling, freezing, filtering, or letting water stand does not reduce the nitrate level.  Boiling can make the nitrates more concentrated, because nitrates remain behind when the water evaporates.

- Adults and children older than six months can drink the tap water (nitrate is a concern for infants because they can’t process nitrates in the same way adults can). However, if you are pregnant or have specific health concerns, you may wish to consult your doctor.

What is being done?

[Describe corrective action and when system expects to return to compliance.]

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail. *

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Chlorine Dioxide MRDL – Template 1-6

Template on Reverse

If a system exceeds the chlorine dioxide maximum residual disinfectant level (MRDL) when one or more of the samples taken in the distribution system on the day after exceeding the MRDL at the entrance to the distribution system or if a system does not take required samples in the distribution system, Tier 1 public notification is required. You must provide public notice to persons served as soon as practical but no more than 24 hours from learning of the violation [40 CFR 141.202(b)]. During this time, you must also contact your state. (Exceeding the chlorine dioxide MRDL at the entry point to the distribution system only, requires Tier 2 public notification; modify this template or Template 2-2 to create a Tier 2 notice.) You should also coordinate with your local health department. You must use one or more of the following methods to deliver the notice to consumers [40 CFR 141.202(c)]:

- Radio

- Television

- Hand or direct delivery

- Posting in conspicuous locations

- Another method approved in writing by the state

You may need to use additional methods (e.g., newspaper, delivery of multiple copies to hospitals, clinics, or apartment buildings), since notice must be provided in a manner reasonably calculated to reach all persons served. If you post or hand deliver, print your notice on your system’s letterhead, if you have it. The notice on the reverse is appropriate for hand delivery or for publication in a newspaper. However, you may wish to modify it before using it for a radio or television broadcast. If you do modify the notice on the reverse, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.  You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Alternative Sources of Water

If you are providing bottled water, your notice should say where it can be obtained. If you are providing bottled water, make sure it meets U.S. Food and Drug Administration (FDA) and/or state bottled water safety standards.

Population at Risk

The language on the reverse lists “young children” as one of the groups at increased risk. Because the potential health effects of chlorine dioxide are based on tests on laboratory animals, there is no way to determine at exactly what age the water is safe to drink. If your consumers have questions, encourage them to err on the side of caution.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with chlorine dioxide violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are resetting the chlorine dioxide generator to generate the correct amount of chlorine dioxide.

- We are repairing the chlorine dioxide generator.

- We have already fixed the problem, but it will take additional time for the extra chlorine dioxide to be flushed from the distribution system (pipes).

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is recommended that you notify health professionals in the area of the violation. People may call their doctors with questions about how the violation may affect their health, and the doctors should have the information they need to respond appropriately. In addition, health professionals, including dentists, use tap water during their procedures and need to know of potential contamination so they can use bottled water.

It is a good idea to inform your consumers when the violation has been resolved. See Template 1-10 for a “problem corrected” notice template.

Chlorine Dioxide MRDL Notice – Template 1-6

DRINKING WATER WARNING

PREGNANT WOMEN AND YOUNG CHILDREN

SHOULD NOT DRINK THE WATER

On [give date], we received notice that the sample collected on [give date] showed chlorine dioxide levels of [level and units]. This is above the standard, or maximum residual disinfectant level (MRDL) of 0.8 milligrams per liter (mg/L). Chlorine dioxide is used for disinfection, but too much of it over a short period of time may harm the development of children, infants, and fetuses.

Chlorine dioxide is used in small amounts every day to kill bacteria and other organisms that may be in your drinking water. A problem occurred with our chlorine dioxide generator, and too much chlorine dioxide was released.

What should I do? What does this mean?

- DO NOT USE THIS WATER IF YOU ARE PREGNANT OR GIVE IT TO YOUNG CHILDREN. Bottled water should be used until further notice. *Some infants and young children who drink water containing chlorine dioxide in excess of the MRDL could experience nervous system effects. Similar effects may occur in fetuses of pregnant mothers who drink water containing chlorine dioxide in excess of the MRDL. Some people may experience anemia.

- The chlorine dioxide violations reported today include exceedances of the EPA standard within the distribution system which delivers water to consumers. Violations of the chlorine dioxide standard within the distribution system may harm human health based on short-term exposures. Certain groups, including fetuses, infants, and young children, may be especially susceptible to nervous system effects from excessive chlorine dioxide exposure. * There are no obvious symptoms, but chlorine dioxide can affect development of the nervous system.

- Ice, juice, and formula for young children and for pregnant women should not be prepared with tap water.

- Adults who are not pregnant and older children can drink the tap water because their nervous systems are already developed. However, if you have specific health concerns, you may wish to consult your doctor.

What is being done?

[Describe corrective action and when you expect to return to compliance.]

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail. *

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for CFE Maximum Turbidity Exceedance, or Turbidity Single Exceedance – Template 1-7

Template on Reverse

If your state has designated this turbidity single exceedance as requiring Tier 1 public notification [40 CFR 141.202(a)] and you must provide public notice to persons served as soon as practical but no more than 24 hours after it has been designated as requiring Tier 1 public notification [40 CFR 141.202(b)]. Turbidity violations require Tier 2 public notification by default but may be elevated by the state who may require you to issue Tier 1 public notice. In addition, violations are automatically elevated to requiring Tier 1 public notification if you are unable to consult with your state within 24 hours. In such cases, you must issue a notice within the next 24 hours. You may elevate the violation to Tier 1 public notice yourself as well. You should also coordinate with your local health department. One or both agencies should tell you whether to instruct consumers to boil water. You must use one or more of the following methods to deliver the notice to consumers [40 CFR 141.202(c)]:

- Radio

- Television

- Hand or direct delivery

- Posting in conspicuous locations

- Another method approved in writing by the state

You may need to use additional methods (e.g., newspaper, delivery of multiple copies to hospitals, clinics, or apartment buildings), since notice must be provided in a manner reasonably calculated to reach all persons served. If you post or hand deliver, print your notice on your system’s letterhead, if you have it. The notice on the reverse is appropriate for hand delivery or for publication in a newspaper. However, you may wish to modify it before using it for a radio or television broadcast. If you do modify the notice on the reverse, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information. You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Alternative Sources of Water

If you are providing bottled water, your notice should say where it can be obtained. If you are providing bottled water, make sure it meets U.S. Food and Drug Administration (FDA) and/or state bottled water safety standards.

Corrective Action

In your notice, describe the corrective actions you are taking. Listed below are some steps commonly taken by water systems with a single turbidity exceedance. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are adding chemicals that reduce turbidity.

- We are sampling both untreated and treated water for the presence of coliform bacteria.

- We are inspecting and repairing the filters.

Source of the Problem

If you know why the turbidity is high, explain it in your notice. For instance, unusual conditions such as heavy rains and flooding can overburden the water plant and treated water may therefore not meet the standards. In addition, run-off from parts of the watershed could contain increased concentrations of sediment and animal waste.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is recommended that you notify health professionals in the area of the violation. People may call their doctors with questions about how the violation may affect their health, and the doctors should have the information they need to respond appropriately. In addition, health professionals, including dentists, use tap water during their procedures and need to know of potential contamination so they can use bottled water. It is a good idea to inform your consumers when the violation has been resolved. See Template 1-10 for a “problem corrected” notice template.

CFE Maximum Turbidity Exceedance, or Turbidity Single Exceedance Notice – Template 1-7

DRINKING WATER WARNING

[Water System Name] has high turbidity levels

BOIL YOUR WATER BEFORE USING

We routinely monitor your water for turbidity (cloudiness). This tells us whether we are effectively filtering the water supply. A water sample taken [give date] showed turbidity levels of [number] turbidity units. This is above the standard of [standard] turbidity units. Because of these high levels of turbidity, there is an increased chance that the water may contain disease-causing organisms.

What should I do? What does this mean?

- DO NOT DRINK THE WATER WITHOUT BOILING IT FIRST. Bring all water to a boil, let it boil for one minute, and let it cool before using, or use bottled water. Boiled or bottled water should be used for drinking, making ice, brushing teeth, washing dishes, and food preparation until further notice. Boiling kills bacteria and other organisms in the water.

- *Turbidity has no health effects. However, turbidity can interfere with disinfection and provide a medium for microbial growth. Turbidity may indicate the presence of disease-causing organisms. These organisms include bacteria, viruses, and parasites, which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches. *

- The symptoms above are not caused only by organisms in drinking water. If you experience any of these symptoms and they persist, you may want to seek medical advice. People at increased risk should seek advice from their healthcare providers about drinking this water.

What is being done?

[Describe corrective action and when the system expects to return to compliance.]

We will inform you when turbidity returns to appropriate levels and when you no longer need to boil your water. We anticipate resolving the problem within [estimated time frame].

For more information, please contact [name of contact] at [phone number] or [mailing address]. General guidelines on ways to lessen the risk of infection by microbes are available from the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail. *

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Waterborne Disease Outbreak – Template 1-8

Template on Reverse

Since a waterborne disease outbreak is a situation requiring Tier 1 public notification and you must provide public notice to persons served as soon as practical but no more than 24 hours from learning of the situation [40 CFR 141.202(b)]. During this time, you must also contact your state. You should also coordinate with your local health department. You must also issue a public notice if you are experiencing a waterborne emergency other than a waterborne disease outbreak, such as one caused by flooding or treatment failure. In such cases, you may be able to modify this template to apply to your situation. Check with your state for more direction. More information on waterborne disease outbreaks and emergencies is available from the Centers for Disease Control and Prevention (http://www.cdc.gov/healthywater/diseases.html, 1-800-232-4636). You must use one or more of the following methods to deliver the notice to consumers [40 CFR 141.202(c)]:

- Radio

- Television

- Hand or direct delivery

- Posting in conspicuous locations

- Another method approved in writing by the state

You may need to use additional methods (e.g., newspaper, delivery of multiple copies to hospitals, clinics, or apartment buildings), since notice must be provided in a manner reasonably calculated to reach all persons served. If you post or hand deliver, print your notice on your system’s letterhead, if you have it. The notice on the reverse is appropriate for hand delivery or for publication in a newspaper. However, you may wish to modify it before using it for a radio or television broadcast. If you do modify the notice on the reverse, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.  You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end. No federal mandatory health effects language exists for waterborne disease outbreaks. You may wish to use the sentence below, if appropriate, or contact your state or health department for other language.

- These symptoms are common to many diseases caused by microscopic organisms: Symptoms may include nausea, cramps, diarrhea, jaundice, and associated headaches and fatigue.

Alternative Sources of Water

If you are providing bottled water, your notice should say where it can be obtained. If you are providing bottled water, make sure it meets U.S. Food and Drug Administration (FDA) and/or state bottled water safety standards.

Describing the Outbreak

If known, list any organisms detected, the number of affected people, any water treatment problems contributing to the waterborne disease outbreak, and any sources of contamination, such as flooding.

Population at Risk

Some people who contract waterborne diseases can be affected more severely than others, as described on the reverse page. The specific language on the reverse is not mandatory, but you must provide information on the population at risk. In addition, make sure it is clear who is served by your water system—you may need to list the areas you serve.

Corrective Action

In your notice, describe the corrective actions you are taking. Listed below are some steps commonly taken by water systems with waterborne disease outbreaks. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are repairing our filtration system.

- We are increasing sampling for disease-causing organisms.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is recommended that you notify health professionals in the area of the violation. People may call their doctors with questions about how the violation may affect their health, and the doctors should have the information they need to respond appropriately. In addition, health professionals, including dentists, use tap water during their procedures and need to know of potential contamination so they can use bottled water. It is a good idea to inform your consumers when the violation has been resolved. See Template 1-10 for a “problem corrected” notice template.

Waterborne Disease Outbreak Notice – Template 1-8

DRINKING WATER WARNING

Disease-causing organisms have entered [Water System Name]’s water supply.

BOIL YOUR WATER BEFORE USING

These organisms are causing illness in people served by [water system name]. We learned of a waterborne disease outbreak from [agency] on [give date].

What should I do? What does this mean?

- DO NOT DRINK THE WATER WITHOUT BOILING IT FIRST. Bring all water to a boil, let it boil for one minute, and let it cool before using, or use bottled water. Boiled or bottled water should be used for drinking, making ice, brushing teeth, washing dishes, and food preparation until further notice. Boiling kills bacteria and other organisms in the water.

- [Describe symptoms of the waterborne disease.] If you experience one or more of these symptoms and they persist, contact your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about this drinking water.

What is being done?

[Describe the corrective action and when the outbreak might end.]

We will inform you when you no longer need to boil your water.

For more information, please contact [name of contact] at [phone number] or [mailing address]. General guidelines on ways to lessen the risk of infection by microbes are available from the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail. *

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Fecal Indicator-Positive Source Sample – Template 1-9

Template on Reverse

Since detection of a fecal indicator (E. coli, enterococci or coliphage) in a ground water source sample is a situation requiring Tier 1 public notification, you must provide public notice to persons served as soon as practical but no more than 24 hours from learning of the violation [40 CFR 141.202(b)]. During this time, you must also contact your state. You should also coordinate with your local health department. You must use one or more of the following methods to deliver the notice to consumers [40 CFR 141.202(c)]:

- Radio

- Television

- Hand or direct delivery

- Posting in conspicuous locations

- Another method approved in writing by the state

You may need to use additional methods (e.g., newspaper, delivery of multiple copies to hospitals, clinics, or apartment buildings), since notice must be provided in a manner reasonably calculated to reach all persons served. If you post or hand deliver, print your notice on your system’s letterhead, if you have it.

The notice on the reverse is appropriate for hand delivery or for publication in a newspaper. However, you may wish to modify it before using it for a radio or television broadcast. If you do modify the notice on the reverse, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Alternative Sources of Water

If you are providing bottled water, your notice should say where it can be obtained. If you are providing bottled water, make sure it meets U.S. Food and Drug Administration (FDA) and/or state bottled water safety standards.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems that have detected a fecal indicator in their ground water source. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are increasing sampling at our sources to determine the source of the contamination.

- We are working with state officials to implement corrective actions to ensure water supplies are protected against contamination.

- We are providing water from an alternative source until the problem is resolved.

- We have discontinued use of the contaminated well and will rely on our other sources to meet demand.

- We are abandoning the contaminated well and will replace it with a well-constructed to standards.

- We are pursuing treatment options for disinfection of the water from this source.

- We have installed temporary disinfection while we pursue long-term treatment or other options to eliminate contamination from this source.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is recommended that you notify health professionals in the area of the violation. People may call their doctors with questions about how the violation may affect their health, and the doctors should have the information they need to respond appropriately. In addition, health professionals, including dentists, use tap water during their procedures and need to know of potential contamination so they can use bottled water.

It is a good idea to inform your consumers when the violation has been resolved. See Template 1-10 for a “problem corrected” notice template.

Fecal Indicator-Positive Source Sample Notice – Template 1-9

DRINKING WATER WARNING

[Water System Name]’s [Source] Tested Positive for Fecal Indicator

BOIL YOUR WATER BEFORE USING

Our water system recently detected a fecal indicator [give contaminant, e.g., E. coli, enterococci, coliphage] (see definition below) in [source]. As our customers, you have a right to know what happened and what we are doing to correct this situation. On [give date], we collected a sample from [source]. The sample tested positive for [give contaminant, e.g., E. coli, enterococci, coliphage].

What should I do? What does this mean?

- DO NOT DRINK THE WATER WITHOUT BOILING IT FIRST. Bring all water to a boil, let it boil for one minute, and let it cool before using, or use bottled water. Boiled or bottled water should be used for drinking, making ice, brushing teeth, washing dishes, and food preparation until further notice. Boiling kills bacteria and other organisms in the water.

- *Fecal indicators are microbes whose presence indicates that the water may be contaminated with human or animal wastes. Microbes in these wastes can cause short-term health effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a special health risk for infants, young children, some of the elderly, and people with severely compromised immune systems. *

- The symptoms above are not caused only by organisms in drinking water. If you experience any of these symptoms and they persist, you may want to seek medical advice. People at increased risk should seek advice from their healthcare providers about drinking this water.

What is being done?

[Describe corrective action.] We will inform you when tests show no [give contaminant, e.g., E. coli, enterococci, coliphage] and you no longer need to boil your water. We anticipate resolving the problem within [estimated time frame].

For more information, please contact [name of contact] at [phone number] or [mailing address]. General guidelines on ways to lessen the risk of infection by microbes are available from the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail. *

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for “Problem Corrected” – Template 1-10

Template on Reverse

It is a good idea to issue a notice when a serious violation or situation has been resolved. Although EPA regulations do not require such notices, your state may require you to issue one. You should coordinate with your local health department as well. Below are some recommended methods for a “problem corrected” notice. You should use the same delivery methods you used for the original notice.

- Radio

- Television

- Hand or direct delivery

- Posting in conspicuous locations

- Another method approved in writing by the state

You may need to use additional methods (e.g., newspaper, delivery of multiple copies to hospitals, clinics, or apartment buildings), since notice must be provided in a manner reasonably calculated to reach all persons served. If you post or hand deliver, print your notice on your system’s letterhead, if you have it.

The notice on the reverse is very general and can be used for any violation or situation. However, to help restore consumers’ confidence in the water system, you should modify the notice to fit your situation. Although the public should have seen your initial notice, there may be additional information you learned after the notice was issued. Therefore, you should describe the violation or situation again and discuss how the problem was solved.

“Problem Corrected” Notice – Template 1-10

DRINKING WATER PROBLEM CORRECTED

Customers of [water system name] were notified on [give date] of a problem with our drinking water and were advised to [describe recommended action]. We are pleased to report that the problem has been corrected and that it is no longer necessary to [describe recommended action]. We apologize for any inconvenience and thank you for your patience.

[Add further details here when appropriate.]

As always, you may contact [contact name] at [phone number] or [mailing address] with any comments or questions.

Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.