2017 Construction General Permit

Inspection Report Template – Field Version

Purpose

This Inspection Report Template (or “template”) is to assist you in preparing inspection reports for EPA’s 2017 Construction General Permit (CGP). If you are covered under the 2017 CGP, you can use this template to create an inspection report form that is customized to the specific circumstances of your site and that complies with the minimum reporting requirements of Part 4.7 of the permit. Note that the use of this form is optional; you may use your own inspection report form provided it includes the minimum information required in Part 4.7 of the CGP.

If you are covered under a state CGP, this template may be helpful in developing a form that can be used for that permit; however, it will need to be modified to meet the specific requirements of that permit. If your permitting authority requires you to use a specific inspection report form, you should not use this form.

Notes:

While EPA has made every effort to ensure the accuracy of all instructions contained in the Inspection Report Template, it is the permit, not the template, that determines the actual obligations of regulated construction stormwater discharges. In the event of a conflict between the Inspection Report Template and any corresponding provision of the 2017 CGP, you must abide by the requirements in the permit. EPA welcomes comments on the Inspection Report Template at any time and will consider those comments in any future revision of this document. You may contact EPA for CGP-related inquiries at cgp@epa.gov.

Overview of Inspection Requirements (see CGP Part 4)

Construction operators covered under the 2017 CGP are subject to the following inspection requirements:

Person(s) Responsible for Inspecting the Site (see Part 4.1)

The person(s) inspecting your site must be a “qualified person” who may be either on your staff or a third party you hire to conduct such inspections.

- A “qualified person” is a person knowledgeable in the principles and practice of erosion and sediment controls and pollution prevention, who possesses the appropriate skills and training to assess conditions at the construction site that could impact stormwater quality, and the appropriate skills and training to assess the effectiveness of any stormwater controls selected and installed to meet the requirements of this permit.

Inspection Frequency (see Part 4.2)

You are required to conduct inspections either:

- Once every 7 calendar days; or

- Once every 14 calendar days and within 24 hours of a storm event of 0.25 inches or greater or the occurrence of runoff from snowmelt sufficient to cause a discharge.

Your inspection frequency is increased if the site discharges to a sensitive water. See Part 4.3. Your inspection frequency may be decreased to account for stabilized areas, or for arid, semi-arid, or drought-stricken conditions, or for frozen conditions. See Part 4.4.

Areas That Need to Be Inspected (see Part 4.5)

During each inspection, you must inspect the following areas of your site:

- Cleared, graded, or excavated areas of the site;

- Stormwater controls (e.g., perimeter controls, sediment basins, inlets, exit points etc.) and pollution prevention practices (e.g., pollution prevention practices for vehicle fueling/maintenance and washing, construction product storage, handling, and disposal, etc.) at the site;

- Material, waste, or borrow areas covered by the permit, and equipment storage and maintenance areas;

- Areas where stormwater flows within the site;

- Stormwater discharge points; and

- Areas where stabilization has been implemented.

What to Check For During Your Inspection (see Part 4.6)

During your site inspection, you are required to check:

- Whether stormwater controls or pollution prevention practices are properly installed, require maintenance or corrective action, or whether new or modified controls are required;

- For the presence of conditions that could lead to spills, leaks, or other pollutant accumulations and discharges;

- For locations where new or modified stormwater controls are necessary to meet requirements of the permit;

- Whether there are visible signs of erosion and sediment accumulation at points of discharge and to the channels and streambanks that are in the immediate vicinity of the discharge;

- If a stormwater discharge is occurring at the time of the inspection, whether there are obvious, visual signs of pollutant discharges; and

- If any permit violations have occurred on the site.

Inspection Reports (see Part 4.7)

Within 24 hours of completing each inspection, you are required to complete an inspection report that includes:

- Date of inspection;

- Names and titles of person(s) conducting the inspection;

- Summary of inspection findings;

- Rain gauge or weather station readings if your inspection is triggered by the 0.25-inch storm threshold; and

- If you determine that a portion of your site is unsafe to access for the inspection, documentation of what conditions prevented the inspection and where these conditions occurred on the site

Instructions for Using This Template

This Field Version of the Inspection Report Template is intended to be used in the field and filled out by hand. If you will be filling out the Inspection Report Template electronically (i.e., you will be typing in your findings), please use the Electronic Version of the Inspection Report Template available at https://www.epa.gov/npdes/stormwater-discharges-construction-activities#resources. The Electronic Version includes text fields with instructions for what to enter.

Keep in mind that this document is a template and not an “off-the-shelf” inspection report that is ready to use without some modification. You must first customize this form to include the specifics of your project in order for it to be useable for your inspection reports. Once you have entered all of your site-specific information into these fields, you may print out this form for use in the field to complete inspection reports.

The following tips for using this template will help you ensure that the minimum permit requirements are met:

- Review the inspection requirements. Before you start developing your inspection report form, read the CGP’s Part 4 inspection requirements. This will ensure that you have a working understanding of the permit’s underlying inspection requirements.

- Complete all required text fields. Fill out all text fields. Only by filling out all fields will the template be compliant with the requirements of the permit. (Note:  Where you do not need the number of rows provided in the template form for your inspection, you may leave those rows blank. Or, if you need more space to document your findings, you may add an additional sheet.)

- Use your site map to document inspection findings. In several places in the template, you are directed to specify the location of certain features of your site, including where stormwater controls are installed and where you will be stabilizing exposed soil. You are also asked to fill in location information for unsafe conditions and the locations of any discharges occurring during your inspections. Where you are asked for location information, EPA encourages you to reference the point on your SWPPP site map that corresponds to the requested location on the inspection form. Using the site map as a tool in this way will help you conduct efficient inspections, will assist you in evaluating problems found, and will ensure proper documentation.

- Sign and certify each inspection report. The operator or a duly authorized representative (see Appendix I, Part I.11.2) must sign and certify each inspection report for it to be considered complete. Where a contractor or subcontractor carries out your inspections, it is recommended that you also have the inspector sign and certify the form, in addition to the signature and certification required of the permitted operator. The template includes a signature block for both parties.

- Include the inspection form with your SWPPP. Once your form is complete, make sure to include a copy of the inspection form in your SWPPP in accordance with Part 7.2.7.e of the CGP.

- Retain copies of all inspection reports with your records. You must also retain in your records copies of all inspection reports in accordance with the requirements in Part 4.7.3 of the 2017 CGP. These reports must be retained for at least 3 years from the date your permit coverage expires or is terminated.

Section-by-Section Instructions

You will find specific instructions corresponding to each section of the report form on the reverse side of each page. These instructions provide you with more details in terms of what EPA expects to be documented in these reports.

| General Information (see reverse for instructions) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name of Project |  |  | NPDES ID No. |  | Inspection Date |  |
| Weather conditions during inspection |  |  | Inspection start time |  | Inspection end time |  |
| Inspector Name, Title & Contact Information |  |  |  |  |  |  |
| Present Phase of Construction |  |  |  |  |  |  |
| Inspection Location (if multiple inspections are required, specify location where this inspection is being conducted) |  |  |  |  |  |  |
| Inspection Frequency (Note: you may be subject to different inspection frequencies in different areas of the site. Check all that apply) Standard Frequency: Every 7 days Every 14 days and within 24 hours of a 0.25” rain or the occurrence of runoff from snowmelt sufficient to cause a discharge Increased Frequency: Every 7 days and within 24 hours of a 0.25” rain (for areas of sites discharging to sediment or nutrient-impaired waters or to waters designated as Tier 2, Tier 2.5, or Tier 3) Reduced Frequency: Twice during first month, no more than 14 calendar days apart; then once per month after first month; (for stabilized areas) Twice during first month, no more than 14 calendar days apart; then once more within 24 hours of a 0.25” rain (for stabilized areas on “linear construction sites”) Once per month and within 24 hours of a 0.25” rain (for arid, semi-arid, or drought-stricken areas during seasonally dry periods or during drought) Once per month (for frozen conditions where earth-disturbing activities are being conducted) |  |  |  |  |  |  |
| Was this inspection triggered by a 0.25” storm event? Yes No If yes, how did you determined whether a 0.25” storm event has occurred? Rain gauge on site Weather station representative of site. Specify weather station source: Total rainfall amount that triggered the inspection (in inches): Was this inspection triggered by the occurrence of runoff from snowmelt sufficient to cause a discharge? Yes No |  |  |  |  |  |  |
| Unsafe Conditions for Inspection Did you determine that any portion of your site was unsafe for inspection per CGP Part 4.5? Yes No If “yes”, complete the following: - Describe the conditions that prevented you from conducting the inspection in this location: - Location(s) where conditions were found: |  |  |  |  |  |  |

Instructions for Filling Out “General Information” Section

Name of Project

Enter the name for the project.

NPDES ID No.

Enter the NPDES ID number that was assigned to your NOI for permit coverage.

Inspection Date

Enter the date you conducted the inspection.

Weather Conditions During Inspection

Enter the weather conditions occurring during the inspection, e.g., sunny, overcast, light rain, heavy rain, snowing, icy, windy.

Inspection start and end times

Enter the time you started and ended the inspection.

Inspector Name, Title & Contact Information

Provide the name of the person(s) (either a member of your company’s staff or a contractor or subcontractor) that conducted this inspection. Provide the inspector’s name, title, and contact information as directed in the form.

Present Phase of Construction

If this project is being completed in more than one phase, indicate which phase it is currently in.

Inspection Location

If your project has multiple locations where you conduct separate inspections, specify the location where this inspection is being conducted. If only one inspection is conducted for your entire project, enter “Entire Site.” If necessary, complete additional inspection report forms for each separate inspection location.

Inspection Frequency

Check the box that describes the inspection frequency that applies to you. Note that you may be subject to different inspection frequencies in different areas of your site. If your project does not discharge to a “sensitive water” (i.e., a water impaired for sediment or nutrients, or listed as Tier 2, 2.5, or 3 by your state or tribe) and you are not affected by any of the circumstances described in CGP Part 4.4, then you can choose your frequency based on CGP Part 4.2 – either every 7 calendar days, or every 14 calendar days and within 24 hours of a 0.25-inch storm event. For any portion of your site that discharges to a sensitive water, your inspection frequency for that area is fixed under CGP Part 4.3 at every 7calendar days and within 24 hours of a 0.25-inch storm event. If portions of your site are stabilized, are located in arid, semi-arid, or drought-stricken areas, or are subject to frozen conditions, consult CGP Part 4.4 for the applicable inspection frequency. Check all the inspection frequencies that apply to your project.

Was This Inspection Triggered by a 0.25 Inch Storm Event or the occurrence of runoff from snowmelt sufficient to cause a discharge?

If you were required to conduct this inspection because of a 0.25-inch (or greater) rain event, indicate whether you relied on an on-site rain gauge or a nearby weather station (and where the weather station is located). Also, specify the total amount of rainfall for this specific storm event. If you were required to conduct this inspection because of the occurrence of runoff from snowmelt, then check the appropriate box.

Unsafe Conditions for Inspection

Inspections are not required where a portion of the site or the entire site is subject to unsafe conditions. See CGP Part 4.5. These conditions should not regularly occur, and should not be consistently present on a site. Generally, unsafe conditions are those that render the site (or a portion of it) inaccessible or that would pose a significant probability of injury to applicable personnel. Examples could include severe storm or flood conditions, high winds, and downed electrical wires.

If your site, or a portion of it, is affected by unsafe conditions during the time of your inspection, provide a description of the conditions that prevented you from conducting the inspection and what parts of the site were affected. If the entire site was considered unsafe, specify the location as “Entire site”

| Condition and Effectiveness of Erosion and Sediment (E&S) Controls (CGP Part 2.2) (see reverse for instructions) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type/Location of E&S Control [Add an additional sheet if necessary] | Maintenance Needed?* | Corrective Action Required?* | Date on Which Maintenance or Corrective Action First Identified? | Notes |
|  | Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No | Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No |  |  |

* Note: The permit differentiates between conditions requiring routine maintenance, and those requiring corrective action. The permit requires maintenance in order to keep controls in effective operating condition. Corrective actions are triggered only for specific conditions, which include: 1) A stormwater control needs repair or replacement (beyond routine maintenance) if it is not operating as intended; 2) A stormwater control necessary to comply with the permit was never installed or was installed incorrectly; 3) You become aware that the stormwater controls you have installed and are maintaining are not effective enough for the discharge to meet applicable water quality standards or applicable requirements in Part 3.1; 4) One of the prohibited discharges in Part 1.3 is occurring or has occurred; or 5) EPA requires corrective actions as a result of a permit violation found during an inspection carried out under Part 4.8. If a condition on your site requires a corrective action, you must also fill out a corrective action form found at https://www.epa.gov/npdes/stormwater-discharges-construction-activities#resources. See Part 5 of the permit for more information.

Instructions for Filling Out the “Erosion and Sediment Control” Table

Type and Location of E&S Controls

Provide a list of all erosion and sediment (E&S) controls that your SWPPP indicates will be installed and implemented at your site. This list must include at a minimum all E&S controls required by CGP Part 2.2. Include also any natural buffers established under CGP Part 2.2.1. Buffer requirements apply if your project’s earth-disturbing activities will occur within 50 feet of a water of the U.S. You may group your E&S controls on your form if you have several of the same type of controls (e.g., you may group “Inlet Protection Measures”, “Perimeter Controls”, and “Stockpile Controls” together on one line), but if there are any problems with a specific control, you must separately identify the location of the control, whether maintenance or corrective action is necessary, and in the notes section you must describe the specifics about the problem you observed.

Maintenance Needed?

Answer “yes” if the E&S control requires maintenance due to normal wear and tear in order for the control to continue operating effectively. At a minimum, maintenance is required in the following specific instances:  (1) for perimeter controls, whenever sediment has accumulated to half or more the above-ground height of the control (CGP Part 2.2.3.a); (2) where sediment has been tracked-out onto the surface of off-site streets or other paved areas (CGP Part 2.2.4); (3) for inlet protection measures, when sediment accumulates, the filter becomes clogged, and/or performance is compromised (CGP Part 2.2.10); and (4) for sediment basins, as necessary to maintain at least half  of the design capacity of the basin (CGP Part 2.2.12.f). Note:  In many cases, “yes” answers are expected and indicate a project with an active operation and maintenance program. You should also answer “yes” if work to fix the problem is still ongoing from the previous inspection.

Corrective Action Needed?

Answer “yes” if during your inspection you found any of the following conditions to be present (CGP, Part 5.1):  (1) a required E&S control needs repair or replacement (beyond routine maintenance required under Part 2.1.4); (2) a require E&S control was never installed or was installed incorrectly; (3) you become aware that the inadequacy of the E&S control has led to an exceedance of an applicable water quality standard; (4) one of the prohibited discharges in Part 1.3 is occurring or has occurred; or (5) EPA requires corrective action for an E&S control as a result of a permit violation found during an inspection carried out under Part 4.8. If you answer “yes”, you must take corrective action and complete a corrective action report, found at https://www.epa.gov/npdes/stormwater-discharges-construction-activities#resources.. Note:  You should answer “yes” if work to fix the problem from a previous inspection is still ongoing.

Date on Which Maintenance or Corrective Action First Identified?

Provide the date on which the condition that triggered the need for maintenance or corrective action was first identified. If the condition was just discovered during this inspection, enter the inspection date. If the condition is a carryover from a previous inspection, enter the original date of the condition’s discovery.

Notes

For each E&S control and the area immediately surrounding it, note whether the control is properly installed and whether it appears to be working to minimize sediment discharge. Describe any problem conditions you observed such as the following, and why you think they occurred as well as actions (e.g., maintenance or corrective action) you will take or have taken to fix the problem:

- Failure to install or to properly install a required E&S control

- Damage or destruction to an E&S control caused by vehicles, equipment, or personnel, a storm event, or other event

- Mud or sediment deposits found downslope from E&S controls

- Sediment tracked out onto paved areas by vehicles leaving construction site

- Noticeable erosion at discharge outlets or at adjacent streambanks or channels

- Erosion of the site’s sloped areas (e.g., formation of rills or gullies)

- E&S control is no longer working due to lack of maintenance

For buffer areas, make note of whether they are marked off as required, whether there are signs of construction disturbance within the buffer, which is prohibited under the CGP, and whether there are visible signs of erosion resulting from discharges through the area.

If maintenance or corrective action is required, briefly note the reason. If maintenance or corrective action have been completed, make a note of the date it was completed and what was done. If corrective action is required, note that you will need to complete a separate corrective action report describing the condition and your work to fix the problem.

| Condition and Effectiveness of Pollution Prevention (P2) Practices (CGP Part 2.3) (see reverse for instructions) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type/Location of P2 Practices [Add an additional sheet if necessary] | Maintenance Needed?* | Corrective Action Required?* | Date on Which Maintenance or Corrective Action First Identified? | Notes |
|  | Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No | Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No Yes No |  |  |

* Note: The permit differentiates between conditions requiring routine maintenance, and those requiring corrective action. The permit requires maintenance in order to keep controls in effective operating condition. Corrective actions are triggered only for specific conditions, which include: 1) A stormwater control needs repair or replacement (beyond routine maintenance) if it is not operating as intended; 2) A stormwater control necessary to comply with the permit was never installed or was installed incorrectly; 3) You become aware that the stormwater controls you have installed and are maintaining are not effective enough for the discharge to meet applicable water quality standards or applicable requirements in Part 3.1; 4) One of the prohibited discharges in Part 1.3 is occurring or has occurred; or 5) EPA requires corrective actions as a result of a permit violation found during an inspection carried out under Part 4.8. If a condition on your site requires a corrective action, you must also fill out a corrective action form found at https://www.epa.gov/npdes/stormwater-discharges-construction-activities#resources. See Part 5 of the permit for more information.

Instructions for Filling Out the “Pollution Prevention (P2) Practice” Table

Type and Location of P2 Controls

Provide a list of all pollution prevention (P2) practices that are implemented at your site. This list must include all P2 practices required by Part 2.3, and those that are described in your SWPPP.

Maintenance Needed?

Answer “yes” if the P2 practice requires maintenance due to normal wear and tear in order for the control to continue operating effectively. Note:  In many cases, “yes” answers are expected and indicate a project with an active operation and maintenance program.

Corrective Action Needed?

Answer “yes” if during your inspection you found any of the following conditions to be present (CGP, Part 5.1):  (1) a required P2 practice needs repair or replacement (beyond routine maintenance required under Part 2.1.4); (2) a require P2 practice was never installed or was installed incorrectly; (3) you become aware that the inadequacy of the P2 practice has led to an exceedance of an applicable water quality standard; (4) one of the “prohibited discharges” listed in CGP Part 1.3 is occurring or has occurred, or (5) EPA requires corrective action for a P2 practice as a result of a permit violation found during an inspection carried out under Part 4.8. If you answer “yes”, you must take corrective action and complete a corrective action report (see https://www.epa.gov/npdes/stormwater-discharges-construction-activities#resources). Note:  You should answer “yes” if work to fix the problem from a previous inspection is still ongoing.

Date on Which Maintenance or Corrective Action First Identified?

Provide the date on which the condition that triggered the need for maintenance or corrective action was first identified. If the condition was just discovered during this inspection, enter the inspection date. If the condition is a carryover from a previous inspection, enter the original date of the condition’s discovery.

Notes

For each P2 control and the area immediately surrounding it, note whether the control is properly installed, whether it appears to be working to minimize or eliminate pollutant discharges, and whether maintenance or corrective action is required. Describe problem conditions you observed such as the following, and why you think they occurred, as well as actions you will take or have taken to fix the problem:

- Failure to install or to properly install a required P2 control

- Damage or destruction to a P2 control caused by vehicles, equipment, or personnel, or a storm event

- Evidence of a spill, leak, or other type of pollutant discharge, or failure to have properly cleaned up a previous spill, leak, or other type of pollutant discharge

- Spill response supplies are absent, insufficient, or not where they are supposed to be located

- Improper storage, handling, or disposal of chemicals, building materials or products, fuels, or wastes

- P2 practice is no longer working due to lack of maintenance

If maintenance or corrective action is required, briefly note the reason. If maintenance or corrective action have been completed, make a note of the date it was completed and what was done. If corrective action is required, note that you will need to complete a separate corrective action report describing the condition and your work to fix the problem.

| Stabilization of Exposed Soil (CGP Part 2.2.14) (see reverse for instructions) |  |  |  |
| --- | --- | --- | --- |
| Stabilization Area [Add an additional sheet if necessary] | Stabilization Method | Have You Initiated Stabilization? | Notes |
|  |  | YES NO If yes, provide date: YES NO If yes, provide date: YES NO If yes, provide date: YES NO If yes, provide date: YES NO If yes, provide date: |  |

| Description of Discharges (CGP Part 4.6.6) (see reverse for instructions) |  |
| --- | --- |
| Was a stormwater discharge or other discharge occurring from any part of your site at the time of the inspection? Yes No If “yes”, provide the following information for each point of discharge: |  |
| Discharge Location [Add an additional sheet if necessary] | Observations |
| 1. | Describe the discharge: At points of discharge and the channels and banks of waters of the U.S. in the immediate vicinity, are there any visible signs of erosion and/or sediment accumulation that can be attributed to your discharge? Yes No If yes, describe what you see, specify the location(s) where these conditions were found, and indicate whether modification, maintenance, or corrective action is needed to resolve the issue: |
| 2. | Describe the discharge: At points of discharge and the channels and banks of waters of the U.S. in the immediate vicinity, are there any visible signs of erosion and/or sediment accumulation that can be attributed to your discharge? Yes No If yes, describe what you see, specify the location(s) where these conditions were found, and indicate whether modification, maintenance, or corrective action is needed to resolve the issue: |

Instructions for Filling Out the “Stabilization of Exposed Soil” Table

Stabilization Area

List all areas where soil stabilization is required to begin because construction work in that area has permanently stopped or temporarily stopped (i.e., work will stop for 14 or more days), and all areas where stabilization has been implemented.

Stabilization Method

For each area, specify the method of stabilization (e.g., hydroseed, sod, planted vegetation, erosion control blanket, mulch, rock).

Have You Initiated Stabilization

For each area, indicate whether stabilization has been initiated.

Notes

For each area where stabilization has been initiated, describe the progress that has been made, and what additional actions are necessary to complete stabilization. Note the effectiveness of stabilization in preventing erosion. If stabilization has been initiated but not completed, make a note of the date it is to be completed. If stabilization has been completed, make a note of the date it was completed. If stabilization has not yet been initiated, make a note of the date it is to be initiated, and the date it is to be completed.

Instructions for Filling Out the “Description of Discharges” Table

You are only required to complete this section if a discharge is occurring at the time of the inspection.

Was a Stormwater Discharge Occurring From Any Part of Your Site At The Time of the Inspection?

During your inspection, examine all points of discharge from your site, and determine whether a discharge is occurring. If there is a discharge, answer “yes” and complete the questions below regarding the specific discharge. If there is not a discharge, answer “no” and skip to the next page.

Discharge Location (repeat as necessary if there are multiple points of discharge)

Location of discharge. Specify the location on your site where the discharge is occurring. The location may be an outlet from a stormwater control or constructed stormwater channel, a discharge into a storm sewer inlet, or a specific point on the site. Be as specific as possible; it is recommended that you refer to a precise point on your site map.

Describe the discharge. Include a specific description of any noteworthy characteristics of the discharge such as color; odor; floating, settled, or suspended solids; foam; oil sheen; and other obvious pollution indicators.

Are there visible signs of erosion or sediment accumulation?  At each point of discharge and the channel and streambank in the immediate vicinity, visually assess whether there are any obvious signs of erosion and/or sediment accumulation that can be attributed to your discharge. If you answer “yes”, include a description in the space provided of the erosion and sediment deposition that you have found, specify where on the site or in the water of the U.S. it is found, and indicate whether modification, maintenance, or corrective action is needed to resolve the issue.

Contractor or Subcontractor Signature and Certification (see reverse for instructions)

“I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gathered and evaluated the information submitted. Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete. I have no personal knowledge that the information submitted is other than true, accurate, and complete. I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.” Signature of Contractor or Subcontractor: ________________________________________________ Date: Printed Name and Affiliation: ________________________________________________

Operator Signature and Certification (see reverse for instructions)

“I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gathered and evaluated the information submitted. Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete. I have no personal knowledge that the information submitted is other than true, accurate, and complete. I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.” Signature of Operator or “Duly Authorized Representative”: ________________________________________________ Date: Printed Name and Affiliation: ________________________________________________

Instructions for Signature/Certification

Each inspection report must be signed and certified to be considered complete.

Contractor or Subcontractor Signature and Certification

Where you rely on a contractor or subcontractor to carry out the inspection and complete the inspection report, you should require the inspector to sign and certify each report. Note that this does not relieve you, the permitted operator, of the requirement to sign and certify the inspection report as well.

Operator Signature and Certification

At a minimum, the inspection report must be signed by either (1) the person who signed the NOI, or (2) a duly authorized representative of that person. The following requirements apply to scenarios (1) and (2):

If the signatory will be the person who signed the NOI for permit coverage, as a reminder, that person must be one of the following types of individuals:

- For a corporation: A responsible corporate officer. For the purpose of this subsection, a responsible corporate officer means: (i) a president, secretary, treasurer, or vice-president of the corporation in charge of a principal business function, or any other person who performs similar policy- or decision-making functions for the corporation, or (ii) the manager of one or more manufacturing, production, or operating facilities, provided, the manager is authorized to make management decisions which govern the operation of the regulated facility including having the explicit or implicit duty of making major capital investment recommendations, and initiating and directing other comprehensive measures to assure long term environmental compliance with environmental laws and regulations; the manager can ensure that the necessary systems are established or actions taken to gather complete and accurate information for permit application requirements; and where authority to sign documents has been assigned or delegated to the manager in accordance with corporate procedures.

- For a partnership or sole proprietorship: A general partner or the proprietor, respectively.

- For a municipality, state, federal, or other public agency: Either a principal executive officer or ranking elected official. For purposes of this subsection, a principal executive officer of a federal agency includes (i) the chief executive officer of the agency, or (ii) a senior executive officer having responsibility for the overall operations of a principal geographic unit of the agency (e.g., Regional Administrator of EPA).

If the signatory will be a duly authorized representative, the following requirements must be met:

- The authorization is made in writing by the person who signed the NOI (see above);

- The authorization specifies either an individual or a position having responsibility for the overall operation of the regulated facility or activity such as the position of plant manager, operator of a well or a well field, superintendent, position of equivalent responsibility, or an individual or position having overall responsibility for environmental matters for the company. (A duly authorized representative may thus be either a named individual or any individual occupying a named position); and

- The signed and dated written authorization is included in the SWPPP. A copy must be submitted to EPA, if requested.