NORTH AMERICAN RESEARCH STRATEGY

for TROPOSPHERIC OZONE

ANALYSIS OF DATA FROM THE 1995 NARSTO-NORTHEAST STUDY

VOLUME-III:  CHEMICAL MASS BALANCE RECEPTOR MODELING

FINAL REPORT

CRC Project A-17

Prepared for:

Coordinating Research Council

219 Perimeter Center Parkway

Atlanta, GA 30346

Under Subcontract to:

ENVIRON

Golden Gate Plaza

101 Rowland Way, Suite 220

Novato, CA 94945-5010

Prepared by:

Eric M. Fujita and Zhiqiang Lu

Energy and Environmental Engineering Center

Desert Research Institute

P.O. Box 60220

Reno, NV 89506

July 31, 1998

PREFACE

High ozone concentrations in the Northeast are the result of a complex interplay between precursor emissions and meteorological conditions.  To better understand these factors, a major field data collection program was undertaken in the northeastern United States under the auspices of NARSTO-Northeast.  NARSTO (the North American Research Strategy for Tropospheric Ozone) is a public/private partnership formed to conduct research on ground-level ozone concentrations in North America.  While most data collection efforts during NARSTO-Northeast were focussed within and just to the west of the urban corridor running from Washington, DC to Boston, the study area included all of New England and stretched as far west as Ohio and as far south as southern Virginia.  An extensive database of routine and supplemental surface and upper-air meteorological and air quality observations was compiled as a result of this effort.  Among the many key features of this database are the extensive speciated VOC measurements taken both at the surface and aloft.

The NARSTO-Northeast database offers a unique opportunity to study the factors influencing ozone formation in this part of the country.  In recognition of this opportunity, in 1996 the Coordinated Research Council (CRC) launched a multi-component data analysis project which represents the first comprehensive look at the NARSTO-Northeast data.  Major objectives of these data analyses were to summarize the validated NARSTO Northeast data and to draw conclusions regarding mechanisms of ozone formation and transport in the Northeast, especially with respect to the relative importance of VOC and NOx emissions, the role of transported ozone and precursors, and the importance of biogenic VOC and regional NOx.

Results of the CRC NARSTO Northeast data analysis project are presented in five volumes:

Analysis of Data from the 1995 NARSTO-Northeast Study.  Volume-I: Data Validation and Statistical Summaries of Routine Data.  T.E. Stoeckenius et al., ENVIRON International Corporation, Novato, CA. and Main et al., Sonoma Technology Inc., Santa Rosa, CA., 1998.

Analysis of Data from the 1995 NARSTO-Northeast Study.  Volume-II: Use of PAMS Data to Evaluate a Regional Emission Inventory.  T.L.Haste et al., Sonoma Technology Inc. Santa Rosa, CA., March 1998.

Analysis of Data from the 1995 NARSTO-Northeast Study.  Volume-III:  Chemical Mass Balance Receptor Modeling.  E.Fujita et al., Desert Research Institute, Reno, NV., 1998.

Analysis of Data from the 1995 NARSTO-Northeast Study.  Volume-IV: Observation-Driven Methods for Delineating VOC and NOx Limitation.  C.Blanchard, Envair, Albany, CA., February 1998.

Analysis of Data from the 1995 NARSTO-Northeast Study.  Volume-V: Three-Dimensional Analysis of Aircraft and Upper-Air Meteorological Data.  P.T.Roberts et al., Sonoma Technology Inc. Santa Rosa, CA., 1998.

TABLE OF CONTENTS

Page No.

Preface		ii

Table of Contents	iii

List of Tables	iv

List of Figures	v

1.0	INTRODUCTION	1-1

1.1	Objectives and Scope of Work	1-2

2.0	CHEMICAL MASS BALANCE RECEPTOR MODELING	2-1

2.1	Fundamentals	2-1

2.2	CMB Version 8	2-2

2.3	Evaluation of Ambient Data and Candidate Fitting Species	2-3

3.0	SOURCE COMPOSITION PROFILE DEVELOPMENT AND EVALUATION	3-1

  - Derivation of Spark-Ignition Vehicle Exhaust Profiles from Tunnel

Measurements	3-11

3.2	Compilation of Profiles from Previous Studies	3-15

3.3	Evaluation of Source Profiles	3-16

4.0	SOURCE APPORTIONMENT OF PAMS HYDROCARBON DATA	4-1

4.1	Average Source Contribution Estimates	4-1

  - Diurnal and Day-of-the-Week Variations in Source Contribution

Estimates	4-5

4.3	Wind Directional Dependence of Source Contributions	4-5

- SOURCE APPORTIONMENT OF NARSTO-NORTHEAST CANISTER

DATA	5-1

5.1	Source Contribution Estimates for Surface Samples	5-1

5.2	Source Contribution Estimates for Aloft Samples	5-10

6.0	SUMMARY AND CONCLUSIONS	6-1

7.0	REFERENCES	7-1

Appendix A:  Individual Source Contribution Estimates to NMHC at Six Surface

Canister Sites	A-1

Appendix B:  Individual Source Contribution Estimates to NMHC at Aircraft Sites	B-1

LIST OF TABLES

Page

Table 1-1	PAMS Sites in NARSTO-Northeast Study Area	 1-3

Table 1-2	Target Ozone Precursors for PAMS	 1-4

Table 2-1	Target List of Hydrocarbons for Photochemical Assessment Monitoring Stations	 2-5

Table 2-2	Percentage of Valid Measurements for the Ten PAMS Sites	 2-6

Table 2-3	Default Fitting Species Used in CMB Modeling	 2-8

Table 2-4	Numbers of Valid Hydrocarbon Samples	2-9

Table 3-1	Source profiles compiled for NARSTO-Northeast Source Apportionment	 3-2

Table 3-2	Source profiles developed for Northeast project	 3-3

Table 3-3	Apportionment of Motor Vehicle Exhaust and Running Evaporative Loss in the Callahan and Lincoln Tunnels	3-12

Table 3-4	CMB sensitivity test for various exhaust profiles 	3-17

Table 3-5	CMB sensitivity test for other source profiles 	3-18

Table 4-1	24-Hour average Source Contributions to total NMHC for PAMS Measurements (6/1/95 to 8/31/95)	 4-2

Table 4-2	Average hourly Source Contributions for PAMS Measurements (6/1/95 to 8/31/95)	 4-6

Table 5-1	Average source contributions to TNMHC at six surface canister sites	 5-2

Table 5-2	Average source contributions to TNMHC at aircraft sites	5-11

LIST OF FIGURES

Page

Figure 4-1	24-hour average source contribution estimates (ug/m3 and % NMHC) during summer 1995 	4-3

Figure 4-2 	Average source contribution estimates by time of day at PAMS sites	4-11

Figure 4-3a	Average SCE at E. Hartford by day of the week, summer 1995 (EDT).	4-16

Figure 4-3b	Average SCE at McMillan Reservoir by day of the week, summer 1995 (EDT).	4-18

Figure 4-3c	Average SCE at Lums Pond by day of the week, summer 1995 (EDT).	4-20

Figure 4-3d	Average SCE at Chicopee, MA by day of the week, summer 1995 (EDT).	4-22

Figure 4-3e	Average SCE at Lynn, MA by day of the week, summer 1995 (EDT).	4-24

Figure 4-3f	Average SCE at Lake Clifton, MD by day of the week, summer 1995 (EDT).	4-26

Figure 4-3g	Average SCE at Rider College, NJ by day of the week, summer 1995 (EDT).	4-27

Figure 4-3h	Average SCE at Bronx, NY by day of the week, summer 1995 (EDT).	4-29

Figure 4-3i	Average SCE at Philadelphia, PA by day of the week, summer 1995 (EDT).	4-31

Figure 4-3j	Average SCE at E. Providence, RI by day of the week, summer 1995 (EDT).	4-33

Figure 4-4a	Wind directional dependence of source contributions by time of the day at Lums Pond, DE.	4-35

Figure 4-4b	Wind directional dependence of source contributions by time of the day at Chicopee, MA.	4-36

Figure 4-4c	Wind directional dependence of source contributions by time of the day at Lynn, MA.	4-37

Figure 4-4d	Wind directional dependence of source contributions by time of the day at Lake Clifton, MD.	4-38

Figure 4-4e	Wind directional dependence of source contributions by time of the day at Rider College, NJ.	4-39

LIST OF FIGURES (continued)

Figure 4-4f	Wind directional dependence of source contributions by time of the day at Bronx, NY.	4-40

Figure 5-1a	Source contributions to NMHC at the NARSTO-Northeast site at Arendstville in 1995.	  5-3

Figure 5-1b	Source contributions to NMHC at the NARSTO-Northeast site at Brookhaven in 1995.	  5-4

Figure 5-1c	Source contributions to NMHC at the NARSTO-Northeast site at Cape Cod in 1995.	  5-5

Figure 5-1d	Source contributions to NMHC at the NARSTO-Northeast site at Holbrook in 1995.	5-6

Figure 5-1e	Source contributions to NMHC at the NARTSO-Northeast site at Kunkletown in 1995.	5-7

Figure 5-1f	Source contributions to NMHC at the NARSTO-Northeast site at Shenandoah in 1995.	5-8

Figure 5-2	Gas chromtogram of a sample from Lynn, MA on 9/14/95 at 1300-1600, EDT (Source: DRI)	5-9

Figure 5-3	Source contributions to NMHC for aircraft canister samples	5-12