Attachment 4-A: Questions to Ask Laboratory

| Is the laboratory recognized by NLLAP? | All samples must be analyzed by an NLLAP laboratory. You can contact the NLIC Clearinghouse at 1-800-424-LEAD for an up-to-date list of NLLAP-recognized laboratories. Even after selecting a laboratory, you should check the laboratory’s accreditation every 6 months. |
| --- | --- |
| What is the turnaround time for sample analysis? | The laboratory turnaround time is an important factor; labs usually provide results within 1 to 3 days. A faster turnaround time allows you to be more responsive to your client but may cost more money. |
| What is the cost per sample? | Prices can vary depending on how quickly you want the results. A 6-hour turnaround will cost more than samples analyzed over a few days. |
| Will the laboratory provide sampling materials? | Some laboratories will provide you with the materials necessary to perform sampling. You may want to select a laboratory that provides these materials because laboratory-supplied materials and forms can help minimize potential errors in the analysis and recordkeeping. |
| Can the laboratory analyze composite samples? | It is good practice to check in advance the laboratory’s capabilities in analyzing composite samples, if your client wants you to collect composite samples. Some laboratories do not offer this service. |
| Does the laboratory perform all the necessary mathematical calculations? | The Federal guidance is provided in µg/ft². Depending on the size of the sample or sample area, some mathematical calculations may need to be performed to convert the sample area to 1 square foot. Selecting a laboratory that will perform this calculation for you can reduce the possibility of mathematical errors. |

Blank Chain of Custody Form

Attachment 4-C: Worksheet for Performing Mathematical Conversions for Dust Samples

| Unit of Measurement | Symbol | Unit of Weight | Symbol |
| --- | --- | --- | --- |
| Inches | in | Micrograms | µg |
| Square inches | in² | Micrograms per square foot | µg/ft² |
| Feet | ft |  |  |
| Square feet | ft2 |  |  |

1. Convert the sample area to square feet (ft²)

If the area you sampled was not a square foot, you will need to convert it to this dimension. One foot equals 12 inches, and 1 square foot equals 144 square inches.

- Record the sample area in inches (in) as opposed to feet (ft).

- Convert the sample area to square inches (in²). If you have a decimal, round the number to three decimal places.

- Divide the square inches by 144 to get square feet (ft²). If you have a decimal, round the number to three decimal places.

| Dimensions of sample area in inches (in) | Length: ______ in Width: ______ in |
| --- | --- |
| Multiply length times width to calculate the area in square inches (in²) | ______ in × _____in = _____ in² |
| Divide the area in square inches (in²) by 144 to calculate the area in square feet (ft²) | ______ in² ÷ 144 =______ft² |

2. Convert the results to micrograms per square foot (µg/ft²)

After you have converted the sample area to square feet, you need to find the amount of lead dust contained in that area. The micrograms per square foot (µg/ft²) describe the quantity of lead dust contained in a 1 square-foot area.

- Divide the amount of lead (µg) by the area (ft2).

| Dimensions of sample area in square feet (ft²) | Area = ______ ft² |
| --- | --- |
| Quantity of lead in micrograms (µg) | Lead = ______ µg |
| Divide micrograms (µg) by square feet (ft²) to calculate micrograms per square foot (µg/ft²) | _____ µg ÷_____ ft² = _____ µg/ft² |

Attachment 4-D: Activity — Interpreting Laboratory Results

Instructions: The purpose of this activity is to test your ability to verify the results received from the laboratory, compare these results to the EPA dust clearance standards, and interpret the results. Using the following excerpt from a lead dust clearance test results form, check the laboratory’s calculation of the weighted lead dust sample.

| Sample # | Location | Surface | Dimensions of sample area (ft²) | Total lead (µg) | µg/ ft² |
| --- | --- | --- | --- | --- | --- |
| 92-1 | Upstairs bedroom | Floor | 1.025 | 10 | 9.8 |
| 92-2 | Upstairs bedroom | Interior windowsill | 0.478 | 150 | 71.7 |
| 92-3 | Kitchen, front window | Interior windowsill | 0.544 | 84 | 154.4 |
| 92-4 | Kitchen, side window | Window trough | 0.928 | 97 | 90.0 |

- Check the results (µg lead/ft²) for each sample. If the results are incorrect, provide the correct results in µg lead/ft².

- After verifying the laboratory’s results, compare these results to the appropriate EPA lead dust clearance standard. Did the individual samples pass or fail the lead dust clearance test?

EPA Clearance Standards for Lead Dust

Floors: < 10 µg/ft²

Interior windowsills: < 100 µg/ft²

Window troughs: < 400 µg/ft²

| 92-1: Result ______ | Clearance Standard: _____________ | Pass or Fail? _______ |
| --- | --- | --- |
| 92-2: Result ______ | Clearance Standard: _____________ | Pass or Fail? _______ |
| 92-3: Result ______ | Clearance Standard: _____________ | Pass or Fail? _______ |
| 92-4: Result ______ | Clearance Standard: _____________ | Pass or Fail? _______ |