Thought Starter for the Expert Public Workshop on Alternatives and Risk Reduction Approaches to Trichloroethylene (TCE)

- Introduction

TCE was identified for risk assessment as part of EPA's Existing Chemicals Management Program. EPA reviewed readily available information on TCE including uses, physical and chemical properties, fate, exposure potential, and associated hazards to humans and the environment. TCE was selected based on concerns about its human health hazard (e.g., human carcinogen) and its exposure profile (i.e., widely used in consumer products and detected in drinking water, indoor environments, surface water, ambient air, groundwater, and soil) using OPPT's TSCA Work Plan screening methodology.

- Background

  - Risk Assessment

EPA’s final risk assessment evaluated health risks to consumers and workers using TCE as a degreaser, to consumers using it as a protective coating to finish prints or artwork, and to dry cleaners' workers using it to remove stains.

This risk assessment evaluated human health risks to consumers and workers, including bystanders, from inhalation exposures. Single (acute) or short-term exposure can potentially affect the developing fetus. High acute concentrations of TCE vapors can irritate the respiratory system and skin and induce central nervous system effects such as light-headedness, drowsiness, and headaches. Repeated (chronic) or prolonged exposure to TCE has been associated with effects in the liver, kidneys, immune system, central nervous system. EPA has concerns for effects in the developing fetus from both acute and chronic exposure. TCE is carcinogenic to humans by all routes of exposure.

  - Production Volume and Uses of TCE

The majority (> 80 %) of TCE is used as an intermediate for manufacturing refrigerant chemicals. Much of the remainder, less than 14 percent, is used as a solvent for metals degreasing, leaving a relatively small percentage to account for all other uses; including its use in consumer products. In 2011, global consumption of TCE was 945 million lbs and U.S. consumption was 255 million lbs. Nine companies, including manufacturers and importers reported a total production of 224.7 million lbs of TCE in 2011 to the Chemical Data Reporting (CDR) database. Based on the Toxics Release Inventory (TRI), 28 companies use TCE as a formulation component, 27 companies use TCE in repackaging, 25 companies use TCE as a manufacturing aid, and 103 companies use TCE for ancillary uses, such as degreasing.

c. TCE Alternatives

There are available substitutes for TCE; however, there is not one alternative that works across all applications where TCE is currently used. Additional analysis is needed to determine if any of the substitutes are as effective as TCE for commercial degreasing to evaluate whether they are safer than TCE, and to determine the most feasible alternatives.

EPA has already initiated discussions with CSPA and member companies to compile information on consumer and small shop uses of TCE, and identify consumer products containing TCE. Based on preliminary information, it appears that, for some functional uses, the significant majority of product volume available for consumer purchase does not contain TCE.

- Objectives of the expert workshop

The objectives of the workshop are to generate more information from experts on TCE uses and work practices when used as a degreaser, to better understand the factors which must be considered when transitioning to safer alternatives, and to develop options to address/mitigate the risks identified in the assessment. Other uses of TCE as evaluated in the Agency’s risk assessment (fixative and dry cleaning uses) will be addressed separately. The workshop will include various presentations, discussion sessions with case studies, and opportunities for public comment.

To start the workshop, EPA will outline the goals of the workshop. Workshop experts will discuss how TCE is used as a degreaser, the process and efficacy considerations important to safer degreaser alternatives selection, present case studies that examine chemical substitution, and discuss safer engineering practices, new products, and technologies that can reduce exposure to TCE. Sessions include charge and outcome questions to guide the discussion.

EPA’s goal is to bring the experts together to better understand the challenges and opportunities when attempting to reduce exposure to TCE. EPA’s purpose is to obtain feedback on substitution strategies and risk mitigation. A key goal of the workshop is to identify criteria that can  be used to develop approaches to decrease exposure to TCE. The Agency will carefully consider all the information as it subsequently considers and develops risk reduction options from TCE when used as a degreaser and looks forward to working with the TCE stakeholder community.

- Content of the workshop

The workshop will examine the following uses:

- TCE Use in Small Commercial Settings for Degreasing--This use category will focus on small commercial shops that use TCE for degreasing, including cold cleaning, aerosol uses, and vapor degreasing.

- TCE Use by Consumers or Hobbyists for Degreasing--This use category will focus on consumers and hobbyists that use TCE as a degreaser, including cold cleaning and aerosol uses.

  - Introductory keynote talks

In the first keynote talks there will be a brief overview of the final risk assessment, goals of the workshop, and federal and state regulations of TCE.

  - Keynote talks with relationship to  sessions

The keynote talks, will include a summary of the current degreasing uses of TCE; the contaminants TCE removes, the substrates TCE is compatible with; the properties of TCE that make it an effective degreaser and preferable to alternatives; the factors that are important to the use of TCE as a degreaser; the health and safety factors to consider when using TCE; and the applications and substrates that are not appropriate for the use of TCE.

  - Stage setting – Overview of the  sessions

The sessions will focus on aerosol, cold cleaning, and vapor degreasing.

As noted above, those parameters that were determined to have the widest applicability and interest will be addressed in the keynote sessions.  These parameters are generally needed to set the stage for more detailed discussion during the plenary sessions. The objective of each session is to identify safer alternatives to TCE.

For each session, the following basic questions should be considered:

- What properties and characteristics of TCE make it effective and desirable for a given application?

- What alternative chemicals can be substituted for TCE in a given use?

- Are there critical uses where no viable chemical alternatives exist? In cases with no chemical alternatives, what other measures can be employed to reduce exposure to TCE thereby mitigating risks?

- Session on Aerosol and Cold Cleaning

This session will address issues related to the use of TCE both in an aerosol form and as a bulk liquid in small commercial and consumer settings.

  - Background: Background information on the criteria that potential alternatives will need to satisfy as replacements for TCE, lessons learned from previous substitutions, possible future substitutions, and specific applications with no viable chemical alternatives where green engineering options could help address the risks.

  - Specific Considerations: Building on EPA’s prior discussions with stakeholders, the discussion should identify TCE alternatives in automotive and/or electronic applications, as well as address flammability concerns and state regulations on consumer products.

  - Additional questions to guide discussion of case studies during the session.

- Where was TCE used and why?

- What was the impetus for seeking an alternative for TCE?

- What considerations were taken into account to identify potential alternatives?

- How was the list of potential alternatives narrowed down?

- How were alternatives evaluated for efficacy?

- How were environmental/health considerations taken into account in identifying alternatives?

- How was a final alternative selected for use?

- Were any of the alternatives found to be unsuitable and why?

- What lessons were learned in reducing exposure to TCE that could help inform future decision making?

- Session on Vapor Degreasing

This session will address issues related to the use of TCE in vapor degreasing in small commercial settings.

- Background: Background information on the criteria that potential alternatives will need to satisfy as replacements for TCE, lessons learned from previous substitutions, possible future substitutions, and specific applications with no viable chemical alternatives where green engineering options could help address the risks.

- Specific Considerations: Building on EPA’s prior discussions with stakeholders, the discussion should identify alternatives in vapor degreasing applications, as well as address engineering controls to reduce exposure when there are no viable alternatives and NESHAP regulations.

- Additional questions to guide discussion of case studies during the plenary session.

- Where was TCE used and why?

- What was the impetus for seeking an alternative for TCE?

- What considerations were taken into account to identify potential alternatives?

- How was the list of potential alternatives narrowed down?

- How were alternatives evaluated for efficacy?

- How were environmental/health considerations taken into account in identifying alternatives?

- How was a final alternative selected for use?

- Were any of the alternatives found to be unsuitable and why?

- What lessons were learned in reducing exposure to TCE that could help inform future decision making?