WISCONSIN CORN GROUNDWATER

12/23/13:

- Updated Table 3b to reflect correct maximum water capacity (half way between Porosity and Field Capacity) and revised table title.

- Updated Table 2 to reflect crop height of 100 cm to be consistent with standard PRZM surface water scenarios.

- Updated allowable depletion of available water (fraction) from 0.70 to 0.92 in Table 2. The allowable depletion in PRZM cannot be greater than 0.9; therefore, the value was capped at 0.9.

Metadata

The field used to represent corn production in Wisconsin is located in the Central Sands region of Wisconsin, in South Central Wisconsin. According to the 2002 USDA Agriculture Census, corn for grain was the second largest crop by area for the state of Wisconsin cultivating over 2.8 million acres. The Draft “Model and Scenario Development for Groundwater Estimates Using PRZM’ (Draft Scenario Guidance) was used to create a new representative vulnerable scenario for areas that may be used for private wells in the upper mid-west.

The Wisconsin Central Sands region (MLRA 89 & 91) was selected as an area that is representative of a vulnerable ground water system that may be used for private wells. Corn is grown in many other areas of the state, but this region of the south central part of the state is considered to be vulnerable with shallow depths to water, and sandy (60-80% sand) soils. Soil series information and soil data was obtained from the Natural Resources Conservation Service soil characterization data from the National Soils Characterization Database (http://soils.usda.gov/survey/nscd/), and the USDA’s soil data mart (Soil Survey Staff, 2008). Soil characterization report is included in Appendix A. Waushara and Adams counties were used because the survey area contained the most reliable data for a predominant soil series (Coloma) for the Wisconsin Central Sands region, as determined by the USDA’s Soil Data Mart (http://soildatamart.nrcs.usda.gov/) and the NRCS’s Web Soil Survey tool (http://websoilsurvey.nrcs.usda.gov).

The texture for the Coloma series is Loamy sand, and is classified as hydrologic soil group B. It is characterized as having approximately 60-80% sand, somewhat excessively drained, with rapid permeability, and slopes ranging from 0 – 70% (USDA/NRCS, 2007). The soils are in MLRA 89 and 91, covering the majority of the Central Sands Region.

| Table 1. PRZM 3.12 Climate and Time Parameters for Waushara, Adams, and Portage Counties - Corn |  |  |
| --- | --- | --- |
| Parameter | Value | Source |
| Starting Date | January 1, 1961 | Meteorological File – La Crosse, WI (W14920) |
| Ending Date | December 31, 1990 | Meteorological File – La Crosse, WI (W14920) |
| Pan Evaporation Factor (PFAC) | 0.76 | PRZM 3 Manual, Figure 5.9 (Suarez, 2006) |
| Snowmelt Factor (SFAC) | 0.36 | PRZM 3 Manual, Table 5.1 (Suarez, 2006) |
| Minimum Depth of Evaporation (ANETD) | 18 | PRZM Manual Figure 5.2 (Suarez, 2006). Use the mid-point of the range of values based on location of the crop scenario. If a crop region crosses one or more boundaries, select the average of the midpoints. |

| Table 2. PRZM 3.12 Crop Parameters for Waushara, Adams, and Portage Counties - Corn |  |  |
| --- | --- | --- |
| Parameter | Value | Source |
| Initial Crop (INICRP) | 1 | The simulation date starts before the emergence date of the crop. PRZM is currently untested for other conditions. |
| Initial Surface Condition (ISCOND) | 1 | The effect of this parameter is nearly irrelevant in EFED standard scenarios. This parameter specifies the curve number in place before the main crop is planted. |
| Number of Different Crops (NDC) | 1 |  |
| Maximum interception of Storage of Crop (CINTCP) | 0.25 cm | PRZM 3 Manual (Suarez, 2006) Table 5.4 |
| Maximum Rooting Depth (AMXDR) | 90 cm | Selection of root depth by professional judgment will be a compromise between a need to accurately describe evapotranspiration and accurately describing irrigation needs. Use USDA crop profiles. |
| Maximum Aerial Canopy Coverage (COVMAX) | 100 |  |
| Surface Condition of Crop after Harvest | 3 | PRZM 3 manual (Suarez, 2006) |
| Curve Number (CN) | 10 | Typically for groundwater scenarios, curve numbers will be low and have little effect on simulated results (e.g., from A or B soils). |
| Max Dry Weight of Crop at Full Canopy (WFMAX) | -- | Not Used in GW scenarios – place holder |
| Maximum Height of Canopy at Maturation (HTMAX) | 100 | Consistent with standard Florida Potato surface water scenario. |
| Number of Cropping Periods (NCPDS) | 30 | Set to weather data. Meteorological File – La Crosse, Wisconsin (W14920) |
| Date of Crop Emergence (EMD/EMM/IYREM) | 01/05 | http://www.ipmcenters.org/CropProfiles/docs/wicorn-sweet.pdf |
| Date of Crop Maturation (MAD/MAM/IYRMAT) | 21/07 | http://www.ipmcenters.org/CropProfiles/docs/wicorn-sweet.pdf |
| Date of Crop Harvest (HAD/HAM/IYRHAR) | 20/10 | http://www.ipmcenters.org/CropProfiles/docs/wicorn-sweet.pdf |
| Crop Number Associated with NDC | 1 | Only one crop modeled |
| IPSCND | 1 | Assume bulk of material is incorporated into ground |
| Extra Water for Leaching | 0.1 | Typical |
| Available Depletion | 0.9 | Value calculated based on updated maximum water. Calculations are included in the attached excel file “Scenario Update Calculations Dec 2013”). |
| Max Rate of Water Supplied | 4.0 | Set high enough such that demand would be met on a single day. James Wolf maintains reference. |

| Table 3a. PRZM 3.12 Coloma Soil Parameters for Waushara, Adams, and Portage Counties - Corn |  |  |
| --- | --- | --- |
| Parameter | Value | Source |
| Soil Property Title (STITLE) | Coloma Loamy Sand – Hydrologic Soil Group B | USDA/NRCS Official Soil Series Name, texture and hydrologic soil grouping |
| Total Soil Depth (CORED) | 1000 cm | Standard GW Scenarios are for 10-meter soil profiles, with the last meter simulated as an aquifer. |
| Number of Horizons (NHORIZ) | 8* | For the upper Horizons use USDA Soil Data Mart (Soil Survey Staff, 2008). Resolution need not be less than 1 cm in the top portion of the profile and not less than 10 cm in the remaining profile. The top profile is resolved into 1 cm increments in order to allow for accurate applications of pesticides into the soil surface. Below 10 cm, discretization is increased to 10 cm in order to simulate realistic dispersion. |

* Using 8 horizons greatly simplifies the procedure for entering declining degradation rates.  The first 2 horizons are 10 cm, and the next four are 20 cm thickness; these 6 horizons represent the aerobic degradation zone. The 7th horizon is variable depending on the depth to the simulated aquifer and represents the depths between the aerobic degradation zone and the water table. The 8th horizon represents the groundwater which starts at a variable depth depending on the parameterization, with the profile extending 100 cm below the water table.   Note that the pore water degradation rate should never be less than the hydrolysis rate.

| Table 3b. PRZM 3.12.2 Coloma Soil Horizon Input and Field Parameters for Waushara, Adams, and Portage Counties - Corn |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Horizon # | Horizon Thickness (THKNS) (cm) | Bulk Density (BD) (g/cm3) | Initial Soil Water Content (THETO)* | Soil Drainage Parameter (AD) | Pesticide Hydrodynamic Solute Dispersion Coefficient (DISP) | Lateral Soil Drainage Parameter (ADL) | Thickness of Compartments in Horizon (DPN) (cm) | Field Capacity (THEFC) (cm3/cm3)* | Maximum Water Capacity1 | Wilting Point (THEWP) (cm3/cm3) | Organic Carbon (OC) (%) | Sand (%) | Clay (%) |
| 1 | 10 | 1.63 | 0.088 | 0 | 0 | 0 | 1 | 0.088 | 0.24 | 0.034 | 0.46 | 95 | 2.3 |
| 2 | 10 | 1.63 | 0.088 | 0 | 0 | 0 | 5 | 0.088 | 0.24 | 0.034 | 0.46 | 95 | 2.3 |
| 3 | 20 | 1.68 | 0.079 | 0 | 0 | 0 | 20 | 0.079 | 0.22 | 0.025 | 0.14 | 96 | 2.5 |
| 4 | 20 | 1.68 | 0.079 | 0 | 0 | 0 | 20 | 0.079 | 0.22 | 0.025 | 0.09 | 96 | 2.5 |
| 5 | 20 | 1.59 | 0.068 | 0 | 0 | 0 | 20 | 0.068 | 0.23 | 0.016 | 0.09 | 97 | 1.7 |
| 6 | 20 | 1.59 | 0.068 | 0 | 0 | 0 | 20 | 0.068 | 0.23 | 0.016 | 0.08 | 97 | 1.7 |
| 7 | 800 | 1.59 | 0.068 | 0 | 0 | 0 | 50 | 0.068 | 0.23 | 0.016 | 0.07 | 97 | 1.7 |
| 8 | 100 | 1.59 | -- | 0 | 0 | 0 | 50 | -- | -- | -- | 0.07 | 97 | 1.7 |
| NRCS, National Soils Characterization Database (NRCS, 2001) http://soils.usda.gov/survey/nscd/ Draft Model and Scenario Development for Groundwater Estimates Using PRZM *THETO is set to the mid-point between field capacity and saturation. 1. 1-[bulk density/2.65 (i.e., grain density)] = porosity; average field capacity and porosity |  |  |  |  |  |  |  |  |  |  |  |  |  |

GW Temperature: 5.6º C (http://0.tqn.com/d/homerepair/1/0/o/3/-/-/climate_temps.jpg)

Albedo: 0.2

REFERENCES:

EPA. 2001. Abel, S.A. Procedure for Conducting Quality Assurance and Quality Control of Existing and New PRZM Field and Orchard Crop Standard Scenarios. Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, D.C.

Soil Survey Staff. 2008. National Soil Survey Characterization Data. Soil Survey Laboratory, National Soil Survey Center, USDA-NRCS, Lincoln, NE. Friday November 7, 2008

Suarez, L.A., 2006. PRZM-3, A Model for Predicting Pesticide and Nitrogen Fate in the Crop Root and Unsaturated Soil Zones: Users Manual for Release 3.12.2.   EPA/600/R-05/111 September 2006, revision a.

USDA/NRCS. 2007. Official Series Description – Coloma Series. Rev. AJO-GWH-MLK. April, 2007.