2008 National Emissions Inventory

Emissions Inventory System

Implementation Plan

Appendix 10

Onroad/Nonroad Activity Data References

Final

October 6, 2008

List of Figures

Page

Figure A10-1  Mobile 6 Vehicle Classifications	A10-1

Figure A10-2  Composite Vehicle Classes for Vehicle Registration Data	A10-2

Figure A10-3  Average Speed Ranges	A10-3

Figure A10-4  Indicator Codes Used for NONROAD Growth-Referenced External File	A10-3

Figure A10-5  Predefined Horsepower Ranges for NONROAD	A10-5

Figure A10-6  Allowed Distinct Average Speed Inputs	A10-6

Figure A10-7  HPMS Road Type	A10-7

Figure A10-8  File Type for County NR File Table	A10-8

[This page intentionally left blank.]

Appendix 10
Onroad/Nonroad Activity Data References

Figure A10-1 
Mobile 6 Vehicle Classifications

| Number | Abbrev. | Description |
| --- | --- | --- |
| 1 | LDGV | Light-Duty Gasoline Vehicles (Passenger Cars) |
| 2 | LDGT1 | Light-Duty Gasoline Trucks 1 (0 - 6,000 lbs. GVWR, 0 - 3,750 lbs. LVW) |
| 3 | LDGT2 | Light-Duty Gasoline Trucks 2 (0 - 6,000 lbs. GVWR, 3751 - 5,750 lbs. LVW) |
| 4 | LDGT3 | Light-Duty Gasoline Trucks 3 (6,001 - 8,500 lbs. GVWR, 0 - 5,750 lbs. ALVW) |
| 5 | LDGT4 | Light-Duty Gasoline Trucks 4 (6,001 - 8,500 lbs. GVWR, 5,751 lbs. and greater ALVW) |
| 6 | HDGV2B | Class 2b Heavy-Duty Gasoline Vehicles (8,501 - 10,000 lbs. GVWR) |
| 7 | HDGV3 | Class 3 Heavy-Duty Gasoline Vehicles (10,001 - 14,000 lbs. GVWR) |
| 8 | HDGV4 | Class 4 Heavy-Duty Gasoline Vehicles (14,001 - 16,000 lbs. GVWR) |
| 9 | HDGV5 | Class 5 Heavy-Duty Gasoline Vehicles (16,001 - 19,500 lbs. GVWR) |
| 10 | HDGV6 | Class 6 Heavy-Duty Gasoline Vehicles (19,501 - 26,000 lbs. GVWR) |
| 11 | HDGV7 | Class 7 Heavy-Duty Gasoline Vehicles (26,001 - 33,000 lbs. GVWR) |
| 12 | HDGV8A | Class 8a Heavy-Duty Gasoline Vehicles (33,001 - 60,000 lbs. GVWR) |
| 13 | HDGV8B | Class 8b Heavy-Duty Gasoline Vehicles (>60,000 lbs. GVWR) |
| 14 | LDDV | Light-Duty Diesel Vehicles (Passenger Cars) |
| 15 | LDDT12 | Light-Duty Diesel Trucks 1 and 2 (0 - 6,000 lbs. GVWR) |
| 16 | HDDV2B | Class 2b Heavy-Duty Diesel Vehicles (8,501 - 10,000 lbs. GVWR) |
| 17 | HDDV3 | Class 3 Heavy-Duty Diesel Vehicles (10,001 - 14,000 lbs. GVWR) |
| 18 | HDDV4 | Class 4 Heavy-Duty Diesel Vehicles (14,001 - 16,000 lbs. GVWR) |
| 19 | HDDV5 | Class 5 Heavy-Duty Diesel Vehicles (16,001 - 19,500 lbs. GVWR) |
| 20 | HDDV6 | Class 6 Heavy-Duty Diesel Vehicles (19,501 - 26,000 lbs. GVWR) |
| 21 | HDDV7 | Class 7 Heavy-Duty Diesel Vehicles (26,001 - 33,000 lbs. GVWR) |
| 22 | HDDV8A | Class 8a Heavy-Duty Diesel Vehicles (33,001 - 60,000 lbs. GVWR) |
| 23 | HDDV8B | Class 8b Heavy-Duty Diesel Vehicles (>60,000 lbs. GVWR) |
| 24 | MC | Motorcycles (Gasoline) |
| 25 | HDGB | Gasoline Buses (School, Transit, and Urban) |
| 26 | HDDBT | Diesel Transit and Urban Buses |
| (cont.) |  |  |
| Figure A10-1 Mobile 6 Vehicle Classifications (cont.) |  |  |
| Number | Abbrev. | Description |
| 27 | HDDBS | Diesel School Buses |
| 28 | LDDT34 | Light-Duty Diesel Trucks 3 and 4 (6,001 - 8,500 lbs. GVWR) |

Figure A10-2 
Composite Vehicle Classes for Vehicle Registration Data

| Number | Abbrev. | Description |
| --- | --- | --- |
| 1 | LDV | Light-Duty Vehicles (Passenger Cars) |
| 2 | LDT1 | Light-Duty Trucks 1 (0 - 6,000 lbs. GVWR, 0 - 3,750 lbs. LVW) |
| 3 | LDT2 | Light-Duty Trucks 2 (0 - 6,000 lbs. GVWR, 3,751 - 5,750 lbs. LVW) |
| 4 | LDT3 | Light-Duty Trucks 3 (6,001 - 8,500 lbs. GVWR, 0 - 5,750 lbs. ALVW*) |
| 5 | LDT4 | Light-Duty Trucks 4 (6,001 - 8,500 lbs. GVWR, 5,751 lbs. and greater ALVW) |
| 6 | HDV2B | Class 2b Heavy-Duty Vehicles (8,501 - 10,000 lbs. GVWR) |
| 7 | HDV3 | Class 3 Heavy-Duty Vehicles (10,001 - 14,000 lbs. GVWR) |
| 8 | HDV4 | Class 4 Heavy-Duty Vehicles (14,001 - 16,000 lbs. GVWR) |
| 9 | HDV5 | Class 5 Heavy-Duty Vehicles (16,001 - 19,500 lbs. GVWR) |
| 10 | HDV6 | Class 6 Heavy-Duty Vehicles (19,501 - 26,000 lbs. GVWR) |
| 11 | HDV7 | Class 7 Heavy-Duty Vehicles (26,001 - 33,000 lbs. GVWR) |
| 12 | HDV8A | Class 8a Heavy-Duty Vehicles (33,001 - 60,000 lbs. GVWR) |
| 13 | HDV8B | Class 8b Heavy-Duty Vehicles (>60,000 lbs. GVWR) |
| 14 | HDBS | School Buses |
| 15 | HDBT | Transit and Urban Buses |
| 16 | MC | Motorcycles (All) |

Figure A10-3 
Average Speed Ranges

| Number | Abbrev. | Description |
| --- | --- | --- |
| 1 | 2.5 mph | Miles with average speed 0 to 2.5 mph |
| 2 | 5 mph | Miles with average speed greater than 2.5 to 7.5 mph |
| 3 | 10 mph | Miles with average speed greater than 7.5 to 12.5 mph |
| 4 | 15 mph | Miles with average speed greater than 12.5 to 17.5 mph |
| 5 | 20 mph | Miles with average speed greater than 17.5 to 22.5 mph |
| 6 | 25 mph | Miles with average speed greater than 22.5 to 27.5 mph |
| 7 | 30 mph | Miles with average speed greater than 27.5 to 32.5 mph |
| 8 | 35 mph | Miles with average speed greater than 32.5 to 37.5 mph |
| 9 | 40 mph | Miles with average speed greater than 37.5 to 42.5 mph |
| 10 | 45 mph | Miles with average speed greater than 42.5 to 47.5 mph |
| 11 | 50 mph | Miles with average speed greater than 47.5 to 52.5 mph |
| 12 | 55 mph | Miles with average speed greater than 52.5 to 57.5 mph |
| 13 | 60 mph | Miles with average speed greater than 57.5 to 62.5 mph |
| 14 | 65 mph | Miles with average speed greater than 62.5 mph |

Figure A10-4 
Indicator Codes Used for NONROAD Growth-Referenced External File

| Code | Description |  |
| --- | --- | --- |
| 010 | Airport Service - Total |  |
| 011 | Airport Service - Diesel |  |
| 012 | Airport Service - Gasoline |  |
| 013 | Airport Service - LPG |  |
| 014 | Airport Service - CNG |  |
| 020 | Construction - Total |  |
| 021 | Construction - Diesel |  |
| 022 | Construction - Gasoline |  |
| 023 | Construction - LPG |  |
| 024 | Construction - CNG |  |
| (cont.) |  |  |
| Figure A10-4 Indicator Codes Used for NONROAD Growth-Referenced External File (cont.) |  |  |
| Code | Description |  |
| 030 | Farm - Total |  |
| 031 | Farm - Diesel |  |
| 032 | Farm - Gasoline |  |
| 033 | Farm - LPG |  |
| 034 | Farm - CNG |  |
| 040 | Industrial - Total |  |
| 041 | Industrial - Diesel |  |
| 042 | Industrial - Gasoline |  |
| 043 | Industrial - LPG |  |
| 044 | Industrial - CNG |  |
| 050 | Lawn & Garden -Total |  |
| 051 | Lawn & Garden - Diesel |  |
| 052 | Lawn & Garden - Gasoline |  |
| 053 | Lawn & Garden - LPG |  |
| 054 | Lawn & Garden - CNG |  |
| 060 | Light Commercial - Total |  |
| 061 | Light Commercial - Diesel |  |
| 062 | Light Commercial - Gasoline |  |
| 063 | Light Commercial - LPG |  |
| 064 | Light Commercial - CNG |  |
| 070 | Logging - Total |  |
| 071 | Logging - Diesel |  |
| 072 | Logging - Gasoline |  |
| 073 | Logging - LPG |  |
| 074 | Logging - CNG |  |
| 080 | Railway - Total |  |
| 081 | Railway - Diesel |  |
| 082 | Railway - Gasoline |  |
| 083 | Railway - LPG |  |
| 084 | Railway - CNG |  |
| (cont.) |  |  |
| Figure A10-4 Indicator Codes Used for NONROAD Growth-Referenced External File (cont.) |  |  |
| Code | Description |  |
| 090 | Recreational - Total |  |
| 091 | Recreational - Diesel |  |
| 092 | Recreational - Gasoline |  |
| 093 | Recreational - LPG |  |
| 094 | Recreational - CNG |  |
| 095 | Recreational - 2sATV |  |
| 096 | Recreational - 4sATV |  |
| 097 | Recreational - Offroad MC |  |
| 098 | Recreational - Snowmobile |  |
| 099 | Rec Marine - PWC (backcast to ~zero in 1970; =092 for 1996 and after) |  |
| 100 | Oil Field - Total |  |
| 101 | Oil Field - Diesel |  |
| 102 | Oil Field - Gasoline |  |
| 103 | Oil Field - LPG |  |
| 104 | Oil Field - CNG |  |

Figure A10-5 
Predefined Horsepower Ranges for NONROAD

| Minimum (must be greater than) | Maximum (must be less than or equal to) |  |
| --- | --- | --- |
| 0 | 1 |  |
| 1 | 3 |  |
| 3 | 6 |  |
| 6 | 11 |  |
| 11 | 16 |  |
| 16 | 25 |  |
| 25 | 40 |  |
| 40 | 50 |  |
| (cont.) |  |  |
| Figure A10-5 Predefined Horsepower Ranges for NONROAD (cont.) |  |  |
| Minimum (must be greater than) | Maximum (must be less than or equal to) |  |
| 50 | 75 |  |
| 75 | 100 |  |
| 100 | 175 |  |
| 175 | 300 |  |
| 300 | 600 |  |
| 600 | 750 |  |
| 750 | 1000 |  |
| 1000 | 1200 |  |
| 1200 | 2000 |  |
| 2000 | 3000 (9999 may be used to represent 3000 in files other than equipment population file) |  |

Figure A10-6
Allowed Distinct Average Speed Inputs

| File Extension | Description |
| --- | --- |
| FW1 | LDVs on rural interstates may have their own average speed distribution. |
| FW4 | LDVs on urban interstates may have their own average speed distribution. |
| FW5 | LDVs on urban freeways and expressways may have their own average speed distribution. |
| FW2 | LDTs on rural interstates may have their own average speed distribution. |
| FW7 | LDTs on urban interstates may have their own average speed distribution. |
| FW8 | LDTs on urban freeways and expressways may have their own average speed distribution. |
| FW3 | HDVs may have their own average speed distribution on rural interstates. |
| FW6 | HDVs may have their own average speed distribution on urban interstates. |
| FW9 | HDVs may have their own average speed distribution on urban freeways and expressways. |
| AR1 | LDVs and LDTs must have the same average speed distribution on rural principle arterials. |
| AR2 | LDVs and LDTs must have the same average speed distribution on rural minor arterials. |
| (cont.) |  |
| Figure A10-6 Allowed Distinct Average Speed Inputs (cont.) |  |
| File Extension | Description |
| AR3 | LDVs and LDTs must have the same average speed distribution on rural major collectors. |
| AR5 | LDVs and LDTs must have the same average speed distribution on rural minor collectors and this must be the same as their average speed distribution on rural locals. |
| AR7 | A single average speed distribution is applied to LDVs and LDTs on urban principal arterials, urban minor arterials, and urban collectors. |
| AR4 | HDVs may have their own average speed distribution on rural principal arterials. |
| AR6 | HDVs may have their own average speed distribution on rural minor arterials. |
| AR8 | A single average speed distribution is applied to HDVs for rural major collectors, rural minor collectors, and rural locals. |
| AR9 | A single average speed distribution is applied to HDVs for urban principal arterials, urban minor arterials, and urban collectors. |

Figure A10-7
HPMS Road Type

| Road Type | Description |
| --- | --- |
| 11 | Interstate: Rural |
| 13 | Other Principal Arterial: Rural |
| 15 | Minor Arterial: Rural |
| 17 | Major Collector: Rural |
| 19 | Minor Collector: Rural |
| 21 | Local: Rural |
| 23 | Interstate: Urban |
| 25 | Other Freeways and Expressways: Urban |
| 27 | Other Principal Arterial: Urban |
| 29 | Minor Arterial: Urban |
| 31 | Collector: Urban |
| 33 | Local: Urban |

Figure A10-8
File Type for County NR File Table

| File Type ID | Description |
| --- | --- |
| air | Air Transportation |
| cen | Census Population |
| com | Wholesale Establishment |
| con | Construction Employees |
| frm | Harvested Acres |
| gc | Golf Course Establishment |
| grw | National Defaults |
| hou | Family Housing |
| log | Logging Employees |
| lsc | Landscape Employees |
| mfg | Manufacturing Employees |
| min | Coal Mining Employees |
| oil | Oil and Gas Employees |
| pop | Population Files |
| rr | Railway Maintenance Equipment |
| rvp | RV Park Establishment |
| sbc | Snowblowers - Commercial |
| sbr | Snowblowers - Residential |
| sea | Seasonality |
| snm | Snowmobiles |
| wib | Recreational Marine Inboard |
| wob | Recreational Marine Outboard |