TITLE V PERMIT RENEWAL STATEMENT

Facility Name: AADG, Inc. dba Ceco Door

City: Milan

County: Gibson

Date Application Received: 12/07/2020 and 02/23/2024 (full application update)

Date Application Deemed Complete: 12/07/2020

Emission Source Reference No.: 27-0100

Permit No.: 578717

INTRODUCTION

This narrative is being provided to assist the reader in understanding the content of the attached Title V operating permit.  This Title V Permit Statement is written pursuant to Tennessee Air Pollution Control Rule 1200-03-09-.02(11)(f)1(v).  The primary purpose of the Title V operating permit is to consolidate and identify existing state and federal air requirements applicable to AADG, Inc. dba Ceco Door and to provide practical methods for determining compliance with these requirements.  The following narrative is designed to accompany the Title V Operating Permit Renewal.  It initially describes the facility renewing the permit, then the applicable requirements and their significance, and finally the compliance status with those applicable requirements.  This narrative is intended only as an adjunct for the reviewer and has no legal standing.  Any revisions made to the permit in response to comments received during the public participation process will be described in an addendum to this narrative.

Acronyms

PSD		Prevention of Significant Deterioration

NESHAP	National Emission Standards for Hazardous Air Pollutants

NSPS		New Source Performance Standards

MACT		Maximum Achievable Control Technology

NSR		New Source Review

I.	Identification Information

A.	Source Description

Fabrication and Surface Coating of Steel Doors

1.	List and describe emission sources included in the permit:

27-0100-01: Natural Gas Fuel Fired Equipment

27-0100-03: Two Paint Touch up Booths (only one booth operates at a time)

27-0100-05: Electrostatic paint booths

27-0100-07: Adhesive Coating Line

27-0100-09: Adhesive Coating Line

27-0100-11: Paint Dip line

27-0100-12: Paint Touch-up

27-0100-13: Electrostatic paint booth

27-0100-14: Paint Touch-up

27-0100-16: Pyrolysis Oven

27-0100-18: Paint Touch-up

27-0100-21: Auto Door Adhesive Application

2.	Insignificant Activities

| Insignificant Activity/Emission Unit Description | Applicable Rule Citation |
| --- | --- |
| Closed Cell Urethane Foam In steel Door-Auto | 1200-03-09-.04(5)(a)(4)(i) |
| Closed Cell Urethane Foam In steel Door-Custom | 1200-03-09-.04(5)(a)(4)(i) |
| Four Emergency generator engines (Emission Source 27-0100-24) | 1200-03-09-.04(5)(a)(4)(i) |

B.	Facility Classification

1.	Attainment or Non-Attainment Area Location

Area is designated as an attainment area for all criteria pollutants.

2.	Company is located in a Class II area. (this means that the facility is not located within a national park or national wilderness area; see 40 CFR 52.21(e) for complete definition)

C.	Regulatory Status

1.	PSD/NSR

This facility is not a major source under PSD.

2.	Title V Major Source Status by Pollutant

|  |  | If emitted, what is the facility’s status? |  |
| --- | --- | --- | --- |
| Pollutant | Is the pollutant emitted? | Major Source Status | Non-Major Source Status |
| PM | Yes | Yes | No |
| PM10 | Yes | No | Yes |
| SO2 | Yes | No | Yes |
| VOC | Yes | Yes | No |
| NOX | Yes | No | Yes |
| CO | Yes | No | Yes |
| Individual HAP | Yes | Yes | No |
| Total HAPs | Yes | Yes | No |

3.	MACT Standards

This facility is a major source for HAPs.  This facility is subject to the following MACT standards:

-	40 CFR 63, Subpart MMMM - National Emission Standards for Hazardous Air Pollutants for Surface Coating of Miscellaneous Metal Parts and Products

Subpart MMMM applies to all surface coating operations (Sources 03, 05, 11, 12, 13, 14, and 18) and two of the three adhesive application operations (Sources 07 and 21). Source 09, Adhesive Roll Coater, is a custom door and auto line operation where a water-based glue is roll-applied to honeycomb material made of fibrous cellulose (Email from Jenna Schoonheim, dated January 29, 2025). Therefore, Subpart MMMM does not apply to Source 09.

-	40 CFR 63, Subpart DDDDD – National Emission Standards for Hazardous Air Pollutants for Major Sources:  Industrial, Commercial, and Institutional Boilers and Process Heaters

The following boilers and process heaters are subject to Subpart DDDDD:

| Unit ID | Emission Unit Description | Heat Input Capacity (MMBtu/hr) | Status | Subcategory |
| --- | --- | --- | --- | --- |
| 27-0100-01 | Cleaver Brooks boiler | 8.0 | Existing | Gas 1 |
| 27-0100-21-IH1 | State 1 Wash Tank | 3.5 | Existing | Gas 1 |
| 27-0100-21-IH2 | Heat Stage 3 Rinse Tank | 2.0 | Existing | Gas 1 |
| 27-0100-05-IH1 | Heat Wash Tank | 5.0 | Existing | Gas 1 |
| 27-0100-05-IH2 | Stage 1 Wash Tank | 2.0 | Existing | Gas 1 |
| 27-0100-11-IH1 | Stage 1 Wash Tank | 5.0 | Existing | Gas 1 |

It has been determined that Unit ID 27-0100-05-IH3, an immersion heater in the Stage 3 Rinse Tank on the Custom Door Line (rated at 0.9MMBtu/hr), is not subject to Subpart DDDDD because it meets the definition of a hot water boiler (included in the definition of hot water heater in §63.7575).

-	40 CFR 63, Subpart ZZZZ – National Emission Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines (Four emergency engines – designated insignificant activities, see table below)

4.	Program Applicability

Are the following programs applicable to the facility?

PSD (no)

NESHAP (yes) – See MACT Standards above

NSPS (no) – The following emergency engines are not subject to 40 CFR 60, Subpart JJJJ because each emergency engine was manufactured (or installed) before January 1, 2009.

| Source Number | Site ID | Engine Make/Model | Engine hp | Manufacture Date | Installation Date |
| --- | --- | --- | --- | --- | --- |
| 27-0100-24 | Unit #1 | Ford Power Products/ LRG4251-6005-A | 38 | Before July 12, 1996 | Pre 1/1/2001 |
|  | Unit #2 | Ford Power Products/ LRG4251-6005-A | 38 |  | May 2006 |
|  | Unit #3 | Ford Power Products/ LRG4251-6005-A | 38 |  | August 2007 |
|  | Unit #4 | Ford Power Products | 65 | Before July 12, 1996 | August 2001 |

CAM (no) - Potential pre-control emissions of applicable pollutants from controlled emission units are less than 100 percent of the major source amount. All the Paint Spray booths for surface coating operations (Sources 03, 05, 12, 13, 14, and18) and the Adhesive Spray Booths (Sources 07 and 21) have exhaust filters as emission control.

II.	Compliance Information

A.	Compliance Status

Is the facility currently in compliance with all applicable requirements? yes

Are there any applicable requirements that will become effective during the permit term? no

III.	Other Requirements

A.	Emissions Trading

The facility is not involved in an emission trading program.

B.	Acid Rain Requirements

This facility is not subject to any requirements in Title IV of the Clean Air Act.

C.	Prevention of Accidental Releases

Not Applicable

IV.	Title V Permit History

The initial Title V Permit (548015) was issued on January 20, 1999.

The first Title V Renewal Permit (560619) was issued on October 15, 2010.

The second Title V Renewal Permit (570158) was issued on June 21, 2016.

This is the third Title V Renewal Permit for this facility.

- Changes/updates in this permit renewal

- The permit shell was updated to the current format used for Title V permits. Some conditions under the General Permit conditions Section A-D have been updated as per the new template.

- The Title V renewal addresses the facility request to remove Source No. 27-0100-22 – Honeycomb Expander.  Per information provided by the facility on October 29, 2024, the honeycomb expander remains onsite, but the facility has discontinued use of chemicals containing phenol and formaldehyde, and heat is no longer used in the process. Additional information provided on November 20, 2024, states that the honeycomb materials is now purchased from a manufacturer offsite, and curing is performed by that manufacturer. The honeycomb expander is still in operation at the site however, the process only involves stretching of the material without the use of heat.

- The permit renewal also addresses the facility request to remove Source No. 27-0100-24- Four emergency NG fired emergency engines from the permit and classify them as insignificant sources.

- Condition E1 was revised to update allowable emissions in the Fee emissions summary table and new APC address.

- MACT standards for 40 CFR 63, Subpart MMMM - National Emission Standards for Hazardous Air Pollutants for Surface Coating of Miscellaneous Metal Parts and Products (Section F1) have been updated to reflect the current version.

- TAPCR 1200-03-18-.20 - Coating of Miscellaneous Metal Parts (Section F2) requirements have been added. Previous permits limited emissions from the spray coating operations to avoid applicability of Rule 20. However, two adhesive application operations (Sources 07 and 21) also apply coatings (adhesives) to metal substrates. When potential VOC emissions from all miscellaneous metal parts and products coating lines at the facility are combined, the potential is greater than 100 tons per year.

- MACT standards for 40 CFR 63, Subpart DDDDD – National Emission Standards for Hazardous Air Pollutants for Major Sources:  Industrial, Commercial, and Institutional Boilers and Process Heaters (Section F3) have been updated to reflect the current version.

- Updated the list of sources and added the specs for the four NG fired insignificant emergency engines and their NSPS/NESHAP applicability in a table in the Statement of Basis.

- The Allowable/Potential Emission table in the Statement of Basis for this renewal has been revised to reflect all the changes made in this permit.

V.	Allowable / Potential Emissions

Emissions from the facility at the time of Title V Renewal issuance

| Tons per Year of Allowable or Potential Emissions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source Number | PM | SO2 | VOC | NOx | CO | Total HAP |
| 27-0100-01 |  |  |  |  |  |  |
| 01 Cleaver-Brooks Boiler | 0.26 | 0.021 | 0.19 | 3.44 | 2.89 | 0.06 |
| 08 Bake Oven | 0.03 | 0.002 | 0.02 | 0.34 | 0.29 | 0.01 |
| 21-IH1 Stage 1 Wash Tank | 0.11 | 0.009 | 0.08 | 1.50 | 1.26 | 0.03 |
| 21-IH2 Stage 3 Rinse Tank | 0.07 | 0.005 | 0.05 | 0.86 | 0.72 | 0.02 |
| 21-D1 Dry Off Oven | 0.05 | 0.004 | 0.04 | 0.64 | 0.54 | 0.01 |
| 05-D1 Paint Bake Oven | 0.11 | 0.009 | 0.08 | 1.50 | 1.26 | 0.03 |
| 05-D2 Paint Bake Oven | 0.11 | 0.009 | 0.08 | 1.50 | 1.26 | 0.03 |
| 05-D3 Dry Off Oven | 0.05 | 0.004 | 0.04 | 0.64 | 0.54 | 0.01 |
| 05-IH1 Wash Tank | 0.16 | 0.013 | 0.12 | 2.15 | 1.80 | 0.04 |
| 05-IH2 Stage 1 Wash Tank | 0.07 | 0.005 | 0.05 | 0.86 | 0.72 | 0.02 |
| 05-IH3 Stage 3 Rinse Tank | 0.03 | 0.002 | 0.02 | 0.39 | 0.32 | 0.01 |
| 11-D1 Paint Bake Oven | 0.11 | 0.009 | 0.08 | 1.50 | 1.26 | 0.03 |
| 11-IH Stage 1 Wash Tank | 0.16 | 0.013 | 0.12 | 2.15 | 1.80 | 0.04 |
| 13-D1 Paint Bake Oven | 0.07 | 0.005 | 0.05 | 0.86 | 0.72 | 0.02 |
| 16 Pyrolysis Oven | 0.01 | 0.001 | 0.01 | 0.17 | 0.14 | 0.00 |
| Subtotal (27-0100-01) | 1.41 | 0.11 | 1.02 | 18.51 | 15.55 | 0.35 |
| 27-0100-03 | 6.40 | -- | 99.5 | -- | -- | 12.12 |
| 27-0100-05 | 26.80 | -- |  | -- | -- |  |
| 27-0100-11 | -- | -- |  | -- | -- |  |
| 27-0100-12 | 10.90 | -- |  | -- | -- |  |
| 27-0100-13 | 9.40 | -- |  | -- | -- |  |
| 27-0100-14 | 10.86 | -- |  | -- | -- |  |
| 27-0100-18 | 10.99 | -- |  | -- | -- |  |
| 27-0100-07 | 9.02 | -- | 119.0 | -- | -- | 49.86 |
| 27-0100-09 | -- | -- |  | -- | -- |  |
| 27-0100-21 | 7.92 | -- |  | -- | -- |  |
| Insignificant Activities | 0.00 | 0.00 | 0.01 | 0.31 | 0.04 | 0.00 |
| Plantwide Total1 | 93.92 | 0.11 | 219.61 | 20.15 |  |  |
| Permit Allowable | 93.92 | 0.11 | 219.60 | 19.84 |  |  |

1	Plant-wide total emissions include emissions from insignificant activities.

VI.	Public Participation Procedures

Notification of this draft permit was mailed to the following environmental agencies:

1.	U.S. EPA Region IV

2.	State of Kentucky

3.	State of Arkansas

4.	State of Missouri

VII. 	Dates of Public Comment and EPA review periods

| EPA concurrent review requested |  |
| --- | --- |
| Public Notice publication date |  |
| Public Notice period completion date |  |
| Public Notice publication comments |  |
| EPA Notification date |  |
| EPA review period completion date |  |
| EPA review comments received |  |
| Final Permit issuance date |  |