# APPENDIX to

# EPA Region 8 SLRP Worksheet

The purpose of this document is to serve as temporary* supplemental guidance to the EPA Region 8 Service Line Replacement Plan – PWS Instructional Worksheet. It contains limited guidance on other Service Line Inventory and Replacement Requirements pursuant 40 CFR 141.84, that are related to the required elements of the Service Line Replacement Plan pursuant 40 CFR 141.84(c). This supplemental guidance to the SLRP worksheet does not set limits to, replace or supersede federal or Primacy Agency (EPA Region 8) drinking water regulations, and does not impose legally binding requirements on any party. Your water system remains solely responsible for understanding and meeting all applicable requirements in 40 CFR 141.84(c) and any more stringent primacy agency (EPA Region 8) requirements, including plan content, implementation, timelines, and reporting.

*The information in this document may be moved into a future updated version of the EPA Region 8 LSLR Policy which can be found on the EPA Region 8 Service Line Inventory and Replacement Website.

Contents

Service Line Replacement Plan Instructions:	2

Terms and Acronyms	3

Terms:	3

Acronyms in the EPA Region 8 Service Line Replacement Plan Worksheet.	3

Program Year	3

Access and Obtaining Property Owner Consent Prior to Replacement Requirements	4

Control/Access	4

Wyoming State laws:	4

Obtaining Consent:	4

Reasonable Effort:	4

Protocols for notification and mitigation for partial and full service line replacements —	4

Notification and mitigation requirements for planned partial service line replacement	4

Notification and mitigation requirements for emergency partial service line replacement.	5

Notification and mitigation requirements for full service line replacement.	5

LSLR Project Contacts	7

# Service Line Replacement Plan Instructions:

- Create an initial service line replacement plan: Use this worksheet to help create your initial service line replacement plan (SLRP). The service line replacement plan should be reflective of the service line inventory version that is submitted with it, which should be no earlier version than the baseline inventory due by Nov.1, 2027, in accordance with 141.84(a) and 141.90 (e). Make sure each of the required elements (sections) is completed in your SLRP and sufficiently detailed to ensure your system is able to comply with all of the service line inventory and replacement requirements in accordance with 40 CFR 141.84. You may also need to provide additional information or documentation along with the information in the worksheet to complete compliance with the service line replacement plan requirement in accordance with 141.84(c)(1).

- Make the service line replacement plan publicly accessible

  - Water systems serving greater than 50,000 persons must make the plan available to the public online.

- Service Line Replacement Plan (SLRP) Updates: Update the SLRP annually with new or updated information and submit it to primacy agency (EPA Region 8).

  - Systems with replacement plans that have no changes to report, may certify to the primacy agency (EPA Region 8) instead that there is no new information or update since the last iteration of the replacement plan that was submitted instead of submitting the same plan again.

  - If there are no longer any lead, GRR, or unknown service lines in the inventory, the system is not required to submit an annual update or certification of no changes to the primacy agency (EPA Region 8).

- Service Line Replacement Plan Reporting Requirements:

  - Initial Replacement Plan Due date: November 1, 2027

  - Annual RP updates:

    - 1st update: January 30, 2029

    - Annual updates thereafter: January 30th of every year until there are no longer any lead, GRR, or unknown service lines in the inventory

- Submitting the SLRP to EPA Region 8: By the reporting deadline, submit all required materials using one of the following methods (include your PWS Name and/or PWS ID# on all correspondence):

Email: R8DWU@EPA.gov RE: “LSLR-SLRP”

Fax: 1 (303) 312-7515 (Attn: LCR LSLI Rule Manager)

Mail: Use the address at the top of this letter and include Mail Code 8WD-SDP on the envelope. (Attn: LCR LSLI Rule Manager)

# Terms and Acronyms

New terms and acronyms that appear in the EPA Region 8 Service Line Replacement Plan that are not in the current version of the  EPA Region 8 LSLR Policy are included below.

## Terms:

- See EPA Region 8 LSLR Policy. EPA Region 8 LSLR Policy by visiting our Website.

## Acronyms in the EPA Region 8 Service Line Replacement Plan Worksheet.

- See EPA Region 8 LSLR Policy. EPA Region 8 LSLR Policy by visiting our Website for full list

| Acronym | Stands For |
| --- | --- |
| SLRP | Service Line Replacement Plan |

# Program Year

Program year: The first or initial program year is November 1, 2027, through December 31, 2028, and each subsequent program year begins January 1 and ends December 31. Service Line Replacement Plans are due at the beginning of each program year. The initial Service Line Replacement Plan will be due by November 1, 2027, then annually thereafter by January 30 at the beginning of each program year.

| Program Year | Beginning Date | Ending Date | Service Line Replacement Plan Reporting Deadline |
| --- | --- | --- | --- |
| 1 (Initial) | November 1, 2027 | December 31, 2028 | November 1, 2027 |
| 2 | January 1, 2029 | December 31, 2029 | January 30, 2029 |
| 3 | January 1, 2030 | December 31, 2030 | January 30, 2030 |
| 4 | January 1, 2031 | December 31, 2031 | January 30, 2031 |
| 5 | January 1, 2032 | December 31, 2032 | January 30, 2032 |
| 6 | January 1, 2033 | December 31, 2033 | January 30, 2033 |
| 7 | January 1, 2034 | December 31, 2034 | January 30, 2034 |
| 8 | January 1, 2035 | December 31, 2035 | January 30, 2035 |
| 9 | January 1, 2036 | December 31, 2036 | January 30, 2036 |
| 10 | January 1, 2037 | December 31, 2037 | January 30, 2038 |

# Access and Obtaining Property Owner Consent Prior to Replacement Requirements

Water systems must fully replace all lead and GRR service lines under their control within 10 years, unless they are required by the primacy agency (EPA Region 8) to replace them sooner or have a deferred deadline

Control/Access: Is the system’s ability to gain physical or legal access to conduct full service line replacement.

- Where the system has access to conduct full SLR, the service line is under its control

- Where a water system does not have access to conduct full SLR, the service line is not under its control.

Whether a service line is under the control of the water system will depend on:

- The relevant state and local laws that authorize and/or condition a water system's ability to exert control over the line to replace it, and

- Whether a water system can gain physical access to the service line to conduct a full replacement

Access criteria include, but are not limited to

- Whether the water system can safely enter the property,

- Whether the water system can safely conduct the replacement, and

- Whether the water system had obtained the property owner’s consent (if required for access)

Wyoming State laws: Information not available at this time.

Obtaining Consent: Where a water system has legal access to conduct full service line replacement only if property owner consent is obtained, the water system must make a “reasonable effort” to obtain property owner consent. If such a water system does not obtain consent after making a “reasonable effort” to obtain it from any property owner, then the water system is not required by this subpart to replace any portion of the service line at that address unless there is a change in ownership of the property as described in 40 CFR 141.84(d)(3)(ii). The water system must provide documentation of the reasonable effort to the Primacy agency (EPA Region 8) pursuant to 40 CFR 141.90(e)(10).

Reasonable Effort: Must include at least four attempts to engage the property owner using at least two different methods of communication (e.g., in-person conversation, phone call, text message, email, written letter, postcard, or information left at the door such as a door hanger) before the applicable deadline of mandatory service line replacement as described in 40 CFR 141.84(d)(4). The Primacy agency (EPA Region 8) may require systems to conduct additional attempts and may require specific outreach methods to be used.

# Protocols for notification and mitigation for partial and full service line replacements —

Notification and mitigation requirements for planned partial service line replacement. Whenever a water system plans to partially replace a lead or galvanized requiring replacement service line in coordination with planned infrastructure work that impacts service lines, the water system must provide written notice to the property owner, or the owner's authorized agent, as well as non-owner occupant(s) served by the affected service line at least 45 days prior to the replacement. Where a water system has access to conduct full service line replacement only if property owner consent is obtained, the water system must make a reasonable effort to obtain property owner consent to replace the remaining portion of the service line in accordance with 40 CFR 141.84(d)(3)(i) of this section. The reasonable effort must be completed before the partial lead service line replacement.

- Before the affected service line is returned to service, the water system must provide written notification that explains that consumers may experience a temporary increase of lead levels in their drinking water due to the replacement and that meets the content requirements of § 141.85(a)(1)(ii) through (iv) and contact information for the water system. In instances where multi-family dwellings or multiple non-residential occupants are served by the affected service line to be partially replaced, the water system may elect to post the information at a conspicuous location instead of providing individual written notification to all residents or non-residential occupants.

- Before the affected service line is returned to service, the water system must provide written information about a procedure for consumers to flush service lines and premise plumbing of particulate lead following partial replacement of a lead or galvanized requiring replacement service line.

- Before the affected service line is returned to service, the water system must provide the consumer with a pitcher filter or point-of-use device certified by an American National Standards Institute accredited certifier to reduce lead, six months of replacement cartridges, and instructions for use. If the affected service line serves more than one residence or non-residential unit (e.g., a multi-unit building), the water system must provide a pitcher filter or point-of-use device, six months of replacement cartridges and use instructions to every residential and non-residential unit in the building.

- The water system must offer to the consumer to collect a follow up tap sample between three months and six months after the completion of any partial replacement of a lead service line. The tap sample must be a first- and fifth-liter paired sample after at least six hours of stagnation, following the tap sampling protocol under § 141.86(b). The water system must provide the results of the sample to the persons served by the service line in accordance with § 141.85(d).

Notification and mitigation requirements for emergency partial service line replacement. Any water system that creates a partial replacement of a lead or galvanized requiring replacement service line due to an emergency repair must provide notice and risk mitigation measures to the persons served by the affected service line in accordance with paragraphs (h)(1)(i) through (iv) of this section before the affected service line is returned to service. The water system must offer to the property owner, or the owner's authorized agent, to replace the partial service line created by the emergency repair within 45 days.

Notification and mitigation requirements for full service line replacement. Any water system that conducts a full lead or galvanized requiring replacement service line replacement must provide written notice to the persons served by the affected service line before the affected service line is returned to service; written notice must be provided to the owner or the owner's authorized agent, no later than 30 days following completion of the replacement.

- The written notification must explain that consumers may experience a temporary increase of lead levels in their drinking water due to the replacement and must meet the content requirements of § 141.85(a)(1)(ii) through (iv) as well as contact information for the water system. In instances where multi-family dwellings or multiple non-residential occupants are served by the lead or galvanized requiring replacement service line to be replaced, the water system may elect to post the information at a conspicuous location instead of providing individual written notification to all persons served in residential and non-residential units.

- Before the replaced service line is returned to service, the water system must provide written information about a procedure for consumers to flush service lines and premise plumbing of particulate lead following full replacement of a lead or galvanized requiring replacement service line.

- Before the replaced service line is returned to service, the water system must provide the consumer with a pitcher filter or point-of-use device certified by an American National Standards Institute accredited certifier to reduce lead, six months of replacement cartridges, and instructions for use. If the lead service line serves more than one residence or non-residential unit (e.g., a multi-unit building), the water system must provide a pitcher filter or point-of-use device, six months of replacement cartridges and instructions for use to every residential and non-residential unit in the building.

- The water system must offer to the consumer to collect a follow up tap sample between three months and six months after completion of any full replacement of a lead or galvanized requiring replacement service line. The tap sample must be a first-liter sample after at least six hours of stagnation, following the tap sampling protocol under § 141.86(b). The water system must provide the results of the sample to the consumer in accordance with § 141.85(d).

# LSLR Project Contacts

| PWS LSLR Project Contact List |  |
| --- | --- |
| Purpose of this worksheet: For water systems to keep track the contact information for each person involved in the inventory and replacement activities and what parts of the project they are responsible for |  |
| Baseline Inventory Development |  |
| Name: | Title: |
| Telephone: | Email: |
| Baseline Inventory Ongoing Maintenance |  |
| Name: | Title: |
| Telephone: | Email: |
| Completing and Submitting Annual Inventory Update |  |
| Name: | Title: |
| Telephone: | Email: |
| Service Line Replacement Plan Development |  |
| Name: | Title/Affiliation: |
| Telephone: | Email: |
| Service Line Replacement Plan Ongoing Maintenance |  |
| Name: | Title/Affiliation: |
| Telephone: | Email: |
| Service Line Replacement Plan Ongoing Maintenance |  |
| Name: | Title/Affiliation: |
| Telephone: | Email: |
| Annual Completion and Resubmission of Replacement Plan (w/Updates if applicable) |  |
| Name: | Title/Affiliation: |
| Telephone: | Email: |
| Unknown Service line Identification Strategy Development |  |
| Name: | Title/Affiliation: |
| Telephone: | Email: |
| Service line material verifications (field investigations) |  |
| Name: | Title/Affiliation: |
| Telephone: | Email: |
| Inventory Validation (non-lead validation pool, selection of verification site, and verification tracking ) |  |
| Name: | Title/Affiliation: |
| Telephone: | Email: |
| Service Line Inventory and replacement communication Strategy |  |
| Name: | Title/Affiliation: |
| Telephone: | Email: |
| Service Line Inventory and replacement Notification delivery |  |
| Name: | Title/Affiliation: |
| Telephone: | Email: |
| Service Line Control and Access (Identification of laws, etc) |  |
| Name: | Title/Affiliation: |
| Telephone: | Email: |