# Chapter 1: Introduction

# Instructor Notes

## Objectives

The objective of this chapter is to introduce students to the course and cover basic information. At the end of this chapter students will know:

- Course objectives

- Basic information on lead and lead in dust and debris

- The role of a lead dust sampling technician

- How this course relates to the lead-based paint inspector and risk assessor courses

- The EPA RRP Rule’s requirements related to dust-lead sampling and cleaning verification

- HUD clearance requirements for non-abatement jobs

## Introduction to this chapter

This is the first chapter of the course. It outlines the course objectives, provides basic information about the class, and provides an overview of the lead-dust sampling technician’s role and responsibilities.

## Activities

Introductions: On Slide 1-3, ask students to identify themselves and the organization they work with and tell how their work involves lead-based paint.

## Review

This chapter has no review.

## Notes

This chapter simply sets the stage for the rest of the course. The requirements for the HUD and EPA rules can be complicated and may require extra time. Be sure to point out Attachment 1-A: Comparing Lead Evaluation Professionals (p.29), which describes the differences in capacity and responsibility of lead dust sampling technicians, lead-based paint inspectors, and lead-based paint risk assessors

# Chapter 1 Presentation Slides

Chapter 1: Introduction

Lead Dust Sampling Technician Initial Training Course

January 12, 2026

1

Notes:

Welcome to the Lead Dust Sampling Technician Training Course.

Course Objectives

- Understand what a dust-lead test is.

- Identify steps in dust-lead testing.

- Learn how to:

  - Conduct a visual inspection

  - Collect lead dust samples

  - Interpret results

  - Write a report

  - Explain the results to the client

2

Notes:

The EPA has provided this model curriculum to teach individuals how to conduct dust-lead testing after renovation activities.

Dust-lead testing is often performed to find out whether lead dust remains after renovation, repair, or painting. It is required by HUD’s LSHR regulations for most renovations. By the end of the course, students will be able to perform the actions listed above.

Introduce Yourself

- Name

- Occupation/organization

- How does your work involve lead-based paint?

3

Notes:

Overview of Student Materials

- Student Manual

- Attachments

- Appendices

- Lead Dust Sampling Technician Field Guide

4

Notes:

The student materials include a student manual, attachments, appendices, and a copy of the Lead Dust Sampling Technician Field Guide.

- The student manual contains copies of the slides that are used by the instructor during the course.

- The attachments and appendices provide important summaries, checklists, tables, and tools you can use.

- The Lead Dust Sampling Technician Field Guide outlines key points and procedures in one easy-to-read reference tool that can be taken along on the job.

Health Risks of Lead

- In children:

  - Damage to the brain and central nervous system

  - Decreased intelligence

  - Learning and behavioral problems, and hyperactivity

  - Damage can be long-term, affecting children throughout their lives

- In pregnant women:

  - Miscarriage

  - Harm to the developing fetus

5

Notes:

### Children under 6 are most at risk from exposure to small amounts of lead.

Children are at a greater risk than adults. During normal and frequent playing or hand-to-mouth activity, children may swallow or inhale lead dust from their hands, toys, food, or other objects.

In children, lead may cause:

- Damage to the brain and nervous system

- Decreased intelligence, and learning and behavioral problems

- Attention deficit disorder and hyperactivity

- Hearing and speech problems

Among adults, pregnant women are especially at risk from exposure to lead. Lead can build up in the body over time, settling in the bones. This stored lead can be transferred during pregnancy to a fetus and can cause an increased risk of :

- Miscarriage

- The baby being born too early or too small

- Harm to a developing fetus’ brain, kidneys and nervous system

Health Risks of Lead continued

- In adults:

  - Heart-related problems

  - High blood pressure

  - Infertility problems

  - Decreased kidney function

6

Notes:

For most adults, the major route of entry of lead into the body is through inhalation (breathing) of lead dust via occupational exposures.

In adults lead exposure can cause:

- Heart related problems and high blood pressure

- Decreased kidney function

- Weakness in fingers, wrists and ankles

- Infertility problems in both men and women

- Digestive problems

- Nerve disorders

- Learning, memory and concentration problems

- Muscle and/or joint pain

Why Are Dust and Debris a Problem?

- Renovations that disturb lead-based paint create dust and debris.

- Very small amounts of lead dust can poison children.

- Adults can swallow or breathe lead dust during work activities.

- Workers can bring lead dust home and poison their families.

7

Notes:

### Dust and debris from renovation, repair, and painting jobs in pre-1978 housing and child-occupied facilities may contain lead.

- Pre-1978 paint may contain lead.

- Common renovation activities like sanding, scraping, cutting, and demolition can create hazardous lead dust and chips by disturbing lead-based paint.

- Some tasks, such as power sanding, open flame burning, and the use of heat guns above 1100 degrees Fahrenheit, create large amounts of extremely fine lead dust that is very difficult to clean up.

A tiny amount of lead can be extremely harmful.

- Lead dust particles are often so small that you cannot see them, yet you can breathe or swallow them. These smaller, inhaled or swallowed lead dust particles are more easily absorbed by the body than larger particles, and can therefore more easily cause poisoning.

- Lead dust may be breathed or swallowed by children, residents, and workers.

- Through normal hand-to-mouth activities, children may swallow or inhale lead dust on their hands, toys, food, or other objects. Children may also ingest paint chips.

- Adults can swallow or breathe lead dust during work activities.

  - When workers perform activities, such as scraping and sanding by hand, or use a power sander or grinding tool, dust is created. The dust goes into the air that they breathe.

  - If workers eat, drink, smoke, or put anything into their mouths without washing up first, they may swallow the lead dust present.

Regulations Addressing Lead Hazards in Housing

- To address the issue of lead hazards in housing, EPA and HUD have issued several regulations.

- EPA currently oversees the training and certification of abatement contractors, inspectors, and risk assessors.

- HUD’s Lead Safe Housing Rule (LSHR) addresses lead hazards in federally owned and assisted housing.

- In April 2008, EPA issued the Renovation Repair and Painting (RRP) Rule to address lead hazards created during renovation.

8

Notes:

Overview of the Lead Hazard Evaluation Courses

- Lead Dust Sampling Technician

- Lead-Based Paint Inspector

- Risk Assessor

  - All three disciplines can conduct dust-lead testing after an RRP project

9

Notes:

There are three courses for professionals who wish to be certified to conduct evaluations for lead dust, lead hazards and/or lead-based paint:

- Lead Dust Sampling Technician: This 8-hour training course teaches you how to become a lead dust sampling technician. You will learn how to conduct a visual inspection, collect dust wipe samples, interpret results, and write a post-work dust-lead testing report.

- Lead-Based Paint Inspector: To become a certified lead-based paint inspector, you must take a 24-hour training course. In the lead-based paint inspector course, you will learn the skills and protocols for

conducting a paint inspection. A lead-based paint inspection is a surface-by-surface investigation to locate all lead-based paint on a property. We will talk more about what a paint inspection is in the next chapter.

- Risk Assessor: To become a certified risk assessor, you must successfully complete a lead-based paint inspector course plus an additional 16-hour risk assessor course. In the risk assessor course, you will learn the skills and protocols necessary for conducting risk assessments. A risk assessment is an on-site investigation to identify all lead-based paint hazards on a property.

Today you are taking the Lead Dust Sampling Technician Training Course.

Refer to Attachment 1-A: Comparing Lead Evaluation Professionals on p. 29 for additional information.

Lead Dust Sampling Technician

- EPA’s RRP rule also established the lead dust sampling technician discipline.

- To work as a lead dust sampling technician, an individual must successfully complete this training course. The course completion certificate will serve as your certification.

- Lead dust sampling technicians are used in both EPA’s and HUD’s regulations.

10

Notes:

A Lead Dust Sampling Technician Can...

- Perform post-renovation dust-lead testing under EPA’s RRP Rule

- Determine whether the work area has been sufficiently cleaned of lead dust after renovation, repair, or painting

- Perform a clearance examination after hazard reduction or maintenance activities in most properties covered by HUD’s LSHR

11

Notes:

A lead dust sampling technician can perform dust-lead testing, but not if associated with an abatement.

The purpose of dust-lead testing after renovation, repair, or painting activities that disturb lead-based paint is to determine if a work area is safe for re-occupancy. These activities can create lead dust, so proper cleanup is critical.

A Lead Dust Sampling Technician Is Not Allowed To...

- Perform post-abatement dust-lead testing

  - Lead abatement jobs are designed to permanently address lead-based paint hazards

  - Post-abatement clearance below the action levels after an abatement must be done by a certified risk assessor or lead-based paint inspector and may not be done by a lead dust sampling technician

- Sample paint for lead content

- Sample soil for lead

12

Notes:

While lead dust sampling technicians can conduct post-renovation dust-lead testing, they are not allowed to conduct post-abatement dust-lead testing. Post-abatement dust-lead testing must be done by a certified risk assessor or a lead-based paint inspector.

Abatements are projects designed to permanently address lead-based paint and lead-based paint hazards.

Abatement does not include renovation, remodeling, landscaping, or other activities, when such activities are not designed to permanently address lead-based paint hazards, but, instead, are designed to repair, restore, or remodel a given structure or dwelling, even though these activities may incidentally result in permanently addressing lead-based paint hazards.

If a renovation job involves abatement, the lead dust sampling technician cannot perform dust-lead testing on the abatement part of the job. Make sure you understand what type of work was done before conducting dust-lead testing.

In addition, a lead dust sampling technician is not trained to test paint for lead content. Paint sampling must be done by a lead-based paint inspector, risk assessor or certified renovator.

Finally, a lead dust sampling technician is not trained to sample soil. Soil sampling must be done by a certified lead-based paint inspector or risk assessor.

HUD NOTE: HUD does not allow lead dust sampling technicians to work on abatement projects. HUD also requires that a certified risk assessor or a certified lead-based paint inspector approve the work of the lead dust sampling technician and sign the abatement report. Lead Dust sampling technicians may work on single-family properties or individually-specified dwelling units and associated common areas in a multi-unit property as directed by a certified risk assessor or a certified lead-based paint inspector, but may not themselves select dwelling units or common areas for testing.

EPA’s RRP Rule

- Common renovation activities like sanding, cutting, and demolition can create hazardous lead dust and chips by disturbing lead-based paint.

- All contractors performing RRP projects in pre-1978 homes, child care facilities and schools must be certified and use lead-safe work practices to prevent lead contamination.

- All lead dust sampling technicians must be trained and certified.

13

Notes:

Contractors performing RRP projects that disturb lead-based paint in homes, child care facilities, and schools built before 1978 must be certified and must follow specific work practices to prevent lead contamination.

EPA’s RRP Rule: Cleaning Verification

- Upon completion of renovation activity, the RRP Rule requires either:

  - Cleaning verification by a certified renovator, or

  - Dust-lead testing by a certified lead dust sampling technician, lead-based paint inspector, or risk assessor

- “Cleaning verification” need not be done if both dust-lead testing and achieving clearance below the actions levels is required by:

  - The contract between the renovator and the property owner, or

  - Another federal, state, or local law

14

Notes:

The cleaning verification process involves a visual inspection of the work area, followed by wiping of the window sills, floors, and countertops with wet disposable cleaning cloths and comparing the wipes to a cleaning verification card.

Cleaning verification must be performed by a certified renovator; it cannot be done by the lead dust sampling technician and lead dust sampling technicians are not allowed to perform clearance below the actions levels on abatement projects or on abatement parts of renovation projects.

See Appendix A on page 179 for a portion of the EPA RRP final rule.

HUD NOTE: A certified lead dust sampling technician may work on HUD-assisted single-family properties or individually-specified dwelling units and associated common areas in a multi-unit property. The certified lead dust sampling technician may work on a random sampling of dwelling units or common areas in multifamily properties only as directed by a certified risk assessor

or a certified lead-based paint inspector, but the lead dust sampling technician may not select dwelling units or common areas for testing, and the risk assessor or inspector must approve the lead dust sampling technician’s work and sign the post-work dust sampling report for the report to be acceptable.

EPA’s RRP Rule: LDST Certification

- You must be a certified LDST to perform post-renovation dust-lead testing under EPA’s RRP Rule.

- Successful completion of this course completes the certification process.

- You will be certified by either EPA, or a state, Tribe, or territory in which you work if they are an authorized program.

15

Notes:

To conduct a post-renovation dust-lead test, the individual must be a certified lead dust sampling technician (LDST), lead-based paint inspector, or risk assessor.

EPA LDST certification allows the certified individual to perform post-renovation dust-lead testing in residential housing and child-occupied facilities. Certified lead dust sampling technicians cannot conduct post-abatement dust-lead testing.

When going to a worksite, lead dust sampling technicians must have a copy of their initial course completion certificate and most recent refresher training course completion certificate.

EPA’s RRP Rule: Dust-Lead Testing

- Before conducting lead dust sampling after a renovation, a visual inspection of the work area for dust and debris is required.

- Results of dust-lead testing must be interpreted according to the EPA dust-lead actions levels/HUD clearance standards and provided to the client.

- All surfaces represented by a test that failed to clear below the dust-lead action levels must be re-cleaned and re-tested until the action levels are met.

16

Notes:

More details on all these topics are discussed later in the course.

HUD’s Lead Safe Housing Rule (LSHR)

- HUD requires clearance testing on all but very small renovation or maintenance jobs.

- Clearance must be performed by a clearance examiner who is independent of those performing work (third party).

- This clearance must be performed by either a certified lead-based paint inspector, risk assessor, or lead dust sampling technician.

- HUD requires a visual inspection (assessment), dust sampling, laboratory analysis, and submission of a clearance report

17

Notes:

HUD’s LSHR requires that lead hazards be corrected in target housing receiving federal housing assistance or being sold. It also requires clearance dust sampling as a routine part of every such activity, unless very small amounts of paint are disturbed.

HUD’s LSHR

- HUD clearance generally covers an entire dwelling unit, common areas, and exteriors.

- Worksite-only clearance is permitted on certain renovation or maintenance jobs.

  - For ongoing lead-based paint maintenance

  - Rehabilitation assistance up to and including $5,000 per unit

- Clearance report must include specifics of property, results of visual inspection, laboratory information, dates, written description of work performed, and dust testing results

18

Notes:

Clearance must be on the entire unit unless the worksite has been properly contained or the LSHR specifically permits worksite-only clearance. The LSHR permits worksite-only clearance for units receiving rehabilitation assistance up to and including $5,000 and for ongoing lead-based paint maintenance activities.

Sampling requirements will be discussed in later chapters.

HUD’s LSHR continued

- Use EPA dust-lead action levels/HUD clearance standards to interpret dust sampling results.

- HUD requires that all surfaces represented by a failed clearance test be re-cleaned and re- tested until the clearance level is met.

- If the work area fails the visual inspection, the sampling technician must stop and require the renovator to re-clean.

- The sampling technician must then re-inspect before dust testing.

19

Notes:

Details on these topics are provided later in the course.