# Attachment 3-D: EPA Lead Dust Wipe Checklist

These are the steps involved in taking a lead dust wipe sample. When you are collecting dust samples, you should follow each of these steps. Note: The procedure for sampling floors is different than the procedure for sampling window sills and troughs.

| Step | Criteria | Check when done |
| --- | --- | --- |
| 1 | Put on disposable shoe covers and lay out the sample area - Clean the template and properly dispose of wipe - Tape down template; or lay out sample area using tape |  |
| 2 | Prepare the tubes - Label tube with identification number - Record identification number on sample collection and chain-of-custody forms - Partially unscrew cap of tube and place tube near the area planned for sampling |  |
| 3 | Put on clean gloves |  |
| 4 | Wipe sample area and place wipe in centrifuge tube |  |
| 4a | First swipe (floors): - Press wipe down firmly - Make overlapping “S”-like motions on the sample surface while moving side-to-side - Do not cross outer boundary tape or template |  |
| 4b | Second swipe (floors): - Fold wipe in half, keeping dust inside, and press wipe down firmly - Make top-to-bottom overlapping “S”-like motions - Do not cross outer boundary tape or template |  |
| 4c | Third swipe (floors): - Fold wipe in half, keeping dust inside, and press wipe down firmly - Repeat the wiping procedure one more time (focusing in on corners) - Do not cross outer boundary tape or template - Fold wipe again, keeping all dust inside wipe - Place wipe in sample container tube |  |
| 4d | Window sills and troughs (side-to-side) - Hold fingers together, wipe surface in one direction, and press wipe down firmly - Fold wipe in half and repeat wiping procedure, using a reverse direction - Fold wipe in half again and repeat wiping procedure concentrating on corners - Fold wipe in half again and insert into a rigid-walled container - Label the rigid-walled container to identify sample |  |
| 5 | Measure the sample area - Measure the area inside the tape, not the outside border - Measure to 1/8 inch |  |

| Step | Criteria | Check when done |
| --- | --- | --- |
| 6 | Record sample area - Calculate sample area - Record measurements on the sample collection form - Fill in chain-of-custody form |  |
| 7 | Clean up - Clean template with new wipe, place template in a plastic bag for storage, and then discard wipe - Put gloves, used shoe covers, and tape from floors and windows into trash bags - Check your clothing and shoes (especially soles) before leaving work site - Wash your face and hands with warm, soapy water or sanitary wipe |  |