# Attachment 4-D: Activity Interpreting Laboratory Results

Instructions: The purpose of this activity is to test your ability to verify the results received from the laboratory, compare these results to EPA dust-lead action levels and interpret the results. Using the following excerpt from a dust-lead testing results form, check the laboratory’s calculation of the weighted lead dust sample.

Using the following excerpt from a dust-lead testing results form, check the laboratory’s calculation of the lead dust loading.

| Sample # | Location | Surface | Total Lead (µg) | Dimensions of Sample Area (ft²) | µg/ft² |
| --- | --- | --- | --- | --- | --- |
| 92-1 | Upstairs bedroom | Floor | 5 | 1.025 | 4.9 |
| 92-2 | Upstairs bedroom | Interior window sill | 150 | 0.478 | 202.7 |
| 92-3 | Kitchen, front window | Interior window sill | 42 | 0.544 | 77.2 |
| 92-4 | Kitchen, side window | Window trough | 87 | 0.928 | 60.0 |

- Check the results (µg lead/ft²) for each sample. If the results are incorrect, provide the correct results in µg lead/ft².

- After verifying the laboratory’s results, compare these results to the appropriate EPA recommended guidance. Did the individual samples pass below the action levels or fail to clear below the action levels?

EPA Action Levels for Lead Dust Floors: < 5 µg/ft²

Interior window sills: < 5 µg/ft² Window troughs: < 5 µg/ft²

| Sample # | Result | Dust-Lead Action Level | Pass or Fail |
| --- | --- | --- | --- |
| 92-1 |  |  |  |
| 92-2 |  |  |  |
| 92-3 |  |  |  |
| 92-4 |  |  |  |