# Attachment 5-A: Sample Written Report

The following report is a sample post-work dust-lead testing report from a small HUD funded rehabilitation job (less than $5,000/unit) in a unit that involved window replacements in the small bedroom and kitchen of a single-family home that is available for rent. The post-work dust-lead testing report covers dust-lead testing of the worksite.

# POST-WORK DUST-LEAD TESTING REPORT

## General Information

| Information Needed | Report Information |
| --- | --- |
| Date of Dust-Lead Test | 8/5/25 |
| Lead Dust Sampling Technician | Jane White |
| Property Address | 80 East Main St. Hammond, IN 89898 |
| Client Name and Address | Smith Family 80 East Main St. Hammond, IN 89898 |
| Laboratory Name and Address | Analysis Services, Inc. 990 45th St., Suite 500 Gary, IN 44444 |
| Telephone Number | 222-222-2222 |
| NLLAP Number | IN 999999 |

Summary of Post-Work Dust-Lead Testing Results

This unit failed the dust-lead testing portion of the lead dust sampling examination. Areas represented by the failed samples should be re-cleaned.

Lead dust above the EPA action levels/HUD clearance standards was found in the following areas:

| Location | Surface | µg lead/ft2 |
| --- | --- | --- |
| Small bedroom | Side facing window (C-1)—window sill | 600 |
| Small bedroom | Floor | 200 |
| Kitchen | Window above sink (A-1)-- window sill | 525 |

Signature: Jane White

Date: 8/12/25

Summary of Hazard Reduction Activities

| Information Needed | Report Information |
| --- | --- |
| Name of Firm | ABC Renovations |
| Address of Firm | 123 Main Street East Chicago, IN 12345 |
| Supervisor Name | John Brown #1634 |
| Supervisor Certification Number | 1634 |
| Start and Completion Date of Hazard Reduction or Completion Activity | 8/4/25 to 8/5/25 |

Description of Hazard Reduction Activities and Areas Addressed

| Location | Activity |
| --- | --- |
| Kitchen | Replaced A-1 window with new, vinyl-clad window |
| 2nd floor small bedroom | Replaced C-1 and C-2 windows with new, vinyl-clad windows |

Description of Work

The supervisor was present on the job site when work was being performed. Workers used lead-safe work practices. Plastic sheeting covered a 5-foot area on the ground outside under the windows being replaced and on the floor inside. Signs were posted at the doors to the bedroom and kitchen. Occupants were not allowed in the kitchen and bedroom and the outside work area during this activity. The window frame was misted prior to tear-out. After removal, workers wrapped the old windows in plastic sheeting and picked up debris on the plastic immediately and bagged it. The plastic sheeting was carefully gathered up and bagged for disposal. Workers replaced their disposable booties when leaving the work area for lunch and breaks. Respirators were not necessary. The new windows were installed and a clearance examination was requested.

# Part I. VISUAL INSPECTION RESULTS FORM

| Information Needed | Report Information |
| --- | --- |
| Date of Lead Dust Sampling | 8/5/25 |
| Lead Dust Sampling Technician | Jane White |
| Client | Smith Family |
| Property Address | 80 East Main St. Hammond, IN 89898 |

Visual Inspection of the Work Area

| Work Area | Deteriorated Paint | Debris | Visible Dust | Notes | Pass or Fail? |
| --- | --- | --- | --- | --- | --- |
| Small bedroom | n/a | n/a | n/a | n/a | Pass |
| Kitchen | n/a | n/a | n/a | n/a | Pass |
| First floor hallway | n/a | n/a | n/a | n/a | Pass |
| Staircase | n/a | n/a | n/a | n/a | Pass |
| Second floor hallway | n/a | n/a | n/a | n/a | Pass |
| Exterior soil under kitchen window | n/a | n/a | n/a | n/a | Pass |
| Exterior soil under bedroom window | n/a | n/a | n/a | n/a | Pass |

# DUST-LEAD TESTING RESULTS FORM

| Information Needed | Report Information |
| --- | --- |
| Date of Lead Dust Sampling | 8/5/25 |
| Lead Dust Sampling Technician | Jane White |
| Client Name | Smith Family |
| Property Address | 80 East Main St. Hammond, IN 89898 |

## Dust-Lead Testing Results

| Sample # | Location | Surface | Dimensions of Sample Area (ft² | µg lead/ft² | Pass or Fail? |
| --- | --- | --- | --- | --- | --- |
| 1-2 | Upstairs small bedroom | Front facing window (C-2) - window sill | 4” x 18” | 17 | Pass |
| 1-3 | Upstairs small bedroom | Floor under C-1 window | 12” x 12” | 80 | Fail |
| 1-4 | Upstairs small bedroom | Side facing window (C-1) – window sill | 4” x 18” | 100 | Fail |
| 2-1 | Second floor hallway, 3 feet from newel post | Floor | 12” x 12” | 2 | Pass |
| 3-1 | Staircase 5th step from bottom | Floor | 12” x 12” | 4 | Pass |
| 4-1 | Kitchen | Floor under A-1 window | 12” x 12” | 8 | Fail |
| 4-2 | Kitchen | Window above sink (A-1) – window sill | 4” x 18” | 225 | Fail |
| 5-1 | First floor, entry foyer, 2 feet inside front door | Floor | 12” x 12” | 3 | Pass |

FIRST FLOOR

X = sample locations

Exterior soil

X

Kitchen/DR

Hallway

Foyer

X

X

1st floor Bedroom

Living Room

X

## SECOND FLOOR

Bathroom

Small

Bedroom

X

X

X

Hall

X

Large Bedroom

Window C-1

Window C-2

## Understanding Your Report

- The Summary Results section lists all of the areas that failed the dust-lead test. The areas represented by the sample needs to be re-cleaned to see if the cleaning removed the contaminated dust. Deteriorated painted surfaces should be repaired using interim controls or abatement techniques.

- For written information on how to address lead hazards, call the National Lead Information Center (NLIC) at 1-800-424-LEAD (1-800-424-5323). You may consider hiring a risk assessor to evaluate lead hazards in your home and recommend a lead hazard control plan. Risk assessors can be located by calling NLIC at

1-800-424-LEAD or by visiting cdxapps.epa.gov/leadpro.

- The laboratory result forms attached to the report list the analyst, all of the areas sampled inside and outside the building and the laboratory analysis results for each sample.

- The dust-lead test results are expressed in micrograms per square foot (μg/ft2); soil samples are expressed in micrograms per gram (μg/g).

- Areas that failed the dust-lead test showed lead dust levels above EPA dust-lead action levels. The guidance that was used during this dust-lead test is as follows:

EPA Actions Levels for Lead Dust Carpeted and uncarpeted floors: < 5 μg/ft2 Interior window sill (stool): < 40 μg/ft2

Window trough: < 400 100 μg/ft2