# Attachment 6-B

Activity 3: Interpreting Laboratory Results

Instructions: The purpose of this activity is to test your ability to verify the results received from the laboratory, compare these results to the EPA action levels/HUD clearance standards, and interpret the results. Using the following excerpt from a dust-lead testing results form, check the laboratory’s calculation of the weighted lead dust sample

| Sample # | Location | Surface | Total Lead (µg) | Dimensions of Sample Area (ft²) | µg/ft² |
| --- | --- | --- | --- | --- | --- |
| X-1 | Kitchen | Floor | 5 | 1.070 | 5.5 |
| X-2 | Kitchen | Interior window sill | 323 | 0.969 | 333.3 |
| X-3 | Kitchen | Window trough | 210 | 0.525 | 400.0 |
| X-4 | Hallway | Floor | 4 | 1.107 | 3.6 |
| X-5 | Bedroom 2 | Floor | 20 | 0.988 | 20.2 |
| X-6 | Hall | Floor | 3 | 1.107 | 2.7 |
| X-7 | Bedroom 3 | Floor | 4 | 1.094 | 3.7 |
| X-8 | Office | Floor | 192 | 1.094 | 17.5 |
| X-9 | Bedroom 3 | Interior window sill | 412 | 0.88 | 468.1 |
| X-10 | Bedroom 3 | Window trough | 52 | 0.67 | 77.6 |
| X-11 | Hall | Floor | 11 | 1.107 | 9.9 |
| X-12 | Office | Interior window sill | 30 | 0.88 | 341.0 |
| X-13 | Office | Window trough | 12 | 0.76 | 15.8 |

- Check the results (µg lead/ft²) for each sample. If the results are incorrect, provide the correct results in µg lead/ft².

- After verifying the laboratory’s results, compare these results to the appropriate EPA dust-lead action levels. Did the individual samples pass or fail to clear to below the dust-lead action levels?

EPA Action Levels for Lead Dust Floors: < 5 µg/ft²

Interior window sills: < 5 µg/ft² Window troughs: < 5 µg/ft²

| Sample # | Result | Dust-Lead Action Level | Pass or Fail? |
| --- | --- | --- | --- |
| X-1 |  |  |  |
| X-2 |  |  |  |
| X-3 |  |  |  |
| X-4 |  |  |  |
| X-5 |  |  |  |
| X-6 |  |  |  |
| X-7 |  |  |  |
| X-8 |  |  |  |
| X-9 |  |  |  |
| X-10 |  |  |  |
| X-11 |  |  |  |
| X-12 |  |  |  |
| X-13 |  |  |  |

# Activity 3: Interpreting Laboratory Results in a HUD-Funded Project

Instructions: The purpose of this activity is to test your ability to verify the results received from the laboratory, compare these results to EPA actions levels/HUD clearance standards, and interpret the results. Using the following excerpt from a dust-lead testing results form, check the laboratory’s calculation of the dust lead loading. (Note: To keep the exercise within the class time available, just the first 13 samples in the lab report are shown)

| Sample # | Location | Surface | Total Lead (µg) | Dimensions of Sample Area (ft²) | µg/ft² |
| --- | --- | --- | --- | --- | --- |
| X-1 | Kitchen | Floor | 5 | 1.070 | 5.5 |
| X-2 | Kitchen | Interior window sill | 323 | 0.969 | 333.3 |
| X-3 | Kitchen | Window trough | 156 | 0.526 | 296.6 |
| X-4 | Hallway | Floor | 4 | 1.107 | 3.6 |
| X-5 | Bedroom 2 | Floor | 50 | 0.988 | 50.6 |
| X-6 | Bedroom 2 | Interior window sill | 289 | 0.898 | 321.8 |
| X-7 | Bedroom 2 | Interior trough | 154 | 0.775 | 198.7 |
| X-8 | Hall | Floor | 3 | 1.107 | 2.7 |
| X-9 | Bedroom 3 | Floor | 4 | 1.094 | 3.7 |
| X-10 | Office | Floor | 192 | 1.094 | 17.5 |
| X-11 | Bedroom 3 | Interior window sill | 412 | 0.88 | 468.1 |
| X-12 | Bedroom 3 | Interior trough | 52 | 0.67 | 77.6 |
| X-13 | Hall | Floor | 900 | 1.107 | 813.0 |

- Check the results (µg lead/ft²) for each sample. If the results are incorrect, provide the correct results in µg lead/ft².

- After verifying the laboratory’s results, compare these results to the appropriate EPA dust-lead action levels. Did the individual samples pass or fail to clear to below the dust-lead action levels?

EPA Action Levels for Lead Dust Floors: < 5 µg/ft²

Interior window sills: < 5 µg/ft² Window troughs: < 5 µg/ft²

| Sample # | Result | Dust-Lead Action Level | Pass or Fail? |
| --- | --- | --- | --- |
| X-1 |  |  |  |
| X-2 |  |  |  |
| X-3 |  |  |  |
| X-4 |  |  |  |
| X-5 |  |  |  |
| X-6 |  |  |  |
| X-7 |  |  |  |
| X-8 |  |  |  |
| X-9 |  |  |  |
| X-10 |  |  |  |
| X-11 |  |  |  |
| X-12 |  |  |  |
| X-13 |  |  |  |