Attachment 1-A: Comparing Lead Evaluation Professionals

| Lead Evaluation Professional | Is qualified to perform | Is not qualified to perform | Training hours required for certification | Skills |
| --- | --- | --- | --- | --- |
| Lead dust sampling technician | - Post-renovation dust-lead testing - Dust-lead testing after hazard reduction or maintenance activities in HUD-covered properties | - Post-abatement clearance - Soil and paint testing | - 8 training hours | Perform: - Visual inspection - Lead dust wipe sampling Identify dust lead hazards after renovation. |
| Lead-based paint inspector | - Paint inspections - All clearance below the action levels | - Risk assessments | - 24 training hours | Perform: - Visual inspection - Paint chip sampling - Paint testing by XRF - Post-work lead dust wipe sampling Identify the existence and location of lead-based paint. |
| Lead-based paint risk assessor | - Risk assessments - Paint inspections - All clearance below the action levels | N/A | - 40 training hours (24 inspector hours and 16 risk assessor hours) | Perform: - Resident interviews - Visual inspection - Lead dust wipe sampling - Soil sampling - Paint chip sampling - XRF testing Assess a unit, identify all lead hazards, and recommend methods for lead hazard reduction. |