# Chapter 2: Visual Inspection

# Objectives

The major objective of this chapter is to teach students to perform visual inspections. Specific objectives include the following:

- List the items that should be identified in a visual inspection

- Identify:

  - Visible dust or debris

  - Paint chips

  - Deteriorated paint

- Record results on a visual inspection form

# Introduction to this chapter

This chapter is designed to give students the tools to perform visual inspections and report their findings on standard forms in a manner that is understandable to clients and future contractors. This chapter will

demonstrate what students should look for as they perform visual inspections and where they should look for potentially problematic areas within housing units.

# Activities

Photographs tell the story. Slides 2-7 through 2-15 have photographs of deteriorated paint caused by different conditions. The object here is to get students thinking about the many sources of deteriorated paint and how they might be creating dust or paint chips in the home. For each picture, discuss with the group the likely source of the problem and why this deteriorating paint could be a problem.

# Review

See Slide 2-17 for this chapter's review topics. Ask students if they have any questions about the material before moving on to the next chapter

# Notes

The pictures included in Slides 2-7 through 2-15 should be "narrated" and explained to students so that they understand that these images demonstrate problematic conditions in the units they are assessing. Students should understand how to recognize each of these conditions by the end of the chapter.

Be sure to point out Attachments 2-A and 2-B, a blank and completed sample visual inspection form, which students can use to model their notetaking as they perform visual inspections.

Chapter 2: Visual Inspection

Lead Dust Sampling Technician Refresher Training

Course

January 12, 2026

1

Notes:

Objectives

- Learn what a visual inspection is

- Learn the steps for performing a visual inspection under both EPA's and HUD's regulations

- Learn when to look for deteriorated paint, visible dust or debris, and paint chips

- Record results on a visual inspection form

2

Notes:

This chapter will outline the steps a lead dust sampling technician must take to perform a visual inspection. A visual inspection is the first activity to perform on site for any dust-lead testing. This chapter will also highlight the differences in visual inspection between EPA's RRP Rule and HUD's LSHR.

Visual Inspection

- Under both EPA's and HUD's rules, visual inspection is the first step in the post-work dust-lead testing process.

- The visual inspection determines whether the unit/work area {interior and exterior) is free of visible dust and debris before dust-lead testing can begin.

3

Notes:

The visual inspection determines whether the unit/work area is clear of conditions that can result in exposure to lead-based paint hazards, such as paint chips, debris, visible dust, and deteriorated paint.

If these conditions are present, the unit does not meet EPA's and HUD's rules for dust-lead testing.

HUD generally requires visual inspection and clearance of an entire unit, with worksite-only clearance allowable under certain conditions.

Visual Inspection - EPA RRP Dust-Lead Testing

- At the conclusion of the renovation, the certified renovator may have conducted a visual inspection to look for paint chips, dust, and debris.

- The LDST must conduct a separate visual inspection of the work area to ensure that the area is ready for dust-lead sampling.

- If any paint chips, dust, or debris are found, the renovation firm should re-clean these areas before the LDST begins to collect dust wipe samples.

4

Notes:

It is important for the lead dust sampling technician to understand that the renovation firm may perform his or her own preliminary visual inspection. The lead dust sampling technician should also perform a visual inspection of the work area. If the lead dust sampling technician observes paint chips, dust, or debris in the work area, these conditions must be brought to the attention of the certified renovator for re- cleaning.

- Addresses entire unit unless worksite-only clearance is allowed.

- Do not perform dust-lead testing if unit/work area does not pass visual inspection.

- If deteriorated paint, dust, or debris is found, it must be eliminated before dust testing may begin.

  - See Attachments 2-A and 2-B

5

Notes:

## Why look for deteriorated paint?

In order to address lead dust in a housing unit, you need to address its sources, including deteriorated lead-based paint. If paint contains lead, deteriorated paint can create chips and dust, which can cause exposure to lead.

NOTE: The LSHR refers to this process as a "visual assessment," but for purposes of this curriculum, the term "visual inspection" is used.

- Inspect exterior area if:

  - Exterior painted surfaces have been disturbed by renovation activity

  - Openings to exterior were not sealed during interior work

- Inspect ground and outdoor living areas close to affected surfaces

- Visible dust or debris must be removed

- Deteriorated paint must be eliminated

- Dust sampling is not performed for exterior work

6

Notes:

## EXTERIOR VISUAL INSPECTION IS NOT REQUIRED IF ONLY INTERIOR WORK IS PERFORMED.

An independent third party is needed to do an exterior visual inspection if the exterior work was done under HUD's LSHR. A certified lead dust sampling technician is qualified to perform this inspection.

For more information, see Appendix B or 24 CFR 35.1340.

HUD NOTE: HUD requires sampling technicians to verify with renovator that openings to the exterior are closed during interior work. If not closed, exterior visual inspection is required for interior work.

Dust-lead sampling is not required for exterior work. There are no dust-lead action levels for porches, balconies, railings, or other horizontal exterior features.

7

Notes:

- Identify any paint that is not intact:

  - Chipping

  - Peeling

  - Chalking

  - Cracking

  - Holes, moisture, and friction damage

- Hairline cracks and nail holes are not considered deteriorated paint.

8

Notes:

Deteriorated paint is any paint that is not intact. It does not have to be peeling paint. As seen in the following photographs, deteriorated paint can include:

- Chipping paint on door and window trim

- Peeling paint and flaking paint on walls and window sashes

- Paint with small bubbles that look like blisters

- Paint with lines and cracks that make it easy to peel the paint away

- Paint that is "chalking" or creating chalk-like dust

Note: Hairline cracks and nail holes are not considered deteriorated paint.

- Dust

  - Dust you can see

- Debris

  - Pieces of wood, bits of plaster, and various other building pieces covered in paint

- Paint chips

  - Small pieces of paint

9

Notes:

What do visible dust, paint chips, and debris look like?

- Visible dust is dust you can see.

- Debris can be pieces of wood, bits of plaster, and various other building pieces covered with paint that are left in the room or near where the work was done.

- Paint chips are little pieces of paint. Chips can be even smaller than your fingernail or larger than your hand. Look for paint chips on floors and windows.

- Chipping paint

10

Notes:

- Holes in wall

11

Notes:

- Deteriorated paint

12

Notes:

- Cracking paint

13

Notes:

The arrow here points to a crack in the paint.

- Moisture damage

14

Notes:

Moisture can be the cause of many paint problems. In this case, it is causing the paint to bubble.

- Friction damange

15

Notes:

- Be precise about locations.

- Write down results as you go.

- Write down other information, indicating source.

- See sample visual inspection form.

  - Attachment 2-B

16

Notes:

When recording the results of a visual inspection, take the following steps:

- Be precise about locations {e.g., room descriptions and/or specific areas in room) where visible dust, debris, paint chips, and deteriorated paint were found.

- Write down results as you go along.

- Write down other information the client provides about the surface in question. See Completed Sample Visual Inspection Form in Attachment 2-B.

Review

- Visual inspection is the first step in the post-work dust-lead testing process.

- EPA's visual inspection determines that the area is free of dust and debris before dust-lead testing can begin.

- HUD's visual inspection also checks for deteriorated paint and generally covers the entire unit unless worksite-only clearance is allowed.

- Visual inspection as part of post-work dust-lead testing is the responsibility of the certified lead dust sampling technician.

- Be methodical in your visual inspection, and record results in a

17

consistent manner.

Notes:

- See Slides 3 to 16 for information and answers.

- See Slide 3 for information and answers.

- See Slides 3 to 9 for information and answers.