# Module 2: Regulations

Review the purpose of this module with the students

- Purpose: To give the renovators in the training course knowledge of the regulatory requirements that affect renovation work, and to make them familiar with resources that they can use to help understand the EPA and HUD Rules.

Overview of this module: The information below summaries the content and teaching methods for this module. This is for your reference, do not cover this with the participants.

##### Module 2: Regulations (40 minutes)

Key message: Know the EPA and HUD Rules. These rules set forth specific and performance-based requirements that must be mastered to achieve compliance.

Presentation Outline:

- 2-1: Regulations Overview

- 2-2: The RRP Rule

- 2-3: The RRP Rule: Exclusions

- 2-4: The RRP Rule: Firm Certification

- 2-5: The RRP Rule: Firm Responsibilities

- 2-6: The RRP Rule: Individual Certification

- 2-7: The RRP Rule: Certified Renovator Responsibilities

- 2-8: The RRP Rule: Work Practice Standards

- 2-9: The RRP Rule: Enforcement

- 2-10: HUD’s Lead Safe Housing Rule

- 2-11: HUD’s Lead Safe Housing Rule: Safe Work Practices

- 2-12: HUD’s Rule Addresses

- 2-13: Know the EPA and HUD Rules!

- 2-14: Tribal, State, Territorial, Local Regulations

- 2-15: Now You Know…

Notes: Know your audience. All contractors need to know all the regulations.

Preparing for this module: Review materials in advance. Read all federal, state and local regulations.

Module 2: Regulations

Lead Safety for Renovation, Repair, and Painting Initial Training Course

0

Notes:

### Slide 2-1: Regulations Overview

Mention that Title X (spoken as “Title Ten”) is the enabling law for both EPA and HUD in which Congress assigned them the authority to regulate various parts of the lead industry.

EPA was tasked to develop training and certification regulations for renovators to follow when working with lead in pre-1978 housing and child-occupied facilities.

HUD was tasked with establishing guidance for work practices used in lead abatement and lead hazard control work. HUD is also responsible for regulations governing renovation and lead hazard control work in federally-assisted target housing. OSHA is responsible for developing safety regulations for lead in the construction and general industries.

Tribal, State, Territorial and Local Regulations:

- Cover applicable state and local regulations at the end of this module.

- The EPA Renovation, Repair, and Painting Program Final Rule allows EPA-authorized Tribes, states and territories to administer the RRP Rule requirements for EPA. In these Tribes, states and territories, EPA certification and accreditation is provided from the authorized program.

- In non-authorized Tribes, states and territories, EPA and Tribal, state and territorial requirements must both be satisfied.

Regulations Overview

- U.S. Environmental Protection Agency (EPA):

  - Established accredited training and certification programs for workers, supervisors, inspectors and risk assessors conducting evaluation or abatement of lead-based paint

  - Established requirements for pre-renovation education

  - Promulgated the Renovation, Repair, and Painting Program Final Rule (RRP Rule)

- U.S. Department of Housing and Urban Development (HUD):

  - Established actions in federally-assisted target housing

  - Established federal grant programs

  - Established guidelines for lead-based paint evaluation and control; established the Lead Safe Housing Rule

- U.S. Occupational Safety and Health Administration (OSHA):

  - Established worker protection standards	1

Notes:

U. S. Environmental Protection Agency (EPA):

- EPA has established training requirements for people involved in lead abatement (permanently addressing lead-based paint hazards). Certified renovators may not perform lead-based paint abatement unless they are certified lead abatement workers or certified lead abatement supervisors.

- Lead abatement is defined as any measure or set of measures designed to permanently address or cover lead-based paint or lead-based paint hazards. Abatement includes, but is not limited to: (1) The removal of paint and dust, the permanent enclosure or encapsulation of lead-based paint, the replacement of painted surfaces or fixtures, or the removal or permanent covering of soil, when lead- based paint hazards are present in such paint, dust or soil; and (2) All preparation, cleanup, disposal, and post-abatement dust-lead testing activities associated with such measures. (40 CFR 745.223).

- Abatement does not include renovation, remodeling, landscaping or other activities, when such activities are not designed to permanently address lead-based paint hazards, but are designed to repair, restore, or remodel a given structure or dwelling, even though these activities may incidentally result in a reduction or elimination of lead-based paint hazards. Furthermore, abatement does not include interim controls, operations and maintenance activities, or other measures and activities designed to temporarily, but not permanently, reduce lead-based paint hazards. (40 CFR 745.223).

- Module 3 has information on the pre-renovation education requirements and RRP Rule details are in the next slides.

U. S. Dept. of Housing and Urban Development (HUD):

- If you work in federally-assisted target housing, certain actions are required to address lead hazards. In these cases, the workers must have proper training. See Appendix 2 for more information on the HUD requirements for worker training and lead hazard reduction in federally-assisted housing.

- HUD has a grant program for state and local governments to fund lead hazard reduction activities.

- Check with nearby states and localities to find out if there are any local programs (which may be state or federally funded) that are designed to address lead hazards.

U. S. Occupational Safety and Health Administration (OSHA): OSHA has a Lead in Construction Standard which outlines worker protection requirements. Your employer should be aware of these. For more information, on the OSHA Lead in Construction Rule, see 29 CFR 1926.62 (www.osha.gov/sites/default/files/publications/osha3142.pdf).

Tribal, State, Territorial and Local Regulations: Tribal, state, territorial and local regulations may also apply to the renovation work you do. Where applicable, these requirements will be covered at the end of this module.

### Slide 2-2: The RRP Rule

When covering this slide make sure to define the terms below.

- Target Housing is a home or residential unit built on or before December 31, 1977, except:

  - Housing built for the elderly or persons with disabilities (unless any child who is less than 6 years of age resides or is expected to reside in such housing); or

  - Zero-bedroom dwellings such as studio apartments, hospitals, hotels, dormitories, etc. (unless any child who is less than 6 years of age resides or is expected to reside in such housing).

  - Note: Target housing may also be referred to as “pre-1978 housing”.

- A Child-Occupied Facility is any pre-1978 building or portion of a building that is regularly visited by the same child, under 6 years of age; and their visits are at least two different days within any week, for at least 3 hours each day and where the combined weekly visits exceed 6 hours and combined annual visits exceed 60 hours. Child-occupied facilities include schools, childcare facilities, and daycare centers.

Note that EPA may authorize Tribes, states and territories to enforce all aspects of the RRP Rule. EPA enforces the RRP Rule in non-authorized Tribes, states and territories.

## The RRP Rule

  - Addresses activities that disturb lead-based paint in target housing and child-occupied facilities

  - Requires:

    - Renovators to be certified through training

    - Firms to be certified

    - Training providers to be accredited

    - Lead-safe work practices during renovations

    - Pre-renovation education in target housing and child-occupied facilities

  - Firms working in pre-1978 homes and child-occupied facilities must be certified and use lead-safe work practices during renovations

  - EPA may authorize Tribes, states and territories to enforce the rule	2

Notes:

EPA’s Renovation, Repair and Painting Final Rule:

- Published April 22, 2008, under the authority of the Toxic Substances Control Act (section 402(c)(3) of TSCA).

- The final rule addresses lead-based paint hazards created by renovation, repair and painting activities that disturb lead-based paint in “target housing” and “child-occupied facilities.”

  - Target housing is a home or residential unit built on or before December 31, 1977, except:

    - Housing designated for the elderly or persons with disabilities (unless any child who is under 6 years of age resides or is expected to reside in such housing).

    - Any zero-bedroom dwelling such as studio apartments, hospitals, hotels, dormitories, etc. (unless any child who is under 6 years of age resides or is expected to reside in such housing).

  - A child-occupied facility is a pre-1978 building that meets all three of the criteria below:

    - Visited regularly by the same child, under 6 years of age.

    - The visits are on at least two different days within any week (Sunday through Saturday period), provided that each day’s visit lasts at least 3 hours.

    - Combined weekly visits last at least 6 hours, and the combined annual visits last at least 60 hours.

  - Child-occupied facilities may be located in a public or commercial building or in target housing. These facilities include schools, childcare facilities, and daycare centers.

- Tribal, State and Territorial Authorization: EPA may authorize Tribes, states and territories to enforce all aspects of the RRP Rule. EPA enforces the Rule in non-authorized Tribes, states and territories.

The RRP Rule Requirements:

- Training providers must be accredited.

- Renovation firms must be certified.

- Renovators and dust sampling technicians must be trained and certified.

- Non-certified workers must work under and be trained on-the-job by a certified renovator.

- Specific work practices must be followed for work covered by the rule.

- Renovators must educate owners/occupants (covered in Module 3).

### Slide 2-3: The RRP Rule: Exclusions

Explain to students that there are several situations in which the rule will not apply to their work, such as:

- When a lead inspection report prepared by a certified lead-based paint inspector or risk assessor finds that no lead-based paint is present in the target home or child-occupied facility on surfaces that will be disturbed during the work.

- When lead analysis of a collected paint chip sample or an EPA-recognized test kit used by a certified renovator to test surfaces that will be disturbed by renovation, repair or painting work identified no lead-based paint during testing.

- When interior renovation, repair, or painting work disturbs less than 6 square feet of paint per room and no prohibited practices are used. Window replacement and demolition do not qualify for exclusions, even if the amount of disturbed known or presumed lead-based paint is below this limit.

- When exterior renovation, repair or painting work disturbs less than 20 square feet of paint per side of the building and no prohibited practices are used.

- When the work is not conducted for compensation of any kind.

- When an emergency renovation is performed, the pre-renovation education and set- up requirements are waived to the extent necessary to respond to the emergency. Other requirements are waived if necessary to respond to the emergency; otherwise they are in effect. The renovation firm must still clean the home and conduct either cleaning verification or post-renovation clearance below the dust-lead action levels prior to re-occupancy. It would be common for there to be no residents in this case. Personal protective measures and prohibitions on work practices still apply to this work. More information about the emergency provision of the RRP Rule can be found online at www.epa.gov/lead/post-disaster-renovations-and-lead-based-paint.

The RRP Rule: Exclusions

- Renovation activities where affected components do not contain lead-based paint

- Emergency renovations (requires cleanup and cleaning verification)

- Minor repair and maintenance activities

  - This exclusion does not apply to window replacement, demolition or activities involving prohibited practices

- Renovations performed by homeowners in their own homes

3

Notes:

The RRP Rule does not apply to renovation work that meets the following exclusions:

- If the renovation only affects components that do not contain lead-based paint, the rule does not apply to renovation of these components.

- EPA has established limits (see below) for minor repairs or maintenance. Work that does not exceed these limits is exempt from the work practice requirements in the rule. The EPA limits for minor repairs and maintenance are larger than the HUD limits (see the HUD box below).

Minor repair and maintenance activities have been defined in the Rule.

- EPA has defined minor repair and maintenance activities as below.

  - Interior work disturbing less than 6 square feet (6 ft2) per room of painted surface is exempt from the work practices requirements in the Rule. Cleanup and cleaning verification are not required after minor repair and maintenance activities, unless they involve window replacement, demolition, or prohibited practices.

  - Exterior work disturbing less than 20 square feet (20 ft2) of painted surface is exempt from the work practices requirements in the Rule. Cleanup and cleaning verification are not required after minor repair and maintenance activities, unless they involve window replacement, demolition, or prohibited practices.

  - Minor repair and maintenance activities do not include window replacement, demolition or activities involving prohibited practices.

  - The entire surface area of a removed component is the amount of painted surface disturbed. Work, other than emergency renovations, performed within a 30-day period must be considered the same job when determining the amount of paint disturbed.

The HUD Lead Safe Housing Rule applies to every home built prior to 1978 that receives federal housing assistance, typically provided through state and local governments, where greater than HUD’s de minimis amounts of painted surfaces will be disturbed. HUD’s de minimis amounts are: 2 square feet of interior lead-based paint, 20 square feet of exterior lead-based paint or 10% of the total surface area on an interior or exterior type of component with a small surface area that contains lead- based paint. Examples include window sills, baseboards, and trim.

### Slide 2-4: The RRP Rule: Firm Certification

- Remind students that firm certification requires an online application and a fee to be paid to EPA. Firm certification is a different certification from the individual certification received by renovators after successful completion of this course.

- Tell students that certification of a firm lasts for 5 years after it is issued by EPA and gives the firm the ability to operate as renovators in any non-EPA-authorized states.

- Explain that firm certification may also be required by Tribal, state and local governments, and that they need to check with the appropriate agencies to determine what requirements apply.

The RRP Rule: Firm Certification

- All covered renovations must be performed by certified firms, using certified renovators and other trained workers

- To become certified, firms must submit an online application, and pay a fee, to EPA

- Certifications will be good for 5 years

- Certification allows the firm to perform renovations in any non-authorized Tribal area or state

4

Notes:

- No firm working in target housing or child-occupied facilities, where lead-based paint will be affected by the work, may perform, offer or claim to perform renovations without firm certification from EPA, or an EPA-authorized Tribe, state or territory.

- One EPA renovation firm certification is all that is needed for a renovation firm to work in any non-authorized Tribe/state/territory. Firm certification is not the same as the personal certification attained by each renovator‘s successful completion of this course.

- Tribes, states and territories may seek authorization from EPA to operate their own programs. Also, Tribes, states and territories, whether authorized by EPA or not, can establish additional requirements for firms working within their jurisdictions. Be sure to determine if your Tribal, state or territorial government has additional regulations that may affect renovation in your community.

### Slide 2-5: The RRP Rule: Firm Responsibilities

- Review the information on the slide and in the student text.

The RRP Rule: Firm Responsibilities

- Ensure overall compliance with the RRP Rule

- Ensure that all renovation personnel are certified renovators or have been trained on-the-job by certified renovators

- Assign a certified renovator to all jobs

- Meet pre-renovation education requirements

- Meet recordkeeping requirements

5

Notes:

- The certified firm must ensure that everyone on the renovation, repair or painting job is trained to perform lead-safe work practices during the work. EPA requires all persons on the job to be trained. The person responsible for lead-safe work practices must be a certified renovator. Other firm employees (non-certified renovation workers), working on the job, must be trained on-the-job by certified renovators, or must be certified renovators themselves. This could be accomplished by:

  - Having all employees trained as certified renovators; or,

  - Having at least one person trained as a certified renovator, who will then train the rest of the employees in lead-safe work practices. Note that this training must be performed by a certified renovator.

- The certified firm must designate a certified renovator: to conduct set-up activities; to ensure that the renovation is performed in accordance with work practice standards; to verify work and cleanup activities using the cleaning verification procedure; and, to train non-certified renovation personnel on- the-job in lead-safe work practices.

- The certified firm must ensure that the renovation is performed in accordance with the work practice requirements in the RRP Rule.

- The certified firm is responsible for complying with pre-renovation education requirements.

- The certified firm is also responsible for keeping all records including:

  - Pre-renovation education documentation (proof of receipt, proof of delivery, waivers, etc.)

  - Documentation of lead-based paint

  - Training and certification records

  - Cleaning verification records

Note: Recordkeeping is covered in detail in Module 7.

### Slide 2-6: The RRP Rule: Individual Certification

- Certification as a certified renovator only requires successful completion of an EPA-approved training course conducted by an EPA-accredited training provider.

- Explain to the students that certification will last for 5 years from the date of successful course completion and gives them the ability to operate as certified renovators in any non-EPA-authorized state.

- Remind students that they will have to carry a copy of their training information with them on the job.

- Explain that Tribal, state and local entities may also require certification in some areas, and that they need to check with the appropriate agencies to determine what requirements apply.

- To maintain their certification, certified renovators must take Renovator Refresher training from an EPA-accredited training provider before the 5-year anniversary of their initial certification course date.

The RRP Rule: Individual Certification

- To become a certified renovator, an individual must take an EPA-approved 8-hour training course from an EPA-accredited training provider

- The course completion certificate serves to certify renovators (no application to EPA is required)

- Refresher training is required every 5 years

- Workers do not need certification so long as on- the-job training is received from a certified renovator and the work is not HUD-regulated

6

Notes:

All renovations must be directed by certified renovators. Individuals may become certified renovators by completing an EPA-approved 8-hour training course in lead-safe work practices taught by an EPA- accredited training provider. Successful completion of that course will result in a 5 year certification as a certified renovator.

To maintain certification, certified renovators must take an EPA-approved 4-hour refresher course taught by an EPA-accredited training provider before their certification expires.

No application or fee is required to become a certified renovator. Instead, the course completion certificate serves as the renovator certification. A “copy” of the initial and/or refresher course completion certificate must be available on-site during the work.

Tribes, states and territories may establish requirements for individual renovators working within their jurisdictions. Be sure to determine if your Tribal, state or territorial government has additional regulations that may affect what you must do and where you may work.

HUD requires instructor-led training for all workers unless they are supervised by a certified abatement supervisor (who, under the RRP Rule, must also be a certified renovator).

### Slide 2-7: The RRP Rule: Certified Renovator Responsibilities

- Review the slide and student text with students.

The RRP Rule: Certified Renovator Responsibilities

- Perform work and direct lead-safe work practices

- Provide on-the-job training to non-certified workers

- Keep copy of the initial and/or refresher training certificates onsite

- When requested, use EPA-recognized test kits or, alternatively, collect paint chip samples for laboratory lead analysis to identify lead-based paint

- Be physically present while posting signs, containing work areas, and cleaning work areas

- Be available by telephone when off-site

- Maintain the containment to keep dust and debris within the work area

- Implement the cleaning verification procedure

- Prepare and maintain required records	7

Notes:

The RRP Rule requires that an individual certified renovator be responsible for the renovation job regardless of the level of training and certification of the other persons working on the job. This individual certified renovator has the following responsibilities:

- Perform work and direct the work of non-certified renovation workers.

- Train all non-certified workers on-the-job in lead-safe work practices.

- Maintain copies of initial and/or refresher training certifications onsite.

- When requested, conduct testing for lead-based paint using EPA-recognized test kits or lead analysis of paint chip samples and report findings.

- Remain onsite during the sign posting, work area setup, and cleanup phases of work.

- When not on site, be available by telephone or pager.

- Make sure that the containment is maintained in a way that prevents the escape of dust and debris. This responsibility implies a need to determine which work practices should be used to minimize dust.

- Conduct the cleaning verification procedure to make sure that the work is complete and that the work area is ready to reoccupy.

- Prepare a summary of the work, maintain training and certification records, and certify that all work was done in a lead-safe manner.

### Slide 2-8: The RRP Rule: Work Practice Standard

- Quickly review the topics on the slide.

- Emphasize that the steps involved in lead-safe work practices: Setup, Prohibited Practices and Dust Reduction Practices, Cleanup and Cleaning Verification/Clearance Below the Action Levels and, Recordkeeping, will be addressed in Modules 4, 5, 6 and 7, respectively.

The RRP Rule: Work Practice Standards

- The RRP Rule covers setup of the work area, prohibited work practices, cleanup and the cleaning verification procedure

  - Setup practices, such as posting signs and containing the work area, will be covered in Module 4

  - Prohibited practices and dust reduction suggestions will be covered in Module 5

  - Cleanup practices and cleaning verification procedures will be covered in Module 6

  - Recordkeeping will be covered in Module 7

8

Notes:

HUD had additional work practice requirements. See Slides 2-10 to 2-12.

### Slide 2-9: The RRP Rule: Enforcement

Documentation: Inform the students of the following:

- Documentation of renovations, testing and training related to renovations must be kept for 3 years after the end of the job.

Enforcement: Inform the students of the following:

- EPA has the authority to seek substantial civil and criminal fines, in addition to jail time, for a firm that knowingly and willfully violates the RRP Rule requirements. The penalties vary depending on the type and seriousness of the violation, and are adjusted for inflation annually. In 2025, the maximum civil fine was

$49,722 per offense, with additional criminal penalties of up to $49,722 per offense (plus jail time).

- EPA can also revoke certifications for firms and individuals who violate the RRP Rule requirements.

The RRP Rule: Enforcement

- EPA may suspend, revoke, or modify a firm’s certification if the certified firm or certified renovator is found to be in non-compliance

- Firms found to be non-compliant may be liable for civil penalties of up to $49,722 (in 2025) for each violation

- Firms who knowingly or willfully violate this regulation may be subject to fines of up to an additional $49,722 per violation in 2025, or imprisonment, or both

9

Notes:

Enforcement:

- EPA has the authority to seek substantial civil and criminal fines, in addition to jail time, for a firm that knowingly and willfully violates the RRP Rule requirements. The penalties vary depending on the type and seriousness of the violation, and are adjusted for inflation annually. In 2025, the maximum civil fine was

$49,722 per offense, with additional criminal penalties of up to $49,722 per offense (plus jail time).

- EPA can also revoke certification of a certified firm or a certified renovator who violates the RRP Rule requirements.

- Note that violators may be both certified renovation firms and non-certified contractors who are not aware of or have ignored the requirement to become a certified renovation firm.

### Slide 2-10: HUD’s Lead Safe Housing Rule

- This slide introduces the HUD Rule and outlines the general target audience for that regulation.

- Discuss the types of affected housing, the program-based nature of the Lead Safe Housing Rule requirements, and effective dates.

- Emphasize that in order to determine if a property receives federal financial assistance, you should ask the questions found in Appendix 2 in part D “How to Find Out About Lead-Based Paint Requirements that Apply to Planned Work in Properties Receiving HUD Housing Assistance, such as Rehabilitation or Acquisition Assistance”.

- Point out that, as they are renovated, the numerous pre-1978 homes in foreclosure may be subject to the EPA RRP Rule or HUD regulations, or both, as applicable.

- For a comparison between HUD’s Lead Safe Housing Rule and EPA’s RRP Rule and guidance on how to comply with both review the “HUDs Lead Safe Housing Rule and EPA’s RRP Rule” handout from HUD’s Lead Safe Housing Rule Toolkit found at www.hudexchange.info/programs/lead-based-paint/lshr-toolkit/ introduction/.

HUD’s Lead Safe Housing Rule

- Covers federally-owned or assisted target housing and federally-owned target housing being sold

- Renovators should ask if the housing receives financial assistance

  - If yes, the renovator should ask the owner to find out if the assistance is federal assistance

- Has evaluation and control requirements based on type of assistance:

  - Visual assessment, lead paint inspection

  - Paint stabilization, interim control, abatement

  - Ongoing lead-based paint maintenance

10

Notes:

HUD requirements for federally-assisted housing are similar to those in the EPA rules with some exceptions. In this curriculum the differences between the HUD rules and the RRP Rule will be highlighted when they occur by special text boxes containing the HUD logo. These boxes are located at the bottom of pages on which an EPA requirement and a HUD requirement differ. Appendix 2 contains an overview of the HUD requirements, and a table detailing differences between the rules.

The HUD “Lead Safe Housing Rule” covers pre-1978 federally-owned or assisted housing and federally-owned housing which is being sold. Housing owned and operated by a federal agency other than HUD may be covered by this regulation.

HUD’s rule does not cover “child-occupied facilities” unless they are part of a residential property covered by the rule. This differs from the EPA RRP Rule, which covers housing and child-occupied facilities, whether or not they are federally-assisted. Wherever the EPA and HUD Rules differ, the more protective standard must be followed.

HUD has many programs that provide financial assistance, for example: rehabilitation, community development, acquisition assistance, etc. HUD requires addressing lead-based paint hazards (such as peeling paint, friction and impact surfaces, and high lead dust levels) by linking those activities to the HUD financial assistance. When asking clients if the housing is receiving federal assistance, renovators should recognize that the assistance may come through a state or local government, community development corporation or other local entity, so they may have to ask the client to check into the ultimate source of the assistance funds.

HUD does not recognize on-the-job worker training alone, and generally requires all individuals performing interim controls (see Slide 11) of lead hazards in federally-owned and federally-assisted housing to complete a HUD-approved training course. HUD’s training requirements for work other than abatement are satisfied by successful completion of this EPA/HUD jointly approved Certified Renovator Course.

### Slide 2-11: HUD’s Lead Safe Housing Rule: Safe Work Practices

- Remind students of the components of interim controls.

- Advise students to check with their Tribe, state or territory to find out if Tribal, state or territorial law or regulation allows interim controls to be done by trained but uncertified individuals. If they do, renovation contractors may perform these activities.

- Explain the four activities where HUD requires lead-safe work practices:

  - Paint stabilization – This activity returns non-intact paint to an intact condition.

  - Rehabilitation (receiving <$5,000 per unit) – These are small scale activities that upgrade existing structures. These include activities such as the replacement of one or two windows. Rehabilitation is defined in the Lead Safe Housing Rule as the improvement of an existing structure through alterations, incidental additions, or enhancements. Rehabilitation includes repairs necessary to correct the results of deferred maintenance, the replacement of principal fixtures and components, improvements to increase the efficient use of energy, and installation of security devices.

  - Standard treatments – These are a group of activities, such as treating doors and windows so they do not rub and generate dust, that are conducted to address the potential generation of lead hazards. These treatments are usually done if lead is assumed to be present.

  - Ongoing lead-based paint maintenance – These are tasks that are done over and over again, and include all maintenance activities that disturb lead-based paint.

- Contrast the “de minimis” limits in the HUD Rule with the minor repair and maintenance limits in the RRP Rule.

HUD’s Lead Safe Housing Rule: Safe Work Practices

- HUD’s rule requires lead-safe work practices for:

  - Paint stabilization

  - Interim control of identified lead-based paint hazards

  - Rehabilitation (renovation)

  - Standard treatments

  - Ongoing lead-based paint maintenance

- HUD’s de minimis level is smaller than the RRP Rule minor repair and maintenance level

11

Notes:

HUD’s Lead Safe Housing Rule requires lead-safe work practices for the activities listed on the slide. It specifies prohibited practices, requirements for protecting occupants, and preparing the work site. Special cleaning techniques must be used and clearance achieved.

Lead-safe work practices are required during:

- Paint Stabilization – Renovation to repair non-intact painted surfaces (flaking, peeling, or otherwise damaged) by performing substrate repair (if needed), surface preparation and repainting. The result is an intact painted surface.

- Interim Controls - Interim controls are defined by HUD to include repairs, painting, temporary containment, specialized cleaning, clearance, ongoing lead-based paint maintenance activities, and the establishment and operation of management and occupant education programs.

- Rehabilitation – This is HUD’s term for the renovation of properties.

- Standard Treatments - a set of measures that reduce all potential lead-based paint hazards in a dwelling unit when lead-based paint is presumed to be present (no lead-based paint evaluation is performed); all deteriorated paint is treated as a lead-based paint hazard.

- Ongoing Maintenance – Normal maintenance activities.

In federally-owned/assisted target housing, all areas of deteriorated paint in the work area must be repaired. Work affecting less than the small – “de minimis” – amounts listed below is not required to follow the lead-safe work practices and clearance requirements in the HUD Rule. HUD’s “de minimis” limits are smaller than the limits for minor repair and maintenance activities in the EPA’s RRP Rule. HUD’s “de minimis” amounts are:

- 2 square feet in any one interior room or space.

- 20 square feet on exterior surfaces.

- 10% of the total surface area of small interior or exterior component type.

HUD’s clearance requirements are covered in Module 6. In general, clearance is required after all work above HUD’s de minimis amounts, and is performed by a certified professional, such as a lead inspector, lead risk assessor, or lead dust sampling technician, who is independent of the certified renovation firm. Tribal, state, territorial and local jurisdictions may have different dust-lead testing requirements than HUD and EPA; the most stringent requirements must be used.

### Slide 2-12: HUD’s Rule Addresses

- Review the slide and student notes with the class. Point out the differences between the EPA and HUD Rules.

- The notes under the slide mirror information found in the Appendix 2 table: Differences between HUD

LSHR and the RRP Rule.

HUD’s Rule Addresses

- Training (usually classroom training for workers)

- Occupant protection and worksite preparation

- Prohibited methods (3 in addition to the RRP Rule)

- De minimis levels (smaller than the RRP Rule)

- Lead-safe work practices

- Specialized cleaning

- Clearance examination (covered in Module 6)

- Occupant notification (within 15 days)

12

Notes:

The HUD Lead Safe Housing Rule (LSHR) covers renovation work in federally-assisted or owned target housing, and specifically addresses the following lead-safe activities.

Training: EPA requires that certified renovators be responsible for renovation projects. Because of this requirement, there are now two major training options for renovation work under the LSHR:

- All renovation workers on the job are trained as certified renovators; or,

- The designated certified renovator is also a certified lead abatement supervisor, and all workers who are not certified renovators have on-the-job training in lead-safe work practices (see Module 8).

Occupant Protection and Worksite Preparation: Occupants have to be kept out of the work area during the renovation work, and must be relocated from the unit during longer renovation projects. EPA-recognized test kits may not be used to test for lead-based paint (LBP); only a certified lead inspector or risk assessor may determine whether LBP is present.

Prohibited Practices: HUD prohibits the same practices as the EPA RRP Rule, plus three more:

- Heat guns that char paint;

- Dry scraping or sanding except within 1 foot of electrical outlets; and,

- Use of a volatile stripper in poorly ventilated space.

De minimis levels: HUD has a smaller de minimis threshold for interior work than EPA’s limit for minor repair and maintenance activities. See the notes on the previous slide for details.

Clearance Examination: HUD requires a clearance examination after renovation work above the de minimis level, in homes regulated by the LSHR. HUD requires a clearance examination by a party independent of the renovator, and, therefore, does not allow acceptance of the certified renovator’s visual inspection or use of the cleaning verification procedure. When the HUD LSHR applies to your work (see Appendix 2), a clearance examination must be performed by a certified professional such as lead inspector, lead risk assessor, or lead dust sampling technician. Some Tribal, state and local authorities have different dust-lead testing requirements and standards.

Occupant Notification: HUD requires notices to be distributed to occupants within 15 days after LBP or LBP hazards in their unit (and common areas, if applicable) are identified, and within 15 days after completion of the hazard control work in their unit or common areas.

### Slide 2-13: Know the EPA and HUD Rules!

- Quickly present this for informational purposes.

Know the EPA and HUD Rules!

- Download the rules and other information from the following websites:

  - www.epa.gov/lead

  - www.epa.gov/lead/lead-renovation-repair-and-painting-program-rules

  - www.hudexchange.info/programs/lead-based-paint/lshr-toolkit/ introduction/

13

Notes:

Individuals and firms performing renovation, repair, and painting in pre-1978 dwellings and child-occupied facilities should understand the EPA RRP Rule.

Individuals performing renovation, remodeling, and rehabilitation in pre-1978 housing that is federally-assisted, need to understand the HUD LSHR. Appendix 2 contains more information on the HUD LSHR.

### Slide 2-14: Tribal, State, Territorial, Local Regulations

- This slide is reserved for Tribal, state, territorial and local regulations that apply to the certified renovator and the certified firm. If Tribal, state, territorial or local requirements for renovations differ from EPA and HUD requirements, they must be included in the course information at this point in the presentation.

- Instructors must determine what additional Tribal, state, territorial and local regulations apply to renovation work and add that information to this page and to the accompanying slide and student notes.

- Copies or summaries of applicable Tribal, state and local regulations must be included in Appendix 7, which has been reserved for that purpose.

- Tribes, states, territories and localities may have different regulations than EPA and HUD for renovations in target housing

- Check with your Tribal, state, territorial and local housing and environmental agencies to obtain information about such requirements

- Appendix 7 is reserved for copies or summaries of Tribal, state, territorial and local regulations

14

Notes:

Reserved for student notes on Tribal, state, territorial and local regulations for renovation that differ from the EPA and HUD regulations

### Slide 2-15: Now You Know…

- Quickly read through the slide and use it as a bridge to the next topic (Module 3: Before Beginning Work).

- EPA’s RRP Rule applies to renovation in housing and child-occupied facilities built before 1978 that contain lead-based paint

- Always take into account the requirements and responsibilities of certification for certified firms and certified renovators, and to

re-certify every 5 years

- Comply with setup of the work area, prohibited work practices, cleanup and the cleaning verification procedure requirements in the RRP Rule

- Determine whether your renovation job is regulated by EPA, HUD, both, or neither

15

Notes: