# Module 4: Contain Dust During Work

Overview of this module: The information below summarizes the content and teaching methods for this module. This is for your reference, do not cover this with the participants.

##### Module 4: Containing Dust During Work (90 minutes)

Key message: Keep the dust in the work area and make it easier to cleanup. Presentation Outline:

  - 4-1: Overview

  - 4-2: What is Containment?

  - 4-3: Keep Dust Within the Containment

  - 4-4: Vertical Containment

  - 4-5: Interior Containment: Limit Access and Post Signs

  - 4-6: Interior Containment: Remove or Cover Belongings

  - 4-7: Interior Containment: Cover Floors

  - 4-8: Interior Containment: Close Windows, Doors, HVAC

  - 4-9: Interior Containment: Work Area Entry Doorway

  - 4-10: Overview of Interior Containment Steps

  - 4-11: Exterior Containment: Establish the Work Area

  - 4-12: Exterior Containment: Close Windows and Doors

  - 4-13: Exterior Containment: Extra Precautions

  - 4-14: Overview of Exterior Containment Steps

  - 4-15: Hands-on Exercise: Interior and Exterior Containment (Skill Sets #2-#5)

  - 4-16: Debrief of Hands-on Exercise

  - 4-17: Now You Know…

Notes: Slides are followed by an exercise.

  - Slides (45 minutes)

  - Hands-On Exercise - Students set up containment in a small area. They lay plastic and secure it. Trainer demonstrates how to do a door flap. (45 minutes)

Preparing for this module: Prepare materials for hands-on exercise and identify appropriate locations for groups to work in. Review materials in advance so you are familiar with Skill Set #2, #3, #4, and #5.

Materials needed: See the Setup Supply List for materials needed.

Option: The hands-on exercise in this module is not optional, but the trainer may choose when to use the exercises. Trainers may choose to follow Lesson Plan 2 and defer hands-on exercises until the end of the day, when hands-on exercises are combined and conducted all at once in the Hands-On Activities Session.

Lead Safety for Renovation, Repair, and Painting Initial Training Course

0

Notes:

### Slide 4-1: Overview

  - Module objective: The purpose of this module is to learn how to follow a few simple setup techniques that will stop the spread of lead-contaminated dust to non-work areas during both interior and exterior work.

  - This module covers the bulleted list of topics on the slide. Review this list with the class participants. Do not spend a lot of time on this slide, as the slides in the rest of the module will answer many questions that arise.

- What is containment?

- Containing dust for interior activities

- Containing dust for exterior activities

1

Notes:

Upon completion of this module you will be able to:

  - Establish containment systems that will keep dust inside of the work area to allow you to clean more efficiently at the end of the day and at completion of the job

  - Identify containment requirements for interior renovations

  - Identify containment requirements for exterior renovations.

### Slide 4-2: What is Containment?

  - This slide may best be covered using a question and answer format. The slides following in the module identify equipment needed for and how to do containment setup. Don’t try to cover everything in the module on this slide.

#### Questions for class discussion:

  - How does containment protect co-workers and residents?

    - Answer: Containment keeps dust isolated to the work area where workers who are protected by and trained in the use of special equipment can work safely. It also keeps residents out of the work area until the job and cleanup are complete.

  - How does containment make cleanup easier at the end of a job?

    - Answer: By limiting the cleanup area to approximately the work area plus 2 feet beyond the work area.

#### Highlights:

  - As explained in the student notes, highlight the requirements under the RRP Rule for containment in target housing and child-occupied facilities as well as the HUD requirement for containment in pre-1978 properties that receive federal housing assistance.

  - See slides 4-6 through 4-9 for additional information on setting up the interior containment. See slides 4-10 though 4-13 for additional information on setting up the exterior containment.

  - Discuss the fact that the work area can be a small area within a room requiring only plastic sheeting (taped to the floor), barrier tape, and signs; or, it can be area including a group of rooms that have been closed off from the rest of the home. The scale of the work area is determined by the Certified Firm based on their experience and on the size of the area which must be protected to prevent the migration of dust

and debris from the work area. The work area is the area that is contained. As the scale of the work area is increased it may include windows, doors and Heating, Ventilation, and Air Conditioning (HVAC) vents. All affected windows must be closed; all affected doors must be closed and sealed; and, all affected HVAC vents must be sealed.

  - Mention that all carpet in the work area must be covered with plastic sheeting. Also explain that a piece of disposable plastic sheeting used as a drop cloth at the work location is very helpful when cleaning up at

the end of the day and during final cleanup. Encourage the use of disposable plastic drop cloths, but noting that they must be disposed of as waste according to the RRP Rule. Show students how to apply plastic drop cloths during the hands-on exercises.

What Is Containment?

- System of temporary barriers used to isolate a work area so that no dust or debris escapes while the renovation is being performed

- Benefits of containment:

  - Protects residents and workers

  - Prevents spread of dust to rest of house/building or neighboring properties

  - Easier cleaning at the end of the job

- Containment is required

2

Notes:

What is containment?

  - In general, there are many degrees of containment, ranging from simple plastic sheeting on the floor surrounding a small work area to a fully enclosed space. Some types of containment are more effective than other types.

  - For purposes of this training, “containment” is what is required under the RRP Rule to prevent dust and debris from spreading beyond the work area to non-work areas.

Containing the work area includes:

  - Removing objects and furniture from the work area, or covering them with plastic sheeting.

  - Covering floors (or the ground) with plastic sheeting a minimum distance beyond the surfaces being renovated (6 feet for interior jobs and 10 feet for exterior jobs).

  - Vertical containment is required for any exterior renovation within 10 feet of the property line.

  - Larger areas of disposable plastic sheeting may also be necessary to prevent the spread of dust.

  - Smaller areas of containment may be used if additional precautions such as vertical containment are used to stop the spread of dust and minimize the area of cleanup.

  - Closing windows and doors, and using plastic sheeting to seal doors and air ducts in the work area.

  - Covering doors used to enter the work area with plastic sheeting in a manner that allows workers to pass through but contains dust and debris within the work area.

Containment is required by the RRP Rule because it:

  - Reduces the risk to you and residents. Following the work area setup requirements of this module will protect you, your co-workers and residents by confining lead-contained dust and debris to a defined and demarcated area. Confining the lead is an important consideration in avoiding exposure. Reducing the risk to you and co-workers is also dependent upon use of personal protective equipment.

  - Facilitates efficient cleaning of the work area. The pre-work setup process is essential to keeping

lead-contaminated dust confined to the work area where it can be easily cleaned. Proper containment of the work area helps to limit the area you need to clean after the job is complete. Knowing exactly where to clean is an important factor in saving time (and money) spent on cleanup.

### Slide 4-3: Keep Dust Within the Containment

  - Review the information on the slide.

  - Ask the participants for examples of dusty jobs. Ask them about the reasoning behind their examples – why one particular job creates and/or spreads more dust than another. Jobs that generate more dust will require larger areas of floor or ground to be covered during work. Discussion about the amount of dust generated by jobs in this module should lead into thinking about how best to design containment to guard against the spread of that dust (and debris) from the work area to other areas of a building or property.

  - Emphasize to the participants that the work practices and equipment used on a job, as well as the size of the job are all factors that affect the amount of dust generated and, therefore, the size and placement of containment. For example, vigorous hand sanding can create enough dust to migrate farther than 6 feet from the work surface.

  - Emphasize that minimizing dust generation through work practice modification and containment planning is helpful in controlling dust produced by high-dust renovation activities.

Keep Dust Within the Containment

- Consider how much dust the renovation will generate

- Containment design is a function of the work practices to be used and the expected amount of dust to be generated during the renovation

- Plan the size and configuration of containment to keep the generated dust within containment

- You are responsible for making sure dust does not migrate out of containment

3

Notes:

If you do not plan and contain the work area correctly, the dust and debris created by renovation can spread beyond the minimum contained area required by the RRP Rule. This means that:

  - For interior locations, dust may migrate more than 6 feet from the surface being renovated.

  - For exterior locations dust may migrate more than 10 feet from the surface being renovated.

Controlling dust and debris may require more extensive containment than is specified in the rule if the job is particularly dusty. Plan accordingly.

In general, renovations that involve only a small amount of paint disturbance create less dust than jobs that involve larger areas of paint disturbance. However, in addition to the size of the area of paint disturbed, the work practices (e.g., sanding) and equipment used will also affect how much dust is created and how the dust migrates. The location of the work activity also has a bearing on the amount of dust that is distributed. For example, small areas of ceiling work can spread dust over the entire room and are very difficult to control.

Required containment is similar for all jobs, but jobs that generate more dust and debris may require protection of larger areas. While the RRP Rule does not require vertical containment except for projects within 10 feet of a property line, these systems may be helpful in limiting the size of the area affected by the work and reduce the area that must be cleaned at the end of the job. Pre-engineered containment systems (purchased and home-made) are very helpful in cutting time spent on the job erecting containment and are easier to install than hanging plastic sheeting with tape. These systems also allow the contractor to create a sealed room within a room where the dust can be completely contained to a limited and controlled area.

Examples of dusty jobs include:

  - Hand scraping large areas

  - Removing paint with a low temperature heat gun and scraper

  - Removing dry residue and paint after using chemical strippers

  - Demolishing painted surfaces

  - Removing building components with painted surfaces that are in poor condition

Remember, you are responsible for making sure that dust and debris remain inside of the contained work area. When planning containment, keep in mind how, how much, and where the work practices to be used will create dust, and plan accordingly.

### Slide 4-4: Vertical Containment

  - Review the information on the slide and in the student notes.

Vertical Containment

- Vertical barrier of plastic sheeting over a rigid frame.

- Required for exterior jobs close to property lines.

- Can be used to minimize floor or ground containment needed.

4

Notes:

What is vertical containment?

Vertical containment means a vertical barrier consisting of plastic sheeting or other impermeable material over scaffolding or a rigid frame, or an equivalent system of containing the work area. Vertical containment is required for some exterior renovations but it may be used on any renovation.

Is vertical containment required for interior jobs?

No. The use of vertical containment is not required for interior jobs, but you can minimize the amount of floor containment needed by making use of vertical containment for interior projects. Floor containment measures may stop at the edge of the vertical barrier when using a vertical containment system consisting of impermeable barriers that extend from the floor to the ceiling and are tightly sealed at joints with the floor, ceiling and walls.

Is vertical containment required for exterior jobs?

Vertical containment, or an equivalent system of containing the work area, is required for exterior jobs where the property line is within 10 feet of the area of paint disturbance. In addition, vertical containment can also be used to minimize the amount of ground containment needed for a project. Ground containment measures may stop at the edge of the vertical barrier when using a vertical containment system.

### Slide 4-5: Interior Containment: Limit Access and Post Signs

  - Review the information on the slide and in the student notes.

Interior Containment: Limit Access and Post Signs

- Notify residents to stay away from the work area

- Do not allow residents or pets near the work area

- Do not allow eating, drinking, or smoking in the work area

- Post warning signs

5

Notes:

Restrict access to the work area and notify residents to stay away while work is underway.

  - Restricting access to the work area will protect residents, especially children and pregnant women, from unnecessary exposures to lead dust and will minimize the spread of dust to non-work areas.

  - Before the job starts, notify the residents not to enter the work area and to stay away from the vicinity of the entrance to the work area as much as possible. Residents and pets coming and going can easily track lead-contaminated dust into non-work areas throughout the home. Non-work areas will likely not be cleaned up promptly or properly.

  - Restricting exposure is especially important for small children under 6 years old and for pregnant women. Be sure to explain to residents that restricting access is for their own protection, and that small children and pregnant women are most at risk of health problems from exposure to lead.

  - You must provide an indication of how long you will be working in a particular area so that residents can plan ahead to obtain items that they may need from the work area before you begin working.

Do not allow eating, drinking, or smoking in the work area.

  - This is primarily for worker protection, but is also important if residents are living near the work area. Post signs that discourage eating, drinking and smoking in the work area. Dust in the air can land on food or be inhaled when smoking. If food is set on a dust-contaminated surface, it can easily pick up the

lead-contaminated dust, which is then ingested when the food is consumed.

Post warning signs.

  - Before beginning the renovation, post a sign in the residents’ native language to warn them and other persons not involved in renovation activities to remain outside of the work area. Signs must remain in place and be readable through completion of the renovation and the post-renovation cleaning verification.

  - A warning sign must be posted: at each entry to a work area; or, at each main and secondary entryway to a building from which occupants have been relocated; or, for exterior work, where it is easily read 20 feet from the edge of the worksite.

### Slide 4-6: Interior Containment: Remove or Cover Belongings

  - Highlight the desirability of removing furniture and other objects from the work area, where practicable. Covering them to prevent dust from settling on them can add to the project workload, as these objects are usually quite hard to clean once they get dust on them. Note that when objects are covered, edges and seams need to be taped or otherwise sealed.

  - Describe the illustration. Point out how all fixtures and pieces of furniture in view are covered and that other pieces (unseen) have been moved.

  - Distribute pieces of plastic sheeting to the class to allow them to get a feel for what they will be working with.

  - It may be helpful to know names of local hardware stores and suppliers that sell protective sheeting, and the typical price per foot of common types or sizes.

Interior Containment: Remove or Cover Belongings

- Remove belongings

- Cover immovable objects in protective sheeting, including:

  - Furniture

  - Carpet

  - Lamps and other fixtures

- Seal edges and seams

6

Notes:

Where practical remove belongings and furniture from the work area.

  - It is desirable to remove all objects from the work area including furniture, rugs and window coverings. Removal is the best option for protecting occupant items from contamination and for reducing post-renovation cleanup time (and cost).

If it can’t be moved out of the work area, cover it.

  - Cover all objects that were not removed from the work area in protective sheeting. Seal the seams and edges with tape. Completely cover all immovable fixtures, furniture, carpets and other personal items with protective sheeting.

  - Secure protective sheeting to the floor with tape so that no dust can get onto the covered items.

  - Protective sheeting such as disposable heavy-duty plastic sheeting is commonly used in many remodeling jobs. Protective sheeting can be bought at most hardware stores.

### Slide 4-7: Interior Containment: Cover Floors

  - Highlight the importance of covering the floor for easier cleanup of dust and debris.

  - Emphasize that this activity is similar to the current practice of covering the floor with a drop cloth but instead they will use plastic protective sheeting.

  - Floors should be covered at least 6 feet out from the surfaces being disturbed or further if necessary to contain the dust.

    - Note: Where vertical containment is used, floor containment measures may stop at the edge of the vertical containment.

  - Highlight the importance of using a disposable tack pad, removing disposal shoe coverings (sometimes called “booties”), wiping shoes, and/or laying plastic on common traffic areas to prevent lead-contaminated dust from being carried to other areas of the building. It is quite common to find high lead-contaminated dust levels along the path from the work area to the bathroom.

  - A disposable tack pad acts like flypaper. It is a sticky paper or cloth that removes dust or debris from a worker’s shoes when he walks on it. Disposable tack pads are available from specialty construction catalogs and many hardware stores.

  - It may be helpful to know names of local hardware stores and suppliers that sell tack pads, wipes and shoe coverings, and the typical cost of each. If possible, bring in samples of these items to show to the class.

  - Advise participants of the importance of cleaning shoes every time they step off the sheeting. Demonstrate using disposable tack pads if available.

Interior Containment: Cover Floors

Required:

- Cover all work area floors with plastic sheeting

- Cover floors a minimum of 6 feet in all directions around the paint being disturbed

Recommended:

- Lay plastic sheeting in high traffic areas

- Take special precautions for carpets

- Use a disposable tack pad at the edge of protective sheeting

- If using chemical stripper, add 2nd plastic layer

7

Notes:

Cover Floors

  - Use protective sheeting to cover all work area floors including installed carpet. The protective sheeting must extend a minimum of 6 feet to the left, right and front – and in some cases to the back – of the area where work will be performed. It should be tightly secured to baseboard or flooring using duct tape

(where appropriate), painters tape or masking tape. The corner edge of the protective sheeting should be reinforced using duct tape or a staple.

    - Note: If vertical containment is used floor containment measures may stop at the edge of the vertical containment.

  - Take special precautions with carpets in the work area. Carpets are a major dust collection medium and it is very difficult to clean the dust out of them once contaminated. When the work area includes carpets, you must cover all carpeted areas that are in the work area with at least one layer of sealed plastic sheeting.

  - Consider covering shoes with removable shoe covers, wiping off the tops and soles of shoes with a damp paper towel each time you step off the sheeting, and/or using a disposable tack pad that removes dust from the soles of shoes. Immediately place used paper towels in a covered garbage bin. Disposable tack pads can be found at many hardware stores or bought through a supply catalog. A tack pad is a sticky pad that you walk on to remove dust from the soles of your shoes. The disposable tack pad can be taped to an outer corner of the sheeting. Replace disposable tack pads at least daily.

  - You may find that using a HEPA vacuum to clean off shoes and clothing is necessary in controlling carry-away dust when personnel leave the work area. This is called a “dry decon” and works well.

  - A second smaller layer of protective sheeting should be used with chemical strippers. This second layer should be taped to the top of the first layer. Place the second layer immediately below the work area. This layer will capture splashes and waste, and allows the mess made by chemical strippers to be cleaned up immediately after use.

  - Use precautions to ensure that all personnel, tools and other items, including the exteriors of waste containers, are free of dust and debris before removing them from the work area. A container of cheap hand or baby wipes is quite useful for such cleaning.

### Slide 4-8: Interior Containment: Close Windows, Doors, HVAC

  - Close and seal HVAC vents to prevent dust from leaving the work area. This is especially important for return (intake) vents as they could easily spread contamination to other areas.

  - Depending on what work is being done and what is required for access to the work area and for construction waste handling, close windows and close and seal all doors in the work area. If doors and windows are left open, air may flow freely through the work area and into non-work areas. Because dust particles are so small, they can easily spread to other areas of the house. Less air flowing through the work area means that there is less chance that lead-contaminated dust will be blown out of the work area.

Interior Containment: Close Windows, Doors, HVAC

- Depending on what work is to be done:

  - Close all windows in the work area

  - Close and seal all doors in the work area

  - Close and seal all HVAC vents in the work area

  - Turn off the HVAC unit (recommended)

8

Notes:

Close Windows

  - Close all windows within the work area.

  - When conducting window replacements from the inside, consider attaching plastic sheeting to the exterior of the window to prevent spread of dust and debris to the ground and other surfaces under the window.

If window replacement affects both interior and exterior surfaces, then setup containment for both the interior and exterior work areas.

  - For dusty jobs, it is strongly recommended that you seal work area windows with protective sheeting to prevent dust from getting into the trough or on the sill, making it harder to clean.

  - When sealing windows, cut plastic sheeting layer slightly larger than the window that you are covering.

  - Attach the plastic sheeting with tape over the window to completely seal it.

  - Make sure that the tape or the sheeting does not cover part of the area on which you are working. Close and Seal Doors

  - Close all doors including closet and cabinet doors in the work area, and cover with plastic sheeting.

  - Doors used as an entrance to the work area must be covered with plastic sheeting or other impermeable material in a manner that allows workers to pass through while confining dust and debris to the work area.

  - As an alternative to putting up plastic, doors may be shut and then sealed closed with painter’s tape. Close and Seal HVAC Vents

  - HVAC systems distribute air throughout the building and thus can also carry dust to other rooms. If possible, turn off the HVAC system for the work area. Close the HVAC supply and return vents in the work area and then cover them tightly with plastic sheeting to prevent air from blowing the dust out of the contained work area and to prevent dust from getting into the HVAC system.

### Slide 4-9: Interior Containment: Work Area Entry Doorway

  - Ask participants: Have you ever limited access to your work area? If so, how? How successful was this? Would you have done this differently?

  - Ask participants: Can you think of any other ways to limit access to the work area?

  - Be sure to highlight all of the points on these two slides if the class discussion has not addressed all of them.

- Cover work area entry doors with two layers of protective sheeting

9

Notes:

  - A physical barrier, such as a cone or warning tape, can be placed outside the entry to remind residents to stay away from the work area, especially in buildings where more than one family lives. The double

layers of plastic on the entry door and other barriers serve as a reminder to residents that people and pets should not enter the work area and also signals that the area has not yet been cleaned up.

When the work area boundary includes a door used to access the work area, cover the door with two layers of protective sheeting as described below.

  - Set up a two-layer entry barrier with closable flaps at the entry to the work area so that workers can pass through but dust and debris stay in the work area. Covering the door with this two-layer system will help contain the dust within the work area. Follow the steps below.

    - Cut the first plastic sheeting layer slightly wider and longer (three inches) than the door frame.

    - Make a small “S” fold at the top of the sheeting and tape it to the top of the door frame. Make a similar “S” fold at the bottom of the sheeting and tape it to the floor. This will ensure that the plastic is not taut.

    - Secure the top corners to the door frame for reinforcement.

    - For exiting and entering the room, tape a vertical line about the size of a man from floor to header on both sides of the plastic. Cut a long vertical slit through the tape, in the middle of the protective sheeting. Leave about 6 inches at the top and bottom uncut. Reinforce the top and bottom of the slit with tape to prevent the plastic from tearing.

    - Tape a second layer of protective sheeting to the top of the door frame. This layer is cut slightly shorter than the door frame so that it will hang down flat against the first sheet of plastic.

    - Tape and secure the top corners of the second layer to the door frame and the first layer. Leave it to hang over the first layer. Weight the bottom of the flap with a dowel to keep it in place. If needed, another weighted flap can be added to the other side of the door to provide a third layer of plastic sheeting.

  - See Steps to LEAD SAFE Renovation, Repair and Painting for more information on how to put the two layer system in place, found in Appendix 5 or www.epa.gov/lead/steps-lead-safe-renovation-repair-and-painting.

### Slide 4-10: Overview of Interior Containment Steps

  - Use this slide to review the previous four steps. Do not go into detail about the steps here. This is meant to provide reinforcement of the four steps to interior containment so that participants will have a structure to organize the information.

The goal of these interior containment practices is to prevent dust and debris from escaping the work area

- Limit access and post signs

- Remove (preferred) or cover belongings

- Cover floors

- Close windows, close and seal doors and HVAC system

- Construct a work area entry doorway

10

Notes:

RRP Rule: Interior Containment General Requirements:

  - Posted signs

    - These must be posted on all sides of the work area to define the work area, must be in the primary language of occupants, must be posted before the beginning of the renovation, and must remain until cleaning verification is achieved.

  - Contain the work area

    - Before renovation, isolate the work area to prevent the escape of dust. During work, maintain the containment integrity and ensure that containment does not interfere with occupant and worker egress from the home or work area.

  - Remove or cover furniture/objects

    - Remove (preferred) objects like furniture, rugs, window coverings; or cover them with plastic sheeting with all seams and edges taped.

  - Cover floors

    - Cover floors including carpets in the work area with taped down plastic sheeting or other impermeable material to 6 feet beyond the perimeter of surfaces undergoing renovation or to a distance sufficient to contain dust, whichever is greater. Remember, if vertical containment is used floor containment measures may stop at the edge of the vertical containment.

  - Close windows, close and seal doors

    - Close windows, close and seal doors in the work area with plastic sheeting or other impermeable material. Doors used as entrances to the work area must be covered with plastic sheeting that allows workers to pass through while confining dust to the work area.

  - Cover duct opening

    - Close and cover all HVAC vents in the work area with taped down plastic sheeting or other impermeable materials (e.g., magnetic covers).

  - Remove dust and debris from everything leaving the work area

    - Use precautions to ensure that all personnel, tools and all other items are free from dust and debris before being removed from the work area.

### Slide 4-11: Exterior Containment: Establish the Work Area

  - Review the importance of protecting the ground and gardens from lead-contaminated dust. Tomatoes are a common homegrown food. Tomato plants concentrate heavy metals from the soil in the fruit. Lead is a heavy metal.

  - The extra length of the protective sheeting required by the Rule is necessary because exterior winds can blow dust a significant distance.

  - Vertical containment or equivalent extra precautions in containing the work area must be used on exterior renovations performed within 10 feet of the property line. It can also be used at other times by the contractor to reduce the minimum size of the work area. For example, if a vertical containment is erected 5 feet from the work plastic sheeting on the ground would not have to extend beyond the vertical containment.

  - The sheeting can be taped to the house or a 2x4 can be wrapped in protective sheeting and placed next to the house if tape will not stick. At the loose edges the sheeting can be weighted down with stones, rocks or any heavy object to prevent the sheeting from flapping or lifting off the ground.

  - Saw horses, tape or orange cones remind residents and alert passersby to stay away from the work area.

  - All toys and belongings should be removed from exterior work areas as part of the pre-work setup.

  - Note that putting a ladder on the plastic sheeting may cause a slip hazard. By putting a piece of plywood between the ladder and the sheeting, that hazard can be avoided. In some cases, you should consider having someone hold the ladder from below.

  - Avoid obscuring tripping hazards with plastic.

Exterior Containment: Establish the Work Area

- Cover the ground with protective sheeting

  - If space permits, extend a minimum of 10 feet from the work area

  - Play special attention and cover nearby vegetable gardens and children’s play areas

- Limit access, place signs

  - Establish a 20 foot perimeter around the work area if space permits

  - Erect a vertical containment

- Vertical containment must be erected if

renovations occur within 10 feet of the property line

11

Notes:

Cover the ground with protective sheeting

  - Lay protective sheeting on the ground below the work area to at least 10 feet from the house, unless the property line prevents 10 feet of such ground cover in which case the firm must erect vertical containment. Extend the work area farther if needed to collect dust and debris; for example, when paint on the second story of a building is being disturbed. Note: Black and clear disposable plastic sheeting can kill plants by making them too hot. Consider using white plastic sheeting instead.

  - Remove toys and other items from the work area and cover all play areas including sandboxes. Protect items that cannot be moved with plastic sheeting.

  - Staple or tape the protective sheeting to the wall of the building, or use a 2x4 wrapped in protective sheeting to hold the material next to the wall. Use heavy objects (e.g., rocks) to weight the other edges of the protective sheeting to the ground so that it won’t blow in the wind.

  - When using ladders on plastic sheeting consider placing a sturdy piece of plywood on the plastic and then set the ladder on the plywood. This will prevent the ladder from puncturing the plastic and will provide a stable surface for the ladder. If plywood is used, take special care to secure it to the ground so that it does not move. This could be done by staking the plywood and later sealing the holes in the plastic with duct tape.

  - Note: Remember children often play in the dirt and may put their hands in their mouths while playing. Dirt, dust or debris on their hands will go into their mouths and may be swallowed.

Limit work area access

  - Limit access to the work area by placing orange cones or sawhorses and warning tape around a 20 foot perimeter of the work area. Ropes with signs at regular intervals could also be used instead of barrier tape. This will help to discourage residents and passersby from entering the work area. Keep pets out of the work area.

Erect a vertical containment

  - If the renovation is within 10 feet of a property line vertical containment or equivalent extra precautions in containing the work area must be used.

  - In addition, firms are permitted to erect vertical containment closer to the renovation activity than the minimum ground containment distance, in which case the ground containment may stop at the edge of the vertical containment.

Exterior work area daily cleaning

  - Cleaning the exterior work area is crucial to prevent the spread of dust and debris. Picking up all debris throughout the day and the use of temporary, plastic-sheeting drop cloths can facilitate easy cleanup. Note that the plastic drop cloths do not take the place of protective sheeting on the ground.

### Slide 4-12: Exterior Containment: Close Windows and Doors

  - Describe the illustration.

  - State the importance of closing windows and doors to prevent dust from blowing into the building. Close all windows and doors within 20 feet of renovation.

Exterior Containment: Close Windows and Doors

- Close all nearby doors and windows that are within 20 feet of the work area

- Use two layers of plastic sheeting on doors in the work area that are being used during the job

12

Notes:

Close and cover all nearby windows and doors.

  - All windows and doors within 20 feet of the work area should be closed to prevent dust from entering the home. Renovators have an obligation to keep the dust and debris from the renovation contained within the work area and contained within the boundaries of the property on which they are working. If the windows and doors of apartments or condominiums are within 20 feet of the work area, consider requesting that the owners or residents of those affected units close the affected windows and doors in order to comply with the RRP Rule. If this is an unavailable option, other methods of restricting dust and debris to the

work area and the work-site property must be considered. These other methods include construction of a vertical containment wall at the property line.

  - On multi-story buildings, close all doors and windows within 20 feet of the renovation on the same floor as the renovation, and close all doors and windows on all floors directly below the area designated as the work area.

Establish two layers of plastic sheeting on the doors in work areas being used during the job.

  - In the exterior work area, there will be times when a door into the house needs to be used to access interior work areas. When this occurs, cover this door with disposable plastic sheeting or other impermeable material in a manner that allows workers to pass through while confining dust and debris to the work area. The steps for placing two layers of plastic sheeting in a doorway are covered in the student notes for slide 4-8.

### Slide 4-13: Exterior Containment: Extra Precautions

  - Review the information on the slide and in the student notes.

Exterior Containment: Extra Precautions

- Some jobs may need additional steps to prevent the spread of dust

  - Extend work area

  - Avoid working in windy conditions, where possible

  - More frequent clean-up of work area

  - Repair any damage to containment promptly

13

Notes:

Jobs needing additional steps

  - Some jobs may require additional steps to avoid the spread of dust to the rest of a property or to adjacent properties. These jobs might include work in high winds that can carry dust out of the work area and work conducted on upper levels of a building during which even light winds can spread dust beyond the containment as it falls.

Extend the work area

  - The simplest solution may be to extend the area of ground covered by plastic sheeting. Avoid high winds where possible

  - Be wary of windy conditions. On days with high winds, it is not advisable to perform dust creating activities. The HUD Rule restricts exterior work in winds in excess of 20 miles per hour. The EPA RRP Rule does not specifically address wind speed, but when the wind is strong enough to move dust and debris, special precautions need to be taken to keep the work area contained. That may mean creating a wind screen of plastic at the edge of the ground-cover plastic to keep dust and debris from migrating.

  - More frequent cleanup of exterior work areas is also not addressed specifically, but frequent cleanup will help the renovator comply with the requirements to contain the work area and prevent dust and debris from getting outside of the work area.

  - Ultimately, you are responsible for preventing dust and debris from leaving the work area, so take appropriate precautions to make that happen when wind is a factor.

### Slide 4-14: Overview of Exterior Containment Steps

  - Use this slide to highlight the three setup practices. Do not go into detail about the steps here.

Overview of Exterior Containment Steps

- Establish the work area

- Close all windows and doors

- Establish, as necessary, additional containment to prevent spread of dust to adjacent properties

- Erect vertical containment for any exterior renovations within 10

feet of the property line

14

Notes:

RRP Rule: Exterior Containment General Requirements:

  - Posted signs

    - Signs must be placed on all sides of the work area to define the area, must be posted in the primary language of occupants, and must be posted before the beginning of the renovation and remain until cleaning verification is achieved.

  - Contain the work area

    - Before renovation, isolate the work area to prevent the escape of dust. During work maintain the containment integrity and ensure that containment does not interfere with occupant and worker egress from the building or work area.

  - Close doors and windows

    - Close all doors and windows within 20 feet of the work area. For multi- story buildings close all windows and doors on the same floor within 20 feet of the work area and all windows on all floors below that are the same horizontal distance from the renovation.

  - Doors used as entrances to the work area

    - Cover doorway openings with plastic sheeting that allows workers to pass through while confining dust to the work area.

  - Cover the ground:

    - Cover the ground with plastic sheeting or other impermeable material extending 10 feet beyond the perimeter of surfaces undergoing renovation or to a sufficient distance to contain dust, whichever is greater, unless the property line prevents 10 feet of such ground cover in which case the firm must erect vertical containment.

  - Ensure that dust and debris do not contaminate or migrate to adjacent areas or properties:

    - In other situations, the renovation will require extra precautions for containing the exterior work area to ensure that dust and debris does not contaminate other properties, such as when working on

multi-story buildings. The RRP Rule establishes performance standards that the contractor must meet, but does not specify how the contractor must meet them. This allows the contractor flexibility in how to comply with the requirement to ensure that no dust or debris leaves the work area.

### Slide 4-15: Hands-On Exercise: Interior and Exterior Containment

Prepare this exercise in advance:

  - Have appropriate tools available. Refer to Skill Sets #2 - #5 in Appendix 6 for needed tools and materials.

  - Have work areas ready. Ideally, each work area will require the participants to move some furniture and cover other furniture, lay plastic on the floor, create a work area entrance, and put up barrier tape or plastic in a doorway. If possible, leave these work areas set up to allow for more realistic exercises as the hands-on exercises are conducted throughout the day.

When conducting the exercise:

- Instruct participants to break into groups of 2 to 6 students.

- Assign each group a work area.

- Circulate while they work to ensure they are doing the work properly. Consult Skill Sets #2 - #5 list of skills they should demonstrate.

- Coach them to stay on task to achieve the training goal.

- Give them a 5-minute warning.

- At the end of each skill set, tell them to stop. Times for each activity are listed on each skill set.

- Debrief at the end of each skill set (#2 - #5).

Options for the Trainer: You may also consider performing Skill Sets #2 - #5 later in the course as part of a larger, comprehensive, hands-on module as described in Lesson Plan 2 in the Note to Instructors on How to Use This Curriculum in the introduction. The comprehensive hands-on module combines the hands-on components from Modules 4, 5 and 6.

Hands-On Exercise: Interior and Exterior Containment

- Practice the following Skills:

  - Skill Set #2: Setting up Barriers, Signs and Flapped Entry Doors

  - Skill Set #3: Cover and Move Furniture

  - Skill Set #4: Establish Interior Containment

  - Skill Set #5: Establish Exterior Containment

- Work in groups of 2 to 6

- Choose the right tools and materials

15

Notes:

Setting up

This exercise gives you a chance to practice setting up the work area signs, barriers and containments. The slide provides basic instruction.

  - Follow the instructions in each skill set. Your instructor may choose to also demonstrate skills.

  - Form groups of 2 to 6 students.

  - Your instructor will assign your group to an area to perform setup activities as if for a job.

  - Choose the right tools and set up the work area to provide proper containment.

### Slide 4-16: Debrief of Hands-on Exercises

Use this slide to debrief the exercise.

  - Ask participants what was difficult about containing the work area.

  - Point out what the participants did well and where there were difficulties.

  - Demonstrate how to do some of the more difficult parts; for example, joining two pieces of plastic, or create an S-fold in the door.

Debrief of Hands-on Exercise

- How did it go?

- What were some of the hard parts?

16

Notes:

Debrief of Hands-on Exercise

  - Consider the questions above.

  - Discuss as a class.

### Slide 4-17: Now You Know…

  - Use this slide to close out the module and make the transition to the next module (Module 5: During the Work).

  - Emphasize that proper containment of the work area will make the rest of the job easier, as they will see in Modules 5 and 6.

Now You Know…

- How to setup:

  - Interior containment

  - Exterior containment

17

Notes:

Now you know how to set up for a job. The next module will discuss the conduct of lead-safe work practices during the job.