# Module 5: During the Work

Overview of this module: The information below summarizes the content and teaching methods for this module. This is for your reference, do not cover this with the participants.

##### Module 5: During the Work (50 minutes)

Key message: Traditional renovation practices produce dust, while lead-safe work practices will reduce dust making the renovation, repair or painting work safer.

Presentation Outline:

- 5-1: Overview

- 5-2: Traditional Renovations Create Airborne Lead Dust

- 5-3: Prohibited Practices

- 5-4: Specialized Tools

- 5-5: Protect Yourself

- 5-6: Control the Spread of Dust

- 5-7: Cleaning During the Job

- 5-8: Exercise: Personal Protective Equipment (Skill Set #6)

- 5-9: Now You Know…

Notes: Slides are followed by an exercise.

- Slides (40 minutes)

- Hands-On Exercise (10 minutes)

Preparing for this module: Prepare a list of tasks for participants to work on and assemble the materials needed for the hands-on exercise.

Materials needed: See the Personal Protective Equipment Supply List.

Options: The hands-on exercise in this module is not optional. Trainers may choose to follow Lesson Plan 2 and defer hands-on exercises until the end of the day, when hands-on exercises are combined and conducted all at once in the Hands-On Activities Session.

Lead Safety for Renovation, Repair, and Painting Initial Training Course

Notes:

### Slide 5-1: Overview

- This module presents lead-safe work practices that minimize dust, the second of the three major steps to lead safety. Setup was the first; the third, cleanup, is covered in the next module.

- This module covers the bulleted list of topics on the slide. Review this list with the class.

- Module objective: The purpose of this module is to teach work practices that reduce the amount of dust that is produced by renovation and to show how to apply them on the job.

- Mention that you will first explain what dust reducing work practices are and then conduct an exercise where the participants can think about how they can apply lead- safe work practices on the job.

- Emphasize that dust reducing work practices are specific practices that have been shown to minimize the creation and spread of lead dust. Many of these practices are not required under EPA’s RRP Rule. Make the point that EPA prohibits certain practices and that the certified firm is responsible to ensure that no dust or debris leaves the containment or contaminates adjacent areas or properties. The student notes have more detail on this issue, and it may be helpful to read the first paragraph.

- Traditional renovations create airborne dust

- Prohibited practices

- Protect yourself and make a personal protective equipment toolkit

- Control the spread of dust

- Hands-on exercise (Skill Set #6)

1

Notes:

Requirements in the EPA RRP Rule

- The RRP Rule prohibits the use of certain practices. These prohibited practices are discussed in this module. This module also contains recommendations regarding how to reduce dust during work activities that are not specifically required or addressed in the RRP Rule.

- Certain practices are prohibited by the RRP Rule. Beyond this you are free to use whatever practices get the job done, provided that all dust and debris you generate stays in the work area and does not migrate to other areas or properties. The recommendations in this section will assist you by reducing the amount of dust released during work. Dust reduction in the work area will make the workplace safer for employees, and will make cleaning easier.

Upon completion of this module, you will know:

- Work practices prohibited under the RRP Rule because they create dangerous amounts of dust and paint chips

- The work practices to use to control dust, debris or paint chips

- The tools you will need

### Slide 5-2: Traditional Renovations Create Airborne Lead Dust

- This chart illustrates that traditional work practices create large amounts of dust. Point out that the chart shows amounts of dust in the air measured for three common work practices.

- The source of the data for this chart is a study that measured amounts of lead dust in the air caused by each type of work. Results were extrapolated as if the activities were performed for eight hours. The dust was measured in micrograms per cubic meter (μg/m3).

- The OSHA trigger level at which special worker protection measures are required (known as the permissible exposure limit or PEL) is 50 μg/m3 (50 micrograms per cubic meter; time-weighted average over an 8-hour workday).

  - As shown in the chart, the amount of lead dust created by power and hand sanding and demolition is much greater than the PEL. OSHA requires special worker protection measures during conduct of these activities.

Traditional Renovations Create Airborne Lead Dust

*OSHA Permissible Exposure Limit (PEL) = 50 µg/m3

2

Notes:

- The data above is from the Lead Exposure Associated with Renovation and Remodeling Activities: Summary Report prepared by Battelle for the U.S. Environmental Protection Agency in May 1997

(Document: EPA-747-R-96-005).

- Traditional work practices create large amounts of dust!

  - This chart shows amounts of lead dust created by three common construction practices: hand sanding, power sanding, and interior demolition.

    - Note: The RRP Rule restricts the use of power sanders, among other things, see following page for more information.

  - By using safe work practices, you can control and significantly reduce the amount of dust created on the job. Controlling lead dust at the source of generation is important because dust released into the air will eventually become settled dust on the ground. Later in this chapter, you will learn safe

work practices that can replace these prohibited work practices. In this section you will also find best practice recommendations for reducing dust in the work area.

Note: If you have questions regarding OSHA’s air sampling requirements, OSHA provides a consultation service that is separate from enforcement and will not result in penalties or citations. This service can be accessed at www.osha.gov/consultation.

### Slide 5-3: Prohibited Practices

This slide lists several practices that are known to create large amounts of dust and create exposure risks for occupants and workers. These practices are:

- Open flame burning or torching.

- Heat gun paint removal above 1100 degrees Fahrenheit.

Note: The student notes on the facing page discuss the fact that minute lead dust particulates (“fume”) may be generated by high temperature heating and burning of paint. These particulates are very, very small and are easy to breathe into the lungs. They are therefore more dangerous than dust, which is generally ingested to the stomach. The lungs transfer lead to the blood stream much more efficiently than does the stomach. Because of this, fume is much more dangerous than ordinary lead dust. For this reason, heat gun removal above 1100 degrees F, open flame burning, and torching for removal of lead-based paint are prohibited.

- Use of machines that remove paint or other surface coatings through high speed operation such as sanding, grinding, power planing, needle gun, abrasive blasting, or sandblasting, unless such machines have shrouds or containment systems and are equipped with a HEPA (High-Efficiency Particulate Air) vacuum attachment to collect dust and debris at the point of generation Machines must be operated so that no visible dust or release of air occurs outside the shroud or containment system.

Note: The term “abrasive blasting” includes special tools that use high-pressure air mixed with another medium such as sand, walnut shells, chemical salts, or dry ice to remove paint from surfaces.

Highlight that the HUD Rule prohibits additional activities in properties receiving federal housing assistance. For example, the HUD Rule prohibits extensive dry scraping or sanding by hand (not prohibited in the EPA RRP Rule). See Slide 2-13 and Appendix 2 for more information.

Prohibited Practices

- Open-flame burning or

torching

- Heat gun above 1100oF

- Power sanding, power grinding, power planing, needle guns, abrasive blasting and sandblasting, without shroud or containment system equipped with HEPA vacuum

3

Notes:

The EPA RRP Rule does not specifically address what measures must be taken to reduce the amount of dust generated on the job. Rather, the rule lists three prohibited practices that must not be used on the job:

- Open-flame burning or torching of painted surfaces.

- Heat gun above 1100oF (degrees Fahrenheit).

- The use of machines designed to remove paint or other surface coatings through high-speed operation such as sanding, grinding, power planing, needle gun, abrasive blasting, or sandblasting unless such machines have shrouds or containment systems and are equipped with a HEPA vacuum attachment to collect dust and debris at the point of generation. Machines must be operated so that no visible dust release of air occurs outside the shroud or containment system.

A key to minimizing the spread of dust and paint chips is not to use certain traditional work practices known to create large amounts of dust and debris.

  - Open-flame burning or torching of paint and using a heat gun above 1100o F create very fine lead dust particulates (“fume”) that are dangerous for workers to breathe. The small lead dust particles created by burning and heating also settle on surrounding surfaces and are very hard to clean up.

  - Power sanding, power grinding, power planning, needle guns, abrasive blasting, and sandblasting create a large amount of dust that floats in the air and then settles on surfaces inside and outside of the work area.

See Appendix 5 Steps to LEAD SAFE Renovation, Repair and Painting for more information.

The practices listed on the slide are also prohibited in pre-1978 properties with lead-based paint that receive federal housing assistance. The HUD Rule also prohibits extensive dry scraping and sanding by hand, and paint stripping in a poorly ventilated space using a volatile paint stripper. Tribes, states, territories or localities may also prohibit these practice.

### Slide 5-4: Specialized Tools

  - Because many contractors use electric power tools on the job, it is often not possible to mist surfaces with water due to the possibility of electric shock. To prevent electric shock hazards, use battery-powered or pneumatic tools with HEPA filtration and that provide ground fault circuit interrupters (GFCIs) when working near electrical outlets and wiring, in wet areas, or on wetted surfaces.

  - Contractors should consider investing in specialized power tools with attached HEPA-filtered local capture ventilation, or buying attachments that fit their current power tools. This investment will pay off in the long run because workers will work quicker and contain dust better with these tools than without them. It may also be possible to rent such tools locally. Point out that the attachments do not capture all the dust created by the work, so the other precautions, especially containment systems, are still important.

Specialized Tools

- Large jobs may require special considerations to get the job done, like:

  - Power tools designed to remove paint or other coating must have shroud or containment system equipped with HEPA vacuum

  - Pneumatic and battery powered tools to protect against shock hazards

  - Specialized planning and containment

4

Notes:

Only power tools designed to remove paint or other surface coatings equipped with attached HEPA-filtered local capture ventilation may be used when lead-based paint is present or presumed to be present.

  - Electric power tools such as sanders, grinders, circular saws, reciprocating saws, planers and drills produce dust and debris.

    - Because they are electric, wet methods are not safe. Pneumatic and battery powered tools prevent shock hazards. Such tools must have a shroud or containment system equipped with a HEPA vacuum attachment to contain the dangerous lead dust and paint chips that are generated by their use.

  - Tools with attached HEPA-filtered capture ventilation collect and filter dust and debris as it is created.

    - A shroud at the head of the tool helps to contain the dust and paint chips as the vacuum draws away dust and debris for safe storage in the vacuum canister. This makes the job cleaner and safer.

  - Abrasive blasting is very effective at removing large areas of paint quickly, but these practices require special HEPA filtration equipment that contains the blast medium, dust and paint chips without releasing dust into the air or into the containment.

Containment is even more important when using specialized tools.

  - Proper containment and cleaning are crucial even when using HEPA-filtered specialized tools.

    - These tools generate a lot of dust inside a localized negative pressure (vacuum) environment. If the vacuum fails or if the vacuum seal created by the shroud is broken, large volumes of dust can be released. Nonetheless, HEPA-filtered specialized tools can reduce dust levels when used properly, and can aid work production by shortening the cleaning time and lowering cost.

  - See the Shopping List of tools and supplies found in Appendix 5 Steps to LEAD SAFE Renovation, Repair and Painting for more information.

### Slide 5-5: Protect Yourself

  - Workers should take precautions to protect themselves from dust hazards on the job.

    - Note: These are minimal precautions. Employers must comply with OSHA regulations which may require more extensive worker protection measures, especially for high dust jobs.

  - As you talk about the specific worker protection precautions, refer to the following:

    - Worker protection. Personal protective equipment: a painter’s hat, coveralls, and N-100 disposable respirator. N-100 is a NIOSH rating for respirators that can be used around lead dust. N-100 means that the respirator has HEPA filtering capability. The disposable N-100 respirator is acceptable for small jobs but under other work conditions, OSHA may require another type of respirator.

    - Workers should wash their hands frequently, especially before eating, smoking, and leaving at the end of the day. Lead from lead dust does not pass through the skin.

    - Disposable coveralls should be disposed of at the end of each work day. Non- disposable work clothing should be laundered by a commercial facility or if washed at home it should be washed separately from other laundry.

    - OSHA rules may specify that more involved worker protection measures be taken at a specific job or for specific work activities. Some work activities, by their nature, trigger OSHA requirements.

Protect Yourself

- Workers should wear:

  - Disposable painter’s hat

  - Disposable coveralls

    - Repair tears with duct tape

    - Dispose of in plastic bag

  - Disposable N-100, R-100 or P-100 respirator

- Wash face and hands frequently and at the end of each shift

  - Washing helps to reduce hand-to-mouth ingestion of lead dust

- OSHA may require more protection depending on what work is done

5

Notes:

Workers should protect themselves.

  - Painter’s hats are an inexpensive way to keep dust and paint chips out of workers’ hair. Painter’s hats can be easily disposed of, and should be disposed of at the end of each day or at the end of the job.

  - Disposable coveralls are a good way to keep dust off workers’ street clothes and reduce the chance of carrying dust away as they come and go. Remember to use a HEPA vacuum to remove dust and debris from coveralls or other outerwear (a “dry decon”) before exiting the work area. Some coveralls have a hood to keep dust out of hair.

  - Respiratory protection. Employers should consider that workers should wear respiratory protection, such as a disposable N-100, R-100 or P-100 respirator, and follow OSHA’s Respiratory Protection Standard (29 CFR 1910.134) to prevent them from inhaling lead dust. These respirators are particulate-filtering respirators and would not be appropriate as protection from chemical stripping compounds.

  - Wash your hands and face at the end of each shift. Workers should wash their hands and faces periodically to avoid ingesting lead dust. You should also wash well before eating, drinking or smoking. You should not eat, drink, or smoke in the work area. Some of the dust that settles on the face around the mouth invariably finds its way into the mouth. Workers should also wash at the end of the day before getting in their car or going home to avoid taking lead dust home to their families.

  - Personal protection is especially important on high dust generating jobs when lead-based paint or lead-contaminated dust is disturbed, and while cleaning is being performed. However, the same level of protection is not necessary during the planning, testing or setup phases of the work when lead is not being disturbed.

  - The protective equipment listed above is meant to show what is needed during activities that disturb lead-based paint and lead-contaminated dust. Depending upon work practices used, OSHA rules may require employers to take further steps to protect the health of workers on the job.

  - OSHA provides additional information on working with lead in their Safety and Health Regulations for Lead in the Construction Industry (29 CFR 1926.62).

### Slide 5-6: Control the Spread of Dust

  - This slide presents steps that workers should take to control the spread of dust from the work site.

  - Dust can be spread when workers leave the work site to get tools, carry away debris, take a break, leave at the end of the day, etc. The boundaries of the work site depend on the containment area. For example, it may be only the area covered by protective sheeting or it may be an entire room.

  - Workers can carry dust outside the work area on their shoes and clothes. They should always wipe the tops and bottoms of their shoes, and vacuum their clothes before stepping off the protective sheeting. Prior to leaving the work area renovators must change out of disposable clothing (such as shoe covers, protective suits, overalls, hats and gloves).

  - Workers should take extra precautions when cleaning themselves before leaving for home, because they can carry dust home to their families on their clothes, in their hair, on their bodies, and in their car. Studies have been conducted that measure the blood lead levels of worker families. These studies confirm that the children of construction workers do get poisoned by lead dust carried home from work sites. Remember the video seen earlier about the contractor who lead-poisoned his own kids.

## Control the Spread of Dust

    - When you leave the work site, clean yourself and your tools

      - Remove shoe coverings and HEPA vacuum or wipe shoes

      - Walk on disposable tack pads to remove dust from your soles

      - HEPA vacuum and remove coveralls, and then HEPA vacuum your clothes

      - Remove gloves, if used, and carefully wash your hands and face

    - At the end of the day don’t take lead home to your family on your clothes

or in your car

      - HEPA vacuum clothes, shoes, etc.

      - Change your clothes, and dispose of disposable clothing or place dusty work cloths in a plastic bag to wash separately from household laundry

    - Don’t hug your family until you are clean!

      - Wash your hands and face

      - Shower as soon as you get home	6

Notes:

  - When you leave the work site (the area covered by protective sheeting or the work room), take precautions to prevent spreading dust and paint chips on your clothes and shoes to other parts of the residence.

  - Every time you leave the plastic sheeting around the surfaces being renovated, remove the disposable shoe covers and wipe or vacuum your shoes before you step off the plastic sheeting. A large disposable tack pad on the floor can help to clean the soles of your shoes.

  - Every time you leave containment, HEPA vacuum and remove your disposable coveralls and disposable shoe covers. Clean and/or vacuum your shoes and wash your hands and face.

  - At the end of the day:

    - Change your clothes and wash yourself to reduce the risk of contaminating your car and taking lead dust home to your family.

    - Before leaving the worksite, remove any protective clothing, HEPA vacuum dust from non-protective clothing, and thoroughly wash your hands and face. Throw away disposable clothing or place clothing in a plastic bag to stop dust from getting on other clothes at home.

    - As soon as you arrive home, take a shower and be sure to thoroughly wash your hair, especially before playing with children. Wash your work clothes separately from regular household laundry to stop lead from getting on your other clothes.

    - Be clean before you come in contact with family members, especially children. Remember the video about the contractor who lead-poisoned his own kids.

### Slide 5-7: Cleaning During the Job

  - By nature, renovation, repair and painting jobs create debris which can pile up in the work area.

  - Debris should be removed periodically to keep it from being a source of dust that can be easily spread by work activity, and the comings and goings from the work site.

    - For example, paint chips are quite easily tracked to other parts of the residence (remember the sweetener demonstration you saw this morning). It is important to clean your shoes before stepping off the protective sheeting.

  - Cleaning to keep debris and dust under control can be done in stages, and should also be done at least daily.

  - Waste containment, removal and disposal are critical for keeping dust and debris in the work area to a minimum.

Cleaning During the Job

- A clean work site reduces the spread of dust and paint chips

- Clean as you work

  - HEPA vacuum horizontal surfaces

  - Remove debris frequently

  - Remove paint chips as they are created

  - As building components are removed, wrap and dispose of them immediately

- Clean frequently (in stages, at least daily)

7

Notes:

Clean the work site frequently.

  - Cleaning the work site frequently as the job progresses will reduce the spread of dust and paint chips. Daily cleaning need not be as thorough as the final cleaning. It should, however, keep debris, dust and paint chips from piling up and spreading beyond the immediate work site.

Daily cleaning during the job includes:

  - Removing debris frequently. Seal and dispose of construction debris as it is created.

  - Vacuuming horizontal surfaces frequently. HEPA vacuum dust and paint chips that settle on surfaces, including protective sheeting. As workers come and go during the work day, this debris is easily spread. Periodic cleaning throughout the work day helps minimize the spread of dust.

  - Collect paint chips as they are created. When removing paint, paint chips can spread outside the immediate work area as workers come and go from the work site. To keep paint chips from spreading beyond the work site, make sure that they are collected as they are created. Periodically HEPA vacuum and dispose of paint chips.

  - Wrapping and disposing of removed components. When removing painted components such as windows, trim and cabinets, wrap them in plastic sheeting and dispose of them in stages. This will prevent the spread of debris and keep residents, especially children, from coming into contact with lead dust created by the work.

  - Safe Waste Disposal. All renovation waste from the work area must be contained prior to its removal, storage, or disposal to prevent releases of dust and debris. Chutes for removing waste from the work area must be covered. At the conclusion of each work day, collect waste and store it in containment, in an enclosure, or behind a barrier that prevents the release of, and access to, dust and debris. When

transporting waste from the renovation work area it must be contained to prevent the release of dust and debris.

How often should cleaning during the job take place?

  - The goal is to keep dust and debris under control, not to maintain a completely spotless site at all times. Every job is different; so clean when it makes sense to, without hindering progress. Remove large amounts of dust, paint chips, and debris frequently, at least daily.

### Slide 5-8: Exercise: Personal Protective Equipment

  - In this exercise, students will learn how to put on and take off personal protective equipment, and how to decontaminate themselves and dispose of used equipment.

  - Prepare this exercise in advance:

    - Have appropriate supplies ready (See the list in Skill Set #6 on page 178, and in Appendix 6).

    - Review Skill Set #6.

  - Option - You may choose to perform this exercise as part of a larger comprehensive hands-on activity session after Module 8, as described in Lesson Plan 2.

Exercise: Personal Protective Equipment

- Watch the instructor dress a volunteer in personal protective equipment

  - Skill Set #6 – Personal Protective Equipment

- Practice putting on and taking off personal protective equipment

- Dispose of used equipment properly and clean up

8

Notes:

Personal Protective Equipment (PPE)

This exercise gives you a chance to learn and practice the proper steps for putting on and taking off PPE, disposing of used equipment, and decontaminating yourself.

The slide provides basic instruction.

### Slide 5-9: Now You Know…

  - Use this slide to close out this module and transition to the next module.

Now You Know…

- Work practices that produce dust

- Work practices prohibited by EPA and HUD

- How to work safely around lead-based paint and lead dust

- Proper use of personal protective equipment

9

Notes:

  - The practices you learned in this module will help you make less dust as you work.

  - In the next module, we will talk about how to conduct final cleaning of the work area, and how to verify that the cleaning is complete.