# Module 6: Cleaning Activities and Checking Your Work

Overview of this module: The information below summarizes the content and teaching methods for this module. This is for your reference, do not cover this with the participants.

##### Module 6: Cleaning Activities and Checking Your Work (90 minutes)

Key message: Do the cleanup right. Use wet mops and HEPA vacuums. Traditional methods don’t do the job. Presentation Outline:

  - 6-1: Overview

  - 6-2: What is Effective Cleanup?

  - 6-3: Interior Cleaning Requirements

  - 6-4: Visual Inspection Procedure

  - 6-5: Cleaning Verification Procedure

  - 6-6: Dust Clearance Examination

  - 6-7: Exterior Cleanup Requirements

  - 6-8: Exterior - Check Effectiveness of Cleaning

  - 6-9: Disposal

  - 6-10: Disposal - Federal, Tribal, State, Territorial, Local Information

  - 6-11: Exercise: Cleaning and the Cleaning Verification Procedure

  - 6-12: Now You Know…

Notes: Slides are followed by an exercise.

  - Slides (40 minutes)

  - Hands-On Exercise (50 minutes)

Preparing for this module: Prepare materials for hands-on exercise and the cleaning verification and clearance demonstrations.

Materials needed: Cleaning verification materials and tools listed on the Cleanup Supply List.

Options: The hands-on exercises in this module are not optional, but the trainer may choose when to use the exercises. Trainers may choose to follow Lesson Plan 2 and defer hands-on exercises until the end of the day, when hands-on exercises are combined and conducted all at once in the Hands-On Activities Session.

Lead Safety for Renovation, Repair, and Painting Initial Training Course

Notes:

### Slide 6-1: Overview

This module focuses on how to clean the work area in order to pass both a visual check and either a verification check or clearance below the action levels.

  - A visual check means that an area has been cleaned to the point that no dust, debris or paint chips can be seen with the naked eye.

  - The cleaning verification check involves comparing a disposable cleaning cloth used following cleaning to a cleaning verification card to determine cleanliness.

  - Clearance below the dust-lead action levels involves taking dust samples and having them analyzed and compared with applicable dust-lead standards to determine if the area is adequately clean.

Effective cleaning includes using specific techniques and following the proper order when cleaning. In this section, participants will learn:

  - How to conduct an effective cleaning;

  - The tools to always keep in your truck and at the work site;

  - Effective techniques to clean up after both interior and exterior jobs;

  - Safe disposal methods; and,

  - How to check your work.

- Effective cleanup

- Interior cleaning techniques

- Exterior cleaning techniques

- How to check your work

- Cleaning verification procedure

- Dust-lead testing

- Safe disposal practices

1

Notes:

This module will cover all the topics listed on the slide above.

  - The goal of cleaning is to leave the work area as clean as or cleaner than when you arrived so that, as a result of your work, lead dust is not left behind to poison the residents.

  - At the end of this module, you will know how to check your work to ensure the work area is clean enough to pass the visual inspection and cleaning verification procedure, or pass a clearance examination.

  - By using the techniques described in this module, you will be able to clean a work area quickly and efficiently. Remember, approaching a cleanup is similar to approaching a job. Proper preparation and planning will help make your cleaning efforts more effective and efficient.

  - Always schedule time at the end of each day to thoroughly clean the work area.

### Slide 6-2: What is Effective Cleanup?

  - Discuss the similarities between planning for cleanup and approaching a job. Explain that, just as you approach a job by planning the work to effectively contain dust and debris, you must approach cleaning by first having effective containment, then carefully following specific procedures to best clean the work area. The techniques outlined in this section should make your cleanup faster, more efficient, and more effective.

Remember:

  - Always conduct a visual check of the work area to make sure all work is complete.

  - Proper waste disposal and checking your work are essential to the process of cleaning.

  - The most effective cleaning will follow this sequence:

- Pick up all visible paint chips and debris.

- Clean and dispose of protective sheeting. For interior renovations:

- Walls - slowly vacuum with a HEPA (High-Efficiency Particulate Air) vacuum or wipe with a damp cloth, working from high to low.

- Other surfaces - thoroughly HEPA vacuum all surfaces including furniture and fixtures. Wipe any remaining surfaces with a damp cloth. The HEPA vacuum must have a beater bar for use on carpeting.

- Mop uncarpeted floors using the two-bucket mopping method.

- Visually inspect your work.

- Bag all waste in heavy-duty plastic bags, “gooseneck” seal, and dispose of them according to federal, state, territorial and local regulations.

- Perform cleaning verification on window sills, countertops and uncarpeted floors.

- Remove warning signs.

##### Demonstrate how to “gooseneck-seal” a heavy-duty plastic bag and note that this will be covered again in the disposal section.

    - Discuss why this cleanup sequence should work well.

    - Picking up all visible debris and paint chips prepares a work area prior to the first HEPA vacuuming.

    - Clean and dispose of protective sheeting. This step should come before HEPA vacuuming in order to collect any dust that may escape from the protective sheeting.

    - HEPA vacuum the area from high to low. This first HEPA vacuuming will collect dust and debris not visible to the naked eye.

    - Wet cleaning and mopping the area will further dislodge any lead-contaminated dust or debris not collected by the first HEPA vacuum. Wet cleaning also gets dust and debris that is “stuck” to surfaces.

    - If necessary, a final pass with the HEPA vacuum or wet cleaning cloth will capture any remaining dust or debris left after the wet cleaning.

    - The last step should be to check your work to make sure that visual check inspection can be passed, and all waste is bagged, sealed and disposed of in accordance with federal, state and local laws.

  - A dust clearance examination may be required by federal, Tribal, state, territorial or local law, or it may be requested by the homeowner. If so, clearance below the action levels will replace the cleaning verification process. Clearance is required by HUD in many homes receiving federal housing assistance. For more information about clearance below the action levels, see Slide 6-6.

What is Effective Cleanup?

- Keeping dust from getting back into areas already cleaned

- Using proper cleaning techniques

- Cleaning all surfaces, tools and clothing

- Checking your work

  - Usually will involve performing cleaning verification

  - Could include a clearance examination

- Safe and secure disposal of waste

2

Notes:

Containment

  - Effective cleaning begins with proper preparation and containment. Cleaning will be much easier and efficient if proper containment has kept all dust and debris confined to the work area. While cleaning, keeping dust in the area that is being cleaned is also important. You don’t want to have cleaned areas become re-contaminated after cleaning.

Proper cleaning techniques

  - Follow a “top to bottom, back your way out” approach so that you don’t have to re-enter an area that has already been cleaned. Start cleaning high to low. You should be careful not to spread dust to other areas while cleaning. Follow an ordered sequence of cleaning to ensure that you do not contaminate other areas. For example, if floors are cleaned before the countertops you must walk on the floors to get to the countertops and this risks re-contaminating the floors. Never re-enter areas already cleaned. Also, countertops are higher than floors and can drop dust onto the floors.

Cleaning all surfaces

  - The term “all surfaces” includes all vertical surfaces such as walls and windows, and all horizontal surfaces such as floors, door tops and moldings, window troughs, and window sills. Cleaning should proceed from high to low, i.e., from top of wall to window to floor.

Checking your work

  - Conduct a visual inspection after cleaning is completed. Look for dust, debris and residue.

  - Perform cleaning verification until all areas pass when compared to the cleaning verification card.

  - A dust clearance examination may replace cleaning verification when required by federal, Tribal, state, territorial or local law, or by the owner.

Safe and secure disposal of waste

  - Bag and “gooseneck-seal” all waste in heavy-duty plastic bags. Safely dispose of all waste in accordance with federal, Tribal, state, territorial and local regulations. See slides 6-9 and 6-10 for information on disposal.

### Slide 6-3: Interior Cleaning Requirements

  - Review the material on the slide by reading the actual requirements as they appear in the student text under the slide.

  - Ask: Why should you pick up paint chips and other debris before picking up the protective sheeting? Why should you mist down and wet wipe the protective sheeting before picking it up?

    - Answer to both questions: To prevent accidental spreading of lead-contaminated paint chips and dust off of the protective sheeting.

  - After the first visual inspection of the work area, cleaning, folding and disposing of the protective sheeting is the next step. Clean your protective sheeting with a HEPA vacuum and wet wipe if necessary. Once cleaned, fold (dirty side in) and seal the sheeting and dispose with the rest of your waste. When you pick up and fold the protective sheeting (dirty side in), be careful not to spread any dust that may remain on the sheeting.

  - This process is followed by HEPA vacuuming and wet mopping (discussed in the next slide) to clean up any dust that escaped the protective sheeting.

  - Note that the sheeting covering the entry to the work area should stay in place until after the cleaning and removal of other sheeting.

  - Emphasize that workers must always clean at least 2 feet beyond the work area.

  - Discuss why cleaning should always proceed from high to low.

    - Answer: Cleaning from high to low is more efficient and effective because any dust or debris dislodged will fall down to the floor. Just as one would clean steps working from the top down, cleaning a work area should proceed from high to low to “push” all dust not collected down to the floor, which should be cleaned last.

## Interior Cleaning Requirements

    - Collect all paint chips and debris, and seal in heavy-duty plastic bags

    - Mist, remove, fold (dirty side in) and tape or seal protective sheeting, dispose of sheeting as waste

    - Plastic sheeting between non-contaminated rooms and work areas must remain in place until after cleaning and removal of other sheeting

    - HEPA vacuum or wet wipe walls from high to low, then HEPA vacuum remaining surfaces and wipe with a damp cloth

    - Clean 2 feet beyond the contained work area

    - Use disposable wipes or change cloths frequently

    - Use HEPA vacuum with beater bar for carpets and rugs

    - HEPA vacuum and wet mop uncarpeted floors with a two-bucket mopping method or wet mopping system

3

Notes:

Pick up

  - Always begin cleaning activities by picking up visible paint chips and debris with a wet disposable cloth without dispersing any of it and then sealing this material in a heavy-duty plastic bag.

  - When the job is complete, mist the sheeting, fold it (dirty side in), and either seal it with tape, or seal it in a heavy-duty plastic bag. Always fold dirty side inwards, and seal with tape or place in a heavy-duty plastic bag. If it is placed in a heavy-duty bag, “gooseneck-seal” the bag and dispose of the bag with the rest of your waste. Dispose of all sheeting as waste by using the correct folding and disposal procedure, after it has been vacuumed.

Clean with a Plan

  - Start cleaning at the far end of the work area and work back to the exit.

  - Clean walls with a HEPA vacuum (operated following manufacturer’s instructions) or by wiping with a damp disposable cloth. Start with the tops of the walls, tops of doors and door frames and work down to the floor.

  - Thoroughly vacuum all remaining surfaces and objects, including furniture and fixtures, in the work area. The HEPA vacuum must be equipped with a beater bar when vacuuming carpeting or rugs.

  - Wipe all surfaces and objects that remained in the work area, except carpeted or upholstered surfaces, with a damp cloth.

Clean the floor last

  - Clean with a wet mopping system or a two-sided bucket and mop.

  - Clean the entire work area and the area within 2 feet of the work area.

  - If using the two-bucket mopping system, repeat the process using a new mop head and clean water. Remember, always keep one bucket for cleaning solution and the other bucket for wringing out the cloth or mop head. You must keep wash and rinse water separate. Change the rinse water often.

Check your work

  - Before a certified renovator visually inspects the work area, check your work to determine whether dust, debris or residue is still present. If dust, debris or residue is still present, these conditions must be corrected before the visual inspection is performed.

### Slide 6-4: Visual Inspection Procedure

  - Always conduct a visual check after cleaning is completed. If you find any dust, debris, or residue make another pass with the HEPA vacuum and, if necessary, wet clean again. You should continue these steps until the site passes a visual check. Slide 6-11 contains an exercise demonstrating cleaning, visual inspection and cleaning verification. This exercise is mandatory.

  - After passing a visual check, you can perform the cleaning verification procedure or have a clearance examination performed to check your work. In some instances, other dust sampling may be required.

  - Emphasize that cleanup should always be performed as if a clearance examination is to be conducted after cleaning.

## Visual Inspection Procedure

Conducted by certified renovator

- Put on disposable foot covers before entering the work area

- Make sure there is adequate lighting in the work area

  - Turn-on all of the lights or use a bright, white-light flashlight

- Systematically look for dust, debris or residue on every horizontal surface in the work area and 2 feet beyond

  - Work from the farthest area from the entry to the entry

  - Closely examine each surface

- If you find visible dust, debris or residue, then re-clean the work area and repeat step 3

- Once you have carefully looked at all of the surfaces and found no dust, debris or residue proceed to the cleaning verification procedure	 or clearance below the action levels	4

Notes:

  - Visual inspection after cleaning is required by the RRP Rule.

  - A visual inspection must be conducted by a certified renovator once cleaning is complete, and prior to the cleaning verification or clearance examination of the work area.

  - In a visual inspection, the certified renovator looks for dust, debris and residue.

  - Make sure that adequate lighting is provided during the cleaning and visual inspection of the work area. You cannot see dust and small paint chips without adequate lighting.

  - Inspect the entire work area and the area 2 feet beyond the work area on all sides of the containment.

  - Visual inspection of the work area alone will not verify that the work area has been cleaned adequately – visual inspection is only the first step.

    - In many instances, lead dust is not visible to the naked eye and will not be detected during a visual inspection.

  - Once the visual inspection has been completed and no dust, debris or residue is present, the work area must pass either the cleaning verification procedure or a clearance examination in order for the project to be completed in compliance with the RRP Rule.

  - Whether the cleaning verification procedure or clearance examination is conducted will be based on regulatory requirements or terms in the renovation contract.

### Slide 6-5: Cleaning Verification Procedure

  - This slide describes the cleaning verification procedure in detail. The instructor should walk students through the procedure.

  - Tell the students they may keep the cleaning verification card (CVC) if they successfully complete this course. These cards are provided by EPA to accredited training providers for distribution to certified renovators.

  - Demonstration: Depending on the time available, the instructor should demonstrate the cleaning verification process either during the lecture or as a part of the hands-on exercise. The demonstration should include attaching a wet disposable cleaning cloth to an application device, cleaning a known area of the floor, removal of the cloth, and comparing it to the CVC.

  - Whether or not you do the demonstration here, you should show the students the CVC and make sure that they understand how to conduct and interpret the results of the comparison process.

  - Discuss the carpet cleaning procedure and emphasize that the cleaning verification procedure does not apply to carpets; therefore, extra care must be taken in carpeted areas to not get dust on the carpet. This may require extra layers of plastic as a good work practice to prevent dust from getting in the carpet during conduct of the work.

## Cleaning Verification Procedure

    - Wipe each window sill within the work area, use a single wet disposable cleaning cloth per window sill

    - Wipe uncarpeted floors and all countertops with wet disposable cleaning cloths, up to a maximum of 40 ft2 per cloth

    - Compare each wipe to the cleaning verification card (CVC), if the cloth matches or is lighter than the CVC, the surface has passed cleaning verification and no further action is required.

    - If the cloth is darker than the CVC, re-clean and repeat the cleaning verification process

    - If the second wet cloth fails, wait 1 hour or until surfaces are dry, and now wipe with an electrostatically-charged white disposable cleaning cloth designed to be used for cleaning hard surfaces, this completes the cleaning verification	5

Notes:

After visual inspection, one of two activities must be conducted. A certified renovator must perform cleaning verification or other certified professionals must conduct a clearance examination. The steps for the cleaning verification procedure are explained below.

Window Sills

  - Using a single, wet disposable cleaning cloth, wipe the entire surface of each window sill in the work area. Wipe Countertops and Floors

  - Wipe the entire surface of each countertop and uncarpeted floor within the work area with wet disposable cleaning cloths. Floors must be wiped using a wet cleaning system, including a long handle device with a head to which a wet disposable cleaning cloth is attached. The cloth must remain damp at all times while being used to wipe the floor.

  - If the surface of a countertop or floor within the work area is greater than 40 square feet, the surface within the work area must be divided into roughly equal sections that are each less than 40 square feet. Wipe each surface section separately using a new wet disposable cleaning cloth.

Interpret the Cleaning Verification Procedure

  - Compare each wipe representing a specific surface section to the CVC. If the cloth used to wipe each surface section within the work area matches or is lighter than the CVC, that surface section has been adequately cleaned.

  - If the cloth is darker than the CVC, re-clean that surface section, then use a new wet disposable cleaning cloth to wipe the surface section. If the cloth matches or is lighter than the CVC, that surface section has been adequately cleaned.

  - If the second cloth does not match and is not lighter than the CVC, wait one hour or until the surface section has dried completely, whichever is longer. Then wipe the surface section with an electrostatically charged white disposable cleaning cloth designed to be used for cleaning hard surfaces. The cleaning verification procedure is now complete and the surface is considered clean.

  - When cleaning verification has been completed for all of the surfaces in the work area (including window sills), warning signs may be removed.

### Slide 6-6: Dust Clearance Examination

  - Discuss the following instances, where a dust clearance examination (40 CFR 745.227(e)(8)) may be required or requested instead of the cleaning verification procedure:

    - HUD requires a dust clearance examination after certain kinds of jobs in target housing receiving federal housing assistance. Ask if the property receives federal assistance. If so, ask if a clearance examination is required.

    - In some states, a clearance examination conducted by a certified or trained person may be required by law. You should be aware of laws regarding clearance examinations and renovation work in your Tribe, state, territory and/or locality.

    - In some instances, the owner may request that dust wipe samples be taken to locate lead hazards and to ensure cleaning has been effective. If you follow the cleaning techniques described earlier, you should pass be able to pass post-work dust-lead testing.

  - Emphasize that once you begin a clearance examination, if it fails to clear below the action levels, you must continue the cycle of re-cleaning, visual inspection, and dust wipe testing until the dust wipe results comply with the dust-lead standards governing the work.

A dust clearance examination may be performed instead of cleaning verification:

- A clearance examination must be a conducted by a certified lead inspector, risk assessor, or dust sampling technician

- If the examination fails to clear below the dust-lead action levels, the renovation firm must re-clean the work area until dust standards comply with applicable Tribal, state, territorial, and local standards

6

Notes:

Clearance Examination (Post-work Dust-lead Testing) is Optional under the RRP Rule

  - Dust-lead testing may be performed to check the effectiveness of the cleaning efforts. Clearance below the action levels is an option under EPA’s RRP Rule and required by the HUD Rule in many cases.

  - Dust-lead testing is performed to check the effectiveness of cleaning efforts.

  - In some cases, dust-lead testing may be required as part of “clearance” (a regulation-defined process to ensure that a work area is not contaminated with lead dust after work is completed).

  - Cleaning verification does not have to be performed if dust-lead testing is required at the conclusion of a renovation. In such cases, dust-lead testing must be performed by a certified lead inspector, risk assessor, or dust sampling technician.

  - Certified renovation firms are required to re-clean the work area until dust-lead levels in the work area meet the dust-lead action levels. Some Tribal, state, territorial, and local laws may require a clearance examination following renovation and remodeling work, to levels that differ from the federal dust-lead standards. The selection of a cleaning verification or a clearance examination will be based on regulatory requirements or the renovation contract.

HUD requires clearance examinations in many pre-1978 properties receiving federal housing assistance. The clearance examination may be scheduled by the agency administering the assistance. A clearance examination is performed by a trained person independent of the renovation firm performing the work. Ask your client or contact the agency administering the assistance to the property to find out if a clearance examination is required at the end of the job and to find out who will schedule it. Remember, if the property fails to clear below the dust-lead action levels, the unit must be re-cleaned and another clearance examination performed. Sometimes the cost of re-cleaning and the additional clearance examination will be the responsibility of the contractor. Cleaning well the first time saves both time and money.

### Slide 6-7: Exterior Cleanup Requirements

  - The main point of cleaning after an exterior renovation job is to prevent dust from spreading beyond the work area.

  - The focus of exterior cleanup should be on the areas accessible to children to include bare soil, play areas, exterior porches and exterior window sills.

  - Always visually inspect beyond the work area. Collect and dispose of all paint chips, dust and debris that you find.

- Clean all surfaces in the work area until no visible dust, debris, or

residue remains

- Collect all paint chips and debris without dispersal, and seal in heavy-duty plastic bags

- Remove protective plastic sheeting and mist before folding it dirty side inward

- Check your work

  - Focus on areas such as window sills, bare soil, and children’s play areas

- Look for dust, debris and paint chips

7

Notes:

Specific exterior jobs

  - If work takes place on an exterior porch or stairwell, HEPA vacuuming, wet cleaning and mopping, in addition to a thorough visual inspection, should be used to clean the work area. For such jobs, the cleanup should be similar to clean up after interior jobs. Collect and dispose of any dust and debris with the rest of your waste.

Remember

  - Lead-contaminated soil can poison children.

  - Avoid dry raking and shoveling to prevent spreading dust. However, raking and shoveling the soil is appropriate if it is misted first.

Protective sheeting

  - Collect all paint chips and debris and, without dispersing any of it, seal this material in a heavy-duty bag.

  - Remove the protective sheeting. Mist the sheeting before folding it, fold the dirty side inward, and either tape shut to seal or seal in heavy-duty bags. Dispose of the sheeting as waste.

Check your work

  - Before a certified renovator visually inspects the work area, check your work to determine whether dust, debris or residue is still present. If dust, debris or residue is still present, these conditions must be corrected before the visual inspection is performed.

### Slide 6-8: Exterior – Check the Effectiveness of Cleaning

  - Discuss why a visual inspection for checking the cleaning is necessary.

  - The visual inspection checks for dust, debris and residue and includes all parts of the work area, areas not covered by the protective sheeting, and areas 2 feet outside the containment.

Certified renovators conduct visual inspections after cleaning:

- Determines if any dust, debris or residue is still present on surfaces in and below the work area

  - Including windows sills and the ground

- If dust, debris or residue is present, these conditions must be eliminated by re-cleaning

- After re-cleaning, the certified renovator conducts another visual inspection

- When all areas pass, warning signs may be removed

8

Notes:

Checking your work

  - A thorough visual inspection by a certified renovator is the main part of checking your cleanup after an exterior job.

  - A certified renovator must perform the visual inspection to determine whether dust, debris or residue is still present on surfaces in or below the work area, including window sills and on the ground.

  - If dust or residue is present, clean again, and then repeat the visual inspection.

  - Warning signs may be removed after passing visual inspection.

For exterior jobs, HUD requires only a visual assessment of the work area to pass clearance. No dust or soil testing is required. If you follow procedures taught in this course you will satisfy HUD requirements.

### Slide 6-9: Disposal

  - Waste should be stored in a secure area to prevent children from getting into it and being exposed to lead dust.

  - Discuss methods to handle wastewater.

  - Wastewater produced during the job from mopping, wet cleaning, or misting should not be poured down the sink or tub (because it will contaminate the sink or tub), into the yard or down a storm drain.

  - Before disposal, wastewater may need to be filtered through a filter capable of filtering out particles 5 microns or larger, depending on Tribal, state, territorial and/or local wastewater regulations.

  - If local regulations allow, wastewater may be poured down the toilet. If local regulations do not allow this, you may be required to contain and test the water, and contact a waste disposal company to assist you with disposal. Your local water authority can assist you with this decision.

  - Always be aware of federal, Tribal, state, territorial and/or local regulations regarding wastewater disposal.

  - All waste should be handled carefully and sealed in heavy-duty plastic bags.

  - Do not overfill the bags. Renovation debris is heavy, and, if overfilled, will split the bags and could injure workers.

  - Certified firms must be aware of all components of the waste produced at the job site and of the proper method of disposal. Again, always be aware of federal, Tribal, state, territorial and/or local waste disposal regulations.

  - Note: Have bags and tape ready for the demonstration, if you choose to perform it, and for the required hands-on exercises.

Disposal

- What should I do with my waste?

- At the work site:

  - Place waste in heavy-duty plastic bags

  - “Gooseneck-seal” the bag

  - HEPA vacuum the exterior of the waste bag before removing it from the work area

  - Store waste in a secure area

  - Carefully dispose of waste in accordance with federal and other regulations

  - Always check local waste requirements

9

Notes:

At the work site

  - Waste from renovation activities must be contained to prevent releases of dust and debris before the waste is removed from the work area for storage or disposal. Architectural components that are too big to fit into bags must be wrapped in plastic and sealed with tape prior to removal from the work area. If needed, “double-bag” your waste to help prevent the waste from escaping if the bag is cut or ripped. If a chute is used to remove waste from the work area, it must be covered. Some examples of waste include protective sheeting, HEPA filters, paint chips, dust, dirty water, used cloths, used wipes, used mop heads, used protective clothing, used respirators, used gloves, and architectural components.

  - At the conclusion of each workday and at the conclusion of the renovation, waste that has been collected from renovation activities must be stored under containment, in an enclosure, or behind a barrier that prevents release of dust and debris out of the work area and prevents access to dust and debris.

  - When the firm transports waste from renovation activities, the firm must contain the waste to prevent release of dust and debris.

Wastewater

  - Water used for cleanup might be able to be filtered and dumped in a toilet if local rules allow. Never dump this water down a sink or tub, down a storm drain or on the ground. Filtering wastewater through a 5-micron filter may be necessary when lead-contamination such as paint chips and dust may be present in the water. Check with your local water treatment authority, and in federal, Tribal, state, territorial and/or local regulations for more information.

##### Always dispose of waste water in accordance with federal, Tribal, state, territorial and/or local regulations.

HUD recommends when building components are recycled or sold that any painted building components should be stripped before re-installation. If components are not stripped, they should never be reinstalled in housing.

### Slide 6-10: Disposal - Federal, Tribal, State, Territorial, Local Information

  - Waste disposal is regulated under the Resource Conservation and Recovery Act (RCRA), and various associated Tribal, state and local laws and regulations.

  - Some waste generated from lead work may meet the definition of “hazardous waste” because it is toxic, corrosive, ignitable or explosive. Therefore, it is important for contractors to segregate waste into categories that are likely to be hazardous and non-hazardous. Examples of hazardous waste may include paint chips, vacuum debris, sludge or chemical waste from stripper, and HEPA filters.

  - Generators of less than 220 pounds of hazardous waste per job site per month are exempt from federal waste disposal regulations and most state regulations.

  - Many states have more stringent regulations than federal requirements. It is, therefore, important for contractors to understand their obligations under these laws and regulations.

  - You should always be aware of how much waste you are generating per job site per month.

  - EPA’s website has links to state information on solid and hazardous waste disposal at www.epa.gov/ hwgenerators/links-hazardous-waste-programs-and-us-state-environmental-agencies.

  - In a memorandum to RCRA Senior Policy Advisors and EPA Regions 1-10, dated July 31, 2000, EPA’s Office of Solid Waste (now the Office of Land and Emergency Management) stated that lead-based paint waste from households may be disposed of as household garbage subject to applicable state regulations. For more information, see Appendix 8 and the EPA website at www.epa.gov/sites/default/files/documents/ hhwmemo-july00fnl.pdf. Although EPA considers lead-based paint waste commonly generated during residential renovation and painting to be household waste, some Tribes and states may not have yet adopted this interpretation. Until Tribes and states do adopt EPA’s interpretation, they may continue to regulate lead-based paint waste as potentially hazardous if generated in large enough quantities.

Disposal - Federal, Tribal, State, Territorial, Local Information

- According to federal law:

  - In housing

    - Waste can be disposed of as normal household waste

  - In non-residential child-occupied facilities

    - If hazardous waste exceeds 220 lbs, dispose of as hazardous waste

- Always check local requirements

10

Notes:

Waste disposal issues

Because EPA considers most residential renovation and remodeling as “routine residential maintenance”, the waste generated during these activities is classified as solid, non-hazardous waste, and should be taken to a licensed solid waste landfill. This does not apply to commercial, public or other non-residential child-occupied facilities.

    - If you generate any hazardous waste, you should determine whether you generate more than 220 pounds (lbs) of hazardous waste per job site per month. If you have less than 220 pounds of hazardous waste per location per month, manage the waste as solid, non-hazardous waste. If you

generate more than 220 lbs of hazardous waste, you should contact your Tribal, state, territorial and local regulators to find out how to properly dispose of it.

    - Some possible examples of hazardous waste include paint chips, vacuum debris, sludge or chemical waste from strippers, and HEPA filters.

    - Some possible examples of non-hazardous waste may include disposable clothing, respirator filters, rugs and carpets, protective sheeting, and solid components with no peeling paint.

    - All waste should be sealed in heavy-duty plastic bags and handled carefully.

    - Large architectural components should be wrapped and sealed in plastic sheeting, and disposed of along with other waste.

    - Always check federal, Tribal, state, territorial and/or local requirements before disposing of waste. Some states have enacted more stringent waste management and disposal requirements than federal regulations. You need to become aware of how federal, Tribal, state, territorial and/or local requirements affect the management and disposal of renovation waste in your area.

### Slide 6-11: Exercise: Cleaning and the Cleaning Verification Procedure

  - Prepare this exercise in advance:

  - Have appropriate tools and supplies ready. Necessary tools and supplies include buckets, mops, water, detergent, a HEPA vacuum, wipes, plastic sheeting, heavy-duty plastic bags, tape, disposable cleaning cloths and CVC’s. See Skill Sets #7 - #11 for a complete list of materials and supplies.

    - You will need access to water.

    - Sprinkle each work area with corn starch or baby powder to simulate dust.

    - Refer to the Skill Sets #7 - #11 on pages 208-216 and in Appendix 6 for a list of skills that participants should practice.

    - Vary the color and type of work area surfaces if possible.

  - When conducting the exercise:

    - Instruct participants to stay in their groups.

    - Circulate while they work to ensure they are doing the work properly.

    - Coach them as necessary to correct any incorrect behaviors.

    - Give them a 5-minute warning.

    - When the time for each skill set is up, tell them to stop. Keep to the schedule.

  - Option: Trainers may choose to follow Lesson Plan 2 and defer hands-on exercises until the end of the day, when hands-on exercises are combined and conducted all at once in the Hands-On Activities Session.

  - Note: In some training facilities, such as hotels, you may not have access to water in the training room. In such cases, instruct participants to walk through the process - practicing the order of the steps: HEPA vacuum, two bucket wash, cleaning from high to low, etc. As a demonstration of how hard it is to clean up dust, consider sprinkling corn starch or baby powder on a tabletop and experimenting with different methods for cleaning it up - broom and dust pan, HEPA vacuum, wet wipe, etc.

Exercise: Cleaning and the Cleaning Verification Procedure

- Work in groups of 2-6

- Assignments:

  - Skill Set #7: Interior Final Cleaning

  - Skill Set #8: Exterior Final Cleaning

  - Skill Set #9: Bagging Waste

  - Skill Set #10: Visual Inspection

  - Skill Set #11: Cleaning Verification Procedure

- Choose the tools and supplies you need to clean the work area

- Clean your work area

- You have 40 minutes

11

Notes:

This exercise gives you a chance to demonstrate cleanup, visual clearance, cleaning verification, and proper waste-bagging techniques. The slide provides basic instruction.

  - Stay in your groups of 2 to 6 students, in your work area.

  - Choose the right tools. Tools available include buckets, mops, water, detergent, HEPA vacuum, wipes, plastic sheeting, heavy-duty plastic bags, tape, etc.

  - Clean up the dust.

  - Bag the waste.

  - Check your work.

  - Verify cleaning.

### Slide 6-12: Now You Know…

  - Review the following slide with students to remind them about what they learned during this module.

Now You Know…

How to:

- Clean the work area systematically

- Check the effectiveness of cleaning

- Perform a visual inspection of the work area

- Perform the cleaning verification procedure

- Release the work area for dust-lead testing

- Properly dispose of waste

12

Notes: