# Module 8: Training

Non-Certified Renovation Workers

Overview of this module: The information below summarizes the content and teaching methods for this module. This is for your reference, do not cover this with the participants.

##### Module 8: Training Non-Certified Renovation Workers (40 minutes)

Key message: Certified renovators are responsible for teaching lead-safe work practices to non-certified renovation workers.

Presentation Outline:

- 8-1: Overview

- 8-2: Teaching Lead-Safe Work Practices Means

- 8-3:The Role of the Certified Renovator

- 8-4: Role of Trained, Non-Certified Renovation Workers

- 8-5: Steps for Teaching Lead Safety During Renovations

- 8-6: Use the Steps Guide

- 8-7: Step 1: Determine if the Job Involves Lead-Based Paint

- 8-8: Step 2: Set It Up Safely

- 8-9: Step 3: Protect Yourself

- 8-10: Step 4: Minimize the Dust

- 8-11: Step 5: Leave The Work Area Clean

- 8-12: Step 6: Control the Waste

- 8-13: Step 7: Cleaning Verification or Dust-Lead Testing

- 8-14: Training Documentation

- 8-15: Now You Know…

Notes: Emphasize that this training is to give certified renovators the tools and skills necessary to conduct either on-site “toolbox” training or classroom training for non-certified renovation workers. All training should focus mainly on teaching students to perform the tasks necessary to work as non-certified renovators on the job.

Special emphasis should be placed on the practical skills and activities of lead-safe work practice using as much hands-on instruction as possible.

Preparing for this module: Review so you are familiar with the materials.

Lead Safety for Renovation, Repair, and Painting Initial Training Course

Notes:

### Slide 8-1: Overview

- Announce the module and move quickly to the next slide.

- Review the goal of this module:

  - To enable certified renovators to effectively teach non-certified workers how to use lead-safe work practices during renovation, repair and painting work in target housing and child-occupied facilities.

Certified renovators are responsible for teaching lead-safe work practices to non-certified renovation workers

1

Notes:

The RRP Rule requires that you, the certified renovator, be responsible for instruction of non-certified renovation workers. Note that the non-certified workers must be trained only on the RRP Rule required lead-safe work practices that the workers will be using in performing their assigned tasks. For example, if a

worker is hired to only provide clean-up services, that worker would not need to be trained on how to set up a work area.

Note: See Slide 2-11 and HUD regulations for more information on the training required for workers on HUD funded renovations.

### Slide 8-2: Teaching Lead-Safe Work Practices Means:

- Review the slide and student notes with the class.

- Training workers to properly use signs, dust barriers, dust minimizing work practices, and dust cleanup practices during the course of renovation, repair, and painting activities to prevent and/or reduce dust-lead contamination in the home

- To effectively train workers you need to:

  - Understand lead safety for yourself

  - Show students what you know

  - Review the shopping list in Steps to LEAD SAFE Renovation, Repair, and Painting and have appropriate materials on hand

2

Notes:

- You will be teaching non-certified workers the skills you mastered during the Skill Set exercises so they can master them.

- The shopping lists in Steps to LEAD SAFE Renovation, Repair and Painting detail all the equipment and supplies you may need to conduct hands-on exercises during on-the-job training of non-certified renovators.

- The information we are going to cover now is presented to help you conduct your own training of non-certified workers.

### Slide 8-3: The Role of the Certified Renovator

- The information on this slide is provided to remind certified renovator students that certified renovators are named in the RRP Rule as responsible for teaching lead-safe work practices to non-certified renovation workers.

- Review the slide and student notes with the class.

Certified renovators:

- Perform lead-safe work as described in the RRP Rule

- Train all non-certified workers in lead-safe work practices

- Provide onsite and regular direction for all non-certified workers during setup and cleanup

- Are available by phone when not physically present at the work site during work

- Maintain onsite proof of certification as a certified renovator

3

Notes:

What are the responsibilities of a certified renovator?

- Certified renovators are responsible for ensuring overall compliance with the RRP Program requirements for lead-safe work practices at renovations to which they are assigned.

A certified renovator must:

- Use an EPA-recognized lead test kit or alternatively collect a paint chip sample for analysis, when requested by the party contracting for renovation services, to determine whether components to be affected by the renovation contain lead-based paint.

- Provide lead-safe work practices training to non-certified workers so those workers can perform assigned tasks safely. This training can be provided by the certified renovator on-the-job or in the classroom, provided adequate hands-on practice is available. This training could also be conducted by a third party although the instructor must be a certified renovator.

- Be physically present at the work site when warning signs are posted, while the work area containment is being established, and while the work area cleaning is performed. (Note: Use the terms Setup and Cleanup to describe this work).

- Monitor work being performed by non-certified workers to ensure that lead-safe work practices are being followed. This includes maintaining the integrity of the containment barriers and ensuring that no dust or debris migrates from the work area.

- Be available, either on-site or by telephone, at all times during performance of the renovation.

- Perform project cleaning verification.

- Have copies of their initial course completion certificate and their most recent refresher course completion certificate at the work site. Certification as a certified renovator lasts for 5 years. The certified renovator must take a refresher course every 5 years in order to maintain certification.

- Prepare required records.

### Slide 8-4: Trained, Non-Certified Renovation Workers

- Review the slide and student notes with the class.

- Toolbox talks are short, informal safety meetings (typically 5-15 minutes) held at the job site, usually at the beginning of a shift or work day. OSHA provides several pre-written toolbox talks on various topics at oshatraining.com/more-osha-training-resources/toolbox-talks-for-osha-safety-and-health/ as well as a few tips for conducting toolbox talks your students may find helpful:

  - Read the toolbox talk to yourself a couple of times before you hold the actual meeting with workers. That way you will be more familiar with the content to be covered and therefore less apt to stumble while reading to the group.

  - Try to hold the toolbox talk in an area that is free of noise and other distractions. If the workers cannot hear you talking, or are distracted by other activities in the area, they won’t be focusing on your talk.

  - Speak clearly and directly. Mumbling or reading too fast makes it difficult for the workers to understand you. Just take a deep breath, and then speak clearly and at a natural pace.

  - Use a prop when possible to help you keep the workers attention. If you are giving a toolbox talk on setting up a portable step ladder, have one set up nearby so you can point out things as you read the toolbox talk.

  - Always give workers an opportunity to ask questions at the end of the toolbox talk. Don’t make snide remarks to employees who do ask a question, as this will discouraging others from asking questions later.

  - Last but not least, practice what you preach. Nothing makes a trainer lose credibility faster than to have a worker see them doing something that violates the safety precautions that were covered in a previous toolbox talk. Always set a good example for others.

Trained, Non-Certified Renovation Workers

- Persons working on renovation, repair and painting jobs who have had on-the-job training or similar classroom training from a certified renovator to perform tasks in compliance with the RRP Rule

- Must perform lead-safe work practices as described in the RRP Rule:

  - Protect the home by “setting up” the work area.

  - Protect themselves

  - Perform renovation work safely

    - Not use prohibited practices

    - Control dust and debris

  - Clean the work area

4

Notes:

- On-the-job training must be provided for each worker and each job to the extent that each worker is adequately trained in the tasks they will be performing. This training may occur while the worker is engaged in productive work, which provides knowledge and skills essential to the full and adequate performance of the job. However, work conducted during training must be in full compliance with the RRP Rule.

- Your trainees will benefit by seeing the lead-safe work practices steps found in Steps to LEAD SAFE Renovation, Repair and Painting (Steps Guide) in Appendix 5. It contains a seven-step primer on lead safety and can be used as a field text to hand out to non-certified worker trainees in the field. In the Steps Guide, steps 2 through 6 contain information specific to work performed by non-certified personnel, while step 1 and step 7 contain information on testing painted surfaces and cleaning verification which are the responsibility of the certified renovator. Step 7 also discusses clearance examination which is performed only by certified lead inspectors, risk assessors, and dust sampling technicians.

- The information in the Steps Guide can be covered in 5-10 minutes per step and then reinforced by on-the-job practical exercises such as setting up barriers and signs, demonstrations of cleaning procedures, etc. It is recommended that the material in the Steps Guide be covered in a toolbox talk (on- site meeting) format with handouts (you create or get from someone else) on the specific information to be covered.

- It is very important that non-certified workers be allowed to participate in hands-on learning as work progresses and that skill sets that are learned by each student are documented. Documentation is required by the RRP Rule to assure that non-certified workers are trained to perform renovation activities to

which they are assigned. Remember that the RRP Rule requires all non-certified workers on the job to be given skills training specific to the tasks that they will perform on the job and that each person’s training must be documented by topic area covered in the on-the-job training that is performed. The required documentation will be discussed in more detail later.

### Slide 8-5: Teaching Lead Safety During Renovations

- Review the slide and student notes with the class.

Teaching Lead Safety During Renovations

- Use the Steps Guide as a basis for your training of non-certified renovation workers

- Training can be:

  - Delivered in one session covering all 7 steps; or

  - Cover the information in a series of “toolbox” meetings over the course of several days

- Spend 5-10 minutes on information in each step and then do on-the-job training to teach the skills needed

- If you offer training in a classroom, you can use slides 8-6 to 8-15

- Give each worker a copy of the Steps Guide	5

Notes:

- Training for non-certified renovation workers can be delivered in one session covering all 7 steps or the information can be covered in a series of toolbox talks over the course of several days. You should spend about 5-10 minutes on the information contained in each step and then conduct on-the-job training to teach the skills needed to renovate lead safely.

- If this training is offered in the classroom, Slides 8-6 through 8-15 can be used to teach the material found

in the Steps to LEAD SAFE Renovation, Repair, and Painting found in Appendix 5 of your student manual.

- Use the Steps Guide as a student handout for training non-certified renovation workers. Your handout(s) should also include write-ups of demonstrations and practical hands-on exercises, and a checklist of desired skills to reinforce the toolbox talks or classroom training. During either the toolbox talks or classroom instruction make sure your trainees have their copy of the handout so you can reference the training handout and where exactly to find the information while teaching about a particular step or skill.

- Before you train non-certified renovation workers make sure to print enough copies of the Steps Guide so you can give one copy to each non-certified renovation worker (trainee) a copy.

Slide 8-6: Use the Steps Guide

- Review the slide and student notes with the class.

Use the Steps Guide

- The Steps Guide covers basic lead-safe work practices and can be used as a training guide outside of the

classroom in conjunction with on-the-job demonstrations and hands-on exercises

6

Notes:

The Steps Guide:

- Designed to help plan for and complete a home renovation, repair or painting project using lead-safe work practices that reduce the amount of dust produced during renovation activities.

- Describes the required lead-safe work practices.

- Provides additional helpful recommendations.

- Includes information about lead, lead-based paint and the RRP Rule.

- Discusses the 7 steps for lead-safe RRP projects, which we are covered in slides 8-7 to 8-13.

### Slide 8-7: Step 1: Determine If the Job Involves Lead-Based Paint

- Review the slide and student notes with the class.

## Step 1: Determine If the Job Involves Lead-Based Paint

  - Lead-based paint (LBP) is found in many older homes:

    - 1960-1978 homes – 1 in 4 have LBP

    - 1940-1960 homes – 7 in 10 have LBP

    - Pre-1940 homes – 9 in 10 have LBP

  - Renovation, repair or painting that disturbs LBP can create significant LBP hazards in homes

  - Just a little lead dust can harm kids, their parents and pets, and cause problems for pregnant women and their unborn children

  - A certified renovator will determine if LBP is present on work surfaces

  - If information about LBP is not available for a pre-1978 home or child-occupied facility, assume that LBP is present and use lead-safe work practices

7

Notes:

During training:

- Review the information on this slide with the non-certified renovation workers as they follow along on pages 9 and 10 of the Steps Guide.

Notes to the on-the-job instructor:

- This information is included in the on-the-job training so non-certified renovation workers will understand why they need to use lead-safe work practices. Non-certified renovation workers are not allowed to determine whether lead-based paint is present, but they should understand that when it is identified as present, generating dust can cause significant problems if not properly and safely controlled.

For more information review pages 9 and 10 of Steps Guide.

### Slide 8-8: Step 2: Set It Up Safely

- Review the slide and student notes with the class

Step 2: Set It Up Safely

- Containment is used to keep dust and debris IN the work area and non-workers OUT

- Signs and barriers (plastic sheeting) are used to limit access

- Inside versus outside jobs

  - Review all procedures and differences in setup

8

Notes:

- When you work on a job with lead-based paint, you must contain the work area to prevent the escape of dust and debris. The goal of proper setup of the work area is to keep dust in the work area and

non-workers out.

- You must post signs clearly defining the work area and warning occupants and other persons not involved in renovation activities to remain outside of the work area.

- Place signs, barrier tape, and/or cones to keep all non-workers, especially children, out of the work area. Have residents keep pets out of the work area for their safety and to prevent them from tracking dust and debris throughout the home.

- You will need to take slightly different steps to setup for inside versus outside jobs. Inside jobs

- Remove furniture and belongings from the work area. If an item is too large or too heavy to move, cover it with heavy plastic sheeting and tape the sheeting securely in place.

- Use heavy plastic sheeting to cover floors in the work area to a minimum of 6 feet from the area of paint disturbance. Close and seal doors, close windows.

- Close and cover air vents in the work area. This will keep dust from getting into the system and moving through the home.

Outside jobs

- Cover the ground and plants with heavy plastic sheeting to catch debris. The covering should extend at least 10 feet out from the building, unless a property line prevents 10 feet of such ground cover, in which case the firm must erect vertical containment. Secure the covering to the exterior.

- Close windows and doors within 20 feet of the work area to keep dust and debris from going into the home.

- Move (if possible) or cover play areas and equipment within 20 feet of the work area.

For more information about Step 2 review pages 11-14 of the Steps Guide.

### Slide 8-9: Step 3: Protect Yourself

- Review the slide and student notes with the class

Step 3: Protect Yourself

- Without the right personal protective equipment workers can:

  - Swallow and inhale lead from the job

  - Carry lead on their skin and work clothes home to their families

- Review the shopping list

- Advise workers to:

  - Keep clothes clean or use disposable clothing

  - Wear a respirator, the appropriate respirator keeps lead out of the lungs and stomach

  - Wash-up each time they leave the work area and especially at

the end of the day

9

Notes:

Wear protective clothing.

- Protective clothing and shoe covers are very important in preventing:

  - Take-home lead;

  - Tracking lead out of the work area; and

  - Contamination of areas that have already been cleaned during final cleanup.

- Keep clothes clean. At the end of the work day, vacuum off dust or change out of dusty clothes. Do not use compressed air to blow dust off clothing. Wash dirty work clothes separately from household laundry.

- Wear a painter’s hat to protect your head from dust and debris. Wear respiratory protection.

- When work creates dust or paint chips, employers should consider respiratory protection, such as a N-100 disposable respirator, to prevent workers from breathing lead dust.

Post warning signs.

- Post a warning sign at each work area entrance.

- Signs should read: “Warning, Lead Work Area, Poison, No Smoking or Eating” to remind workers that eating, drinking and smoking in the work area is prohibited.

Wash up.

- Workers should wash their hands and faces each time they stop work. It is especially important to wash up before eating and at the end of the day.

Note: OSHA rules may require employers to take further steps to protect the health of workers on the job.

For more information about Step 3 review pages 15-16 of the Steps Guide.

### Slide 8-10: Step 4: Minimize the Dust

- Review the slide and student notes with the class

Step 4: Minimize the Dust

- The goal is to minimize and control the spread of dust by:

  - Using the right tools, including disposable heavy-duty plastic sheeting

  - Keeping the work area closed off

  - Using lead-safe work practices

  - Avoiding prohibited practices

- Review the shopping list

10

Notes:

Minimize and control the spread of dust.

- Keep the work area closed off from the rest of the home.

- Don’t track dust and debris out of the work area.

  - Vacuum off all workers paying particular attention to shoes when exiting the work area so the dust stays inside containment.

  - Remove disposable shoe covers and make sure your shoes are clean by using tack pads or damp paper towels to wipe off your shoes each time you step off the protective sheeting.

- Stay in the contained work area and on the contained paths.

- Keep components that are being disposed of in the work area until they are wrapped securely in heavy plastic sheeting or bagged in heavy-duty plastic bags. Once wrapped or bagged, HEPA vacuum the exterior and remove them from the work area and store them in a safe area away from residents.

- Launder non-disposable protective clothing separate from family laundry.

- Do not use prohibited practices, including:

  - Open-flame burning or torching

  - Using a heat gun at temperatures greater than 1100oF (to remove paint)

  - Sanding, grinding, planing, needle gunning or blasting with power tools and equipment not equipped with a shroud and HEPA vacuum attachment

For more information about Step 4 review pages 17-19 of the Steps Guide.

The HUD Rule also prohibits extensive dry scraping and sanding by hand, use of heat guns that char paint, and paint stripping in a poorly ventilated space using a volatile paint stripper. States, localities or Tribes may prohibit additional work practices.

### Slide 8-11: Step 5: Leave the Work Area Clean

- Review the slide and student notes with the class

Step 5: Leave the Work Area Clean

- The goal is to leave the work area completely free of dust, debris and residue:

  - Clean at the end of every workday

  - Thoroughly clean at the end of the job

- Review the shopping list

- Discuss both daily and end of job cleaning procedures

11

Notes:

On a daily basis, pick up the work area (recommended).

- Pick up as you go. Put trash in heavy-duty plastic bags.

- Vacuum the work area with a HEPA vacuum several times during the day and at the end of the day. Do not clean with a standard household or shop vacuum cleaner. Use only HEPA vacuums.

- Clean your tools at the end of the day.

- Wash your hands each time you leave the work area and especially before you go home.

- Dispose of all disposable personal protective clothing daily. When the job is complete, clean the work area (required).

- Make sure all paint chips, dust, trash and debris, including building components, are removed from the area to be cleaned and disposed of properly.

- Carefully remove plastic sheeting on the floor, fold it with the dirty side in, tape the edges shut or seal it in a heavy-duty plastic bag, and dispose of it. Keep plastic sheeting in doorways and openings that separate the work area from non-work areas in place until the work area is released as clean.

- HEPA vacuum or wet wipe all wall surfaces. HEPA vacuum all other surfaces in the work area. Use a beater bar attachment on carpets.

- Wet wipe all remaining surfaces in the work area and wet mop all uncarpeted floors until dust, debris and residue are removed.

- Visually inspect your work. Look around the work area and two feet beyond, and on paths where debris was carried. You should see no dust, debris or residue.

- Re-clean the area thoroughly if you find dust, debris or residue.

For more information about Step 5 review pages 20-21 of the Steps Guide.

### Slide 8-12: Step 6: Control the Waste

- Review the slide and student notes with the class

- Waste from renovation activities must be contained to prevent releases of dust and debris before it is removed from the work area

- Discuss:

  - Waste bagging procedure

  - Temporary storage of waste

  - How to deal with wastewater appropriately

  - Waste disposal rules that apply to the specific job

- Demonstrate how to fold a small section of plastic with the dirty side turned in

12

Notes:

Waste from renovation activities must be contained to prevent releases of dust and debris before the waste is removed from the work area for storage or disposal.

Bag or wrap your waste at the work site and in the work area.

- Collect and control all your waste. This includes dust, debris, paint chips, protective sheeting, HEPA filters, dirty water, clothes, mop heads, wipes, protective clothing, respirators, gloves, architectural components, and other waste. Use heavy-duty plastic sheeting or bags to collect waste. Gooseneck seal the bag with duct tape. Consider double bagging waste to prevent tears. Large components should be wrapped in protective sheeting and sealed with tape. Bag and seal all waste before removing it from the work area.

HEPA vacuum the exterior of waste bags and bundles before removing them from the work area. Store all waste that has been collected from renovation activities preventing access to and release of dust and debris.

Dispose of wastewater appropriately.

- Water used in the work area to remove paint or clean surfaces should be filtered through a 5 micron filter. Never dump this water down a sink or tub, in a storm drain, or on the ground. It may be dumped in a toilet if local rules allow. If local regulations do not allow this, you may be required to contain and test the

water, and contact a waste disposal company to assist with disposal. Check with your local water treatment authority, and federal, Tribal, state, and territorial regulations for more information.

Be aware of waste disposal rules.

- EPA considers most residential renovation, repair and painting activities “routine residential maintenance.”

The waste generated by these activities is classified as solid, non-hazardous waste and can be disposed of in an ordinary waste landfill. Some Tribes, states, territories and localities have more stringent waste disposal requirements that must be followed.

For more information about Step 6 review pages 22-23 of the Steps Guide.

### Slide 8-13: Step 7: Verify Work Completion with Cleaning Verification or Dust-Lead Testing

- Review the slide and student notes with the class

Verification or Dust-Lead Testing

- Cleaning verification:

  - Required by the RRP Rule

  - Must be performed by a certified renovator

- Clearance examination, which includes dust-lead testing:

  - May be requested in place of cleaning verification by the owner

  - Required by HUD for housing receiving federal housing assistance

- Discuss what happens when cleaning verification and/or clearance below the action levels is not passed

13

Notes:

- When all the work is complete, and before the space is reoccupied, a cleaning verification or clearance examination must be conducted to make sure lead dust is not left behind.

  - If the HUD Rule applies, a clearance examination is required in place of the cleaning verification procedure.

- The first step for both cleaning verification and a clearance examination is a visual inspection of the work area to determine if dust, debris or residue was left behind.

  - If dust, debris or residue are present in the work area, cleaning must be repeated and the visual inspection repeated until the work area is free of dust, debris and residue.

- Once the visual inspection by the certified renovator is complete, either the cleaning verification or clearance examination can proceed.

  - For exterior projects, when work areas have passed the visual inspection, the project is complete and the area may be turned over to the occupants.

  - For interior projects, when work areas have passed the visual inspection, the certified renovator performs cleaning verification by wiping all dust collection surfaces in the work area with a wet, disposable cleaning cloth and comparing that cloth visually to a cleaning verification card.

  - Clearance examination or dust-lead testing must be performed by a certified lead-based paint inspector, risk assessor or dust sampling technician.

For more information about Step 7 review pages 24-28 of the Steps Guide.

### Slide 8-14: Training Documentation

- Review the slide and student notes with the class

- The certified renovator assigned to the job must maintain a record for any on-the-job training:

  - Written certification of non-certified renovation worker training must including all of the following:

    - Show which workers received what training

    - List all training topics covered for each worker

    - Be signed by the certified renovator who did the training

  - All training documentation must be kept for 3 years following completion of the renovation

14

Notes:

When you give your toolbox talks or classroom training, use of a training guide, such as the Steps Guide, will make documentation easier.

You should keep a copy of the training guide used for the toolbox talks you conduct on-site or classroom training.

You are required to keep a record of any on-the-job training for 3 years.

Your records (written certification) of non-certified renovation worker training must include:

- A list of each lead-safe practical skills covered for each individual non-certified renovation worker.

- A list of the practical skills taught to each non-certified renovation worker with the material covered in the toolbox talk or classroom training will provide adequate documentation to meet RRP Rule requirements.

### Slide 8-15: Now You Know…

- Review the slide and student notes with the class

Now You Know…

- Certified renovators are responsible for training non-certified renovation workers

- Roles of certified renovators and trained, non-certified renovation workers during a renovation

- How to use the Steps Guide to train non-certified renovation workers

15

Notes: